| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on AAI_LBS-MSGs Supporting LBS in IEEE P802.16m (LBS) |  |
| Date Submitted | 2009-09-17 |  |
| Source(s) | Eunkyung Kim ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D1 LBS,” in response to the TGm Call for Contributions on Project 802.16m Amendment Content, IEEE 802.16m-09/0037 |  |
| Abstract | Comment on LBS related messages |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comment on AAI_LBS-MSGs Supporting LBS in IEEE P802.16m (LBS)

Eunkyung Kim

ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD)[1] describes the support for Located Based Services (LBS) and Section 12 in SDD describes some primitive concept of LBS, which is not enough to cover LBS operation on the next version of IEEE 802.16m Amendment Draft Standard [2] . Therefore, in this contribution, we describe the some AAI MAC management messages, which support LBS operation in IEEE 802.16m.

# 2. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

Proposed Text for Support for LBS (15.2 in the 802.16m Amendment Draft Standard)

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[modify and add the following rows into Table 729 – parameters for AAI_PAG-ADV message format]

| Name | Value | Usage |
| --- | --- | --- |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement(initiate/change) 0b11 = Reserved Stop performing LBS measurement | Paging action instruction to each paged AMS. |
| Dedicated ranging code and opportunity | [TBD] | Ranging to perform LBS measurement It may be included when Action Code = 0b10 |

[Adopt the following rows to the Table 682 – parameters for AAI_RNG-REQ]

| Name | Value | Usage |
| --- | --- | --- |
| … |  |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#5 is set to 1, it indicates that the AMS is attempting to report the LBS related metric to perform LBS measurement. Bit#56-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO, or location update, or LBS measurement |
| … |  |  |
| LBS GPS-based measurement metric | In case of GPS based method, An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimated its location and the following parameters are included: Longitude Latitude Altitude | If the ranging purpose is to perform LBS measurement and/or the AMS wants to measure the location, it is included in the AAI_RNG-REQ LBS measurement metric transmitted by the AMS which is used to perform GPS based LBS. measurement |
| LBS Non-GPS-based measurement metric | In case of Non-GPS based method, LBS measurement may be supported in the DL and UL and according to the measurement metric, the following parameters are included: CINR RSSI Relative Delay (RD) Round Trip Delay (RTD) Time of arrival (ToA) Angle | If the ranging purpose is to perform LBS measurement and/or the AMS wants to measure the location, it is included in the AAI_RNG-REQ LBS measurement metric transmitted by the AMS which is used to perform Non-GPS based LBS. measurement |
| Preferred update timer | It indicates the next preferred time to perform LBS measurement | If the ranging purpose is to perform LBS measurement and/or the AMS wants to measure the location, it may be included in the AAI_RNG-REQ. It may help the ABS to transmit the next AAI_PAG-ADV |

[Adopt the following rows to the Table 683 – parameters for AAI_RNG-RSP]

| Name | Value | Usage |
| --- | --- | --- |
| LBS measurement indication | 0b0 = No LBS measurement 0b1 = Needs LBS measurement report | Indicates whether the LBS measurement report is needed or not. |
| LBS Method | 0b0 = GPS-based Method 0b1 = Non-GPS-based Method | Indicates a LBS measurement method. It shall be included in the AAI_RNG-RSP when the LBS measurement indication is set (0b1). It may be included in the AAI_RNG-RSP if the ranging is for the LBS measurement. |
| LBS measurement metric | 0b000 = CINR 0b001 = RSSI 0b010 = Relative Delay (RD) 0b011 = Round Trip Delay (RTD) 0b100 = Time of arrival (ToA) 0b101 = Angle 0b110-0b111 = Reserved | Indicates a measurement parameter when Non-GPS based method is used. It shall be included when LBS Method = 0b1 |
| Preferred timer | Preferred time to report the next LBS-REP | It indicates the next preferred time to perform LBS measurement. It may be included in the AAI_RNG-RSP if the ranging is for the LBS measurement. |
| Threshold | Threshold to compare the previous and current LBS measurement metric | It indicates the threshold to compare the previous and current LBS measurement metric. It may be included in the AAI_RNG-RSP if the ranging is for the LBS measurement. |
| Resource info. to report AAI_LBS-REP | [TBD] | UL resource info.(region, MCS) to report AAI_LBS-REP MAC management message It shall be included in the AAI_RNG-RSP when the LBS measurement indication is set (0b1). |

[Modify15.2.6.12 as follows]

15.2.6.12 AAI_SCN-RSP

An AAI_SCN-RSP message shall be transmitted by the ABS either unsolicited or in response to an AAI_SCN-REQ message sent by an AMS. An ABS may transmit AAI_SCN-RSP to start AMS scan reporting with or without scanning allocation. If Scan Duration field contains a non-zero value, the system parameters which are carried through SFH shall not be changed during whole scanning duration specified in the AAI_SCN-RSP message. If Scan Duration field contains a non-zero value, scanning interval pattern which includes the AAI_SCN-RSP message replaces previous existing scanning interval pattern.

The AAS_SCN-RSP message can also be used for the purpose of LBS supporting. The ABS shall send AAI_SCN-RSP to initiate the LBS report, or to acknowledge the AAI_SCN-REP sent by the AMS.

The following parameters shall be included in the AAI_SCN-RSP message:

Scan duration

Duration (in units of subframes) of the requested scanning period.

Report mode

0b00:No report

0b01:Periodic report

0b10:Event-triggered report

0b11:One-time scan report

Report metric

Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric:

Bit 0: BS CINR mean

Bit 1: BS RSSI mean

Bit 2: Relative delay

Bit 3: BS RTD

Bits 4-5: Reserved; shall be set to zero

Report period

The period of MS's report of scanning result when the MS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the BS sends an unsolicited AAI_SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the MS receives the AAI_SCN-RSP message. For MS scanning request denied by AAI_SCN-RSP with Scan Duration set to zero, Report period is the number of frames that BS suggests to MS before transmitting next AAI_SCN-REQ

N_Recommended_BS_Index_indication

Indicates whether N_Recommended_BS_Index field is included in this message. This is valid only if Scan duration is not equal to zero.

N_Recommended_BS_Full_indication

Indicates whether N_Recommended_BS_Full field is included in this message. This is valid only if Scan duration is not equal to zero.

According to Indication values that BS indicates, the following parameters may be included in the AAI_SCN-RSP message:

Start super frame number

Represents the 6 least significant bits of the absolute frame number in which the first Scanning interval starts.

Start frame number

Represents start frame number in the Start super frame number.

0b00: The 1st frame in the super frame

0b01: The 2nd frame in the super frame

0b10: The 3rd frame in the super frame

0b11: The 4th frame in the super frame

Interleaving Interval

The period of AMS's normal operation which is interleaved between Scan Durations.

Scan Iteration

The number of iterating scanning interval.

N_Recommended_BS_Index

Number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message.

Configuration Change Count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

Neighbor_BS_Index

Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included.

N_Recommended_BS_Full

Number of neighboring BS to be scanned, which are using full 48-bit BS ID.

Recommended BS ID

Based on the value of N_Recommended_BS_Full, Recommended BS ID list for scan is included

The following parameters are included according to the usage of the AAI_SCN-RSP message:

Table XXX – parameters for AAI_SCN-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Purpose indication | Indicates the purpose of this message: 0b0: Scanning 0b1: LBS purpose |  |
| Scan duration | Duration (in units of subframes) of the requested scanning period | It shall be include only if the purpose indication is 0b0(Scanning) |
| LBS Mode | Used to indicate the start of LBS report or end of report. 0: LBS off, 1:LBS on | It shall be included only if the purpose indication is 0b1(LBS purpose) |
| LBS Method | 0b0: GPS-Based Method 0b1: Non-GPS-Based Method | It shall be included only if the purpose indication is 0b1(LBS purpose) In case of Method = 0b0, An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimate its location. In case of Method = 0b1, Non-GPS-Based Methods rely on the role of the serving and neighboring ABSs/ARSs. |
| Report Mode | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| Report Metric | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero | It shall be included unless LBS Method=0b0 (GPS-Based Method). It shall be used both scanning and LBS purpose |
| Absolute Position | GPS position metrics that serving ABS requests the AMS to report. 0: Long Format, 1: Short Format | It shall be include if LBS Method = 0b0 (GPS-Based Method). |
| N_Recommended_BS_index_indication | Indicates whether N_recommended_BS_Index field is included in this message. | This is valid only if scan duration is not equal to zero. |
| N_Recommended_BS_Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message | This is valid only if scan duration is not equal to zero. It shall be include only if the purpose indication is 0b0(Scanning) |
| Start super frame number | Represents the 6 least significant bits of the absolute frame number in which the first Scanning interval starts. | It shall be include only if the purpose indication is 0b0(Scanning) |
| Start frame number | Represents start frame number in the Start super frame number. 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame |  |
| Interleaving Interval | The period of AMS’s normal operation which is interleaved between Scan Durations. |  |
| Scan Iteration | The number of iterating scanning interval |  |
| N_Recommended_BS_Index | Number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. |  |
| Configuration Change Count for AAI_NBR-ADV | Referred in order to compress neighbor BSID |  |
| Neighbor_BS_index | BS index corresponds to position of BS in AAI_NBR-ADV message is included |  |
| N_Recommended_BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BS ID | It shall be include only if the purpose indication is 0b0(Scanning) |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, Recommended BS ID list for scan is included |  |
| Update timer(t) | next preferred update timer due to crossing the boundary of the cell | It shall be included only if the purpose indication is 0b1(LBS purpose) |
| Threshold | threshold to measure for moving distance threshold |  |

[Modify15.2.6.13 as follows]

15.2.6.13 AAI_SCN-REP

When the report mode is 0b10 (i.e., event triggered) in the most recently received AAI_SCN-RSP, the MS shall transmit a AAI_SCN-REP message to report the scanning results to its serving ABS after each scanning period if the trigger condition is met. For a periodic report (i.e., Report Mode is 0b01) and for One-time Scan Report (Report Mode is 0b11), the AMS reports the scanning results to its serving ABS at the time indicated in the AAI_SCN-RSP message except when it is in the scanning interval. For a periodic report (i.e., Report Mode is 0b01), the AMS stops reporting after all scanning intervals in the AAI-SCN-RSP message. The AMS shall include all available scanning results for the requested ABSs specified in the said AAI_SCN-RSP message. The MS may transmit an AAI_SCN-REP message to report the scanning results to its serving ABS at anytime.

In addition, the AMS shall send AAI_SCN-REP to report the measure result according to the measurement metric requested by the ABS, or to request measure the location of the AMS itself.

The following parameters shall be included in the AAI_SCN-REP message:

Report mode

Action code for an AMS's scan report of its measurement

0: Event-triggered report

1: Periodic report according to Scan report period of AAI_SCN-RSP

Report metric

Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be:

Bit 0: BS CINR mean

Bit 1: BS RSSI mean

Bit 2: Relative delay

Bit 3: BS RTD

Bits 4-5: Reserved; shall be set to zero

N_Neighbor_BS_Index_indication

Indicates whether N_Neighbor_BS_Index field is included in this message

N_Neighbor_BS_Full_indication

Indicates whether N_Neighbor_BS_Full field is included in this message.

According to Report metric that MS indicates, the AAI_SCN-REP message includes the following parameters:

BS CINR mean

The BS CINR Mean parameter indicates the CINR measured by the AMS from the serving ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period.

BS RSSI mean

The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the serving ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. The measurement shall be performed on the frame preamble and averaged over the measurement period.

Relative delay

This parameter indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. The value shall be interpreted as a signed integer in units of samples.

BS RTD

The BS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the serving ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs.

According to Indication values that MS indicates, the following parameters may be included in the AAI_SCN-RSP message:

N_Neighbor_BS_Index

Number of neighboring BS reported in this message and which are included in AAI_NBR-ADV message.

Configuration Change Count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

Neighbor_BS_Index

Based on the value of N_Neighbor_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included.

N_Neighbor_BS_Full

Number of neighboring BS reported in this message that are using full 48 bits BS ID.

Neighbor_BSID

Based on the value of N_Neighbor_BS_Full, full BSID (48-bit) is included.

The following parameters are included according to the usage of the AAI_SCN-REP message:

Table XXY – parameters for AAI_SCN-REP

| Name | Value | Usage |
| --- | --- | --- |
| Purpose indication | Indicates the purpose of this message: 0b0: Scanning 0b1: LBS purpose |  |
| LBS Method | 0b0: GPS-Based Method 0b1: Non-GPS-Based Method | It shall be included only if the purpose indication is 0b1(LBS purpose) In case of Method = 0b0, An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimate its location. In case of Method = 0b1, Non-GPS-Based Methods rely on the role of the serving and neighboring ABSs/ARSs. |
| Report mode | Action code for an AMS's scan report of its measurement 0: Event-triggered report 1: Periodic report according to Scan report period of AAI_SCN-RSP |  |
| Report metric | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero | It shall be included unless LBS Method=0b0 (GPS-Based Method). It shall be used both scanning and LBS purpose |
| N_Neighbor_BS_Index_indication | Indicates whether N_Neighbor_BS_Index field is included in this message |  |
| N_Neighbor_BS_Full_indication | Indicates whether N_Neighbor_BS_Full field is included in this message. | It shall be included only if the purpose indication is 0b0(Scanning) |
| BS CINR mean | The BS CINR Mean parameter indicates the CINR measured by the AMS from the serving ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period. | According to Report metric that AMS indicates |
| BS RSSI mean | The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the serving ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. The measurement shall be performed on the frame preamble and averaged over the measurement period. |  |
| Relative delay | This parameter indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. The value shall be interpreted as a signed integer in units of samples. |  |
| BS RTD | The BS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the serving ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs. |  |
| GPS PositionBS onlyd update timer due to crossing the boundary of the cellAAI_SCN-REP sent by the AMS. o 0000000000000000000000000000000000 | An AMS, which is equipped with GPS capability can utilize IEEE 802.16m MAC and PHY features to estimated its location and the following parameters are included: Longitude Latitude Altitude | It shall be included only if LBS Method=0b0 (GPS-Based Method). |
| N_Neighbor_BS_Index | Number of neighboring BS reported in this message and which are included in AAI_NBR-ADV message. | According to Indication values that AMS indicates |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |  |
| Neighbor_BS_Index | Based on the value of N_Neighbor_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. |  |
| N_Neighbor_BS_Full | Number of neighboring BS reported in this message that are using full 48 bits BS ID. | It shall be included only if the purpose indication is 0b0(Scanning) |
| Neighbor_BSID | Based on the value of N_Neighbor_BS_Full, full BSID (48-bit) is included. |  |
| Updated timer(t) | Next preferred update timer due to crossing the boundary of the cell | It shall be included only if the purpose indication is 0b1(LBS purpose) |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document,” July 2009

- IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 200