| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of E-MBS Zone Specific Pilot Patterns for MBSFN |  |
| Date Submitted | 2009-09-18 |  |
| Source(s) | Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu, Zirong Li, Dihao Ning ZTE Corporation | Voice: Email: {liang.ting12, sun.bo1}@zte.com.cn |
| Re: | IEEE P802.16m/D1: EMBS DG |  |
| Abstract | The contribution proposes the text of EMBS pilot pattern to be included in amendment document. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# E-MBS Zone Specific Pilot Pattern for MBSFN

# Ting Liang, Bo Sun, Dengkui Zhu, Kaiying Lv, Ying Liu

Zirong Li, Dihao Ning

# ZTE Corporation

# Introduction

In current SDD description, E-MBS zone specific pilot is transmitted for MBSFN transmissions. The pilot structures of E-MBS should be supported up to two streams. In SFN transmission, the Delay spread is higher than typical unicast scenarios, and the unicast pilot patterns may be not sufficient. So a new pilot pattern designs is needed for E-MBS zone.

This contribution proposes two single-pilot-pattern solutions for E-MBS zone for selection.

# Simulation Result

- 4 E-MBS specific pilot patterns from contributions for #63 meeting

  - A: Pilot pattern defined in C80216m-09/1688 (ITRI),

  - C: Pilot pattern defined in C80216m-09/1951 (Samsung)

  - D: Pilot pattern 1 defined in C80216m-09/2052r1 (LGE)

  - E: Pilot pattern 2 defined in C80216m-09/2052r1 (LGE)

- 3 more EMBS specific pilot patterns added before DG requested deadline

  - F:  Pilot pattern 1 proposed by ZTE

  - G:  Pilot pattern 2 proposed by ZTE

  - H:  Pilot pattern defined in C80216m-09/0929 (Intel)

Aligned simulation conditions are listed as below.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| Resource Unit | RU (18x6) |
| Permutation | Tone-pair Logical DRU |
| Data Size | 4PRU |
| Channel Code | LTE Turbo |
| MCS | QPSK1/2, 3/4; 16QAM1/2,3/4; 64QAM1/2 |
| Channel decoding | Max-log-Map, max 8 iteration |
| Channel condition | ISD1500m and ISD5Km; 3km/h and 120km/h |
| Channel estimation method | 2D-MMSE, Window size = 3PRU |
| The number of antennas | 2Tx, 2Rx |
| MIMO schemes | Rate-1 SFBC@ISD1500&ISD5000 Rate-2 SM@ISD1500 only |
| MIMO Receiver | MRC(SFBC ),Soft-MMSE(SM) |
| Scenarios | Noise limited |
| Pilot boosting | 5dB |
| Performance matrix | BLER Vs. SNR and SE Vs. SNR |

Simulation results are listed as below.

ISD1.5km, 120km/h, 2*2 SFBC, QPSK1/2

ISD1.5km, 120km/h, 2*2 SFBC, 16QAM3/4

ISD1.5km, 120km/h, 2*2 SFBC, 64QAM1/2

ISD1.5km, 120km/h, 2*2 SM, QPSK1/2

ISD1.5km, 120km/h, 2*2 SM, 16QAM3/4

ISD1.5km, 120km/h, 2*2 SM, 64QAM1/2

ISD5km, 3km/h, 2*2 SFBC, QPSK1/2

ISD5km, 3km/h, 2*2 SFBC, 16QAM1/2

# References

[1] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[2] “E-MBS Zone Specific Pilot Patterns for MBSFN”, IEEE C802.16m-09/0929

# Text proposal for inclusion in the 802.16m SDD

Option 1:

-------------------------------  Option 1 Text Start  ---------------------------------------------------------------

11.5.3.2 E-MBS zone specific pilot for MBSFN

E-MBS zone specific pilot is transmitted for MBSFN transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilot, that’s, common inside one E-MBS zone but different between neighboring E-MBS zones, is configured. Synchronous transmissions of the same contents with common pilot from multiple ABS in one MBS zone would result in correct MBSFN channel estimation. The E-MBS zone specific pilot streams depends on the maximum number of streams within the E-MBS zone.

Pilot structures/patterns should be supported up to two streams.

The definitions of the E-MBS zone specific pilots are FFS.

The E-MBS pilot pattern is shown in Figure xxx.

Figure xxx Proposed E-MBS pilot pattern in a RU for 2 Stream

-------------------------------  Option 1 Text End  -----------------------------------------------------------------------

Option 2:

-------------------------------  Option 2 Text Start  ---------------------------------------------------------------

11.5.3.2 E-MBS zone specific pilot for MBSFN

E-MBS zone specific pilot is transmitted for MBSFN transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilot, that’s, common inside one E-MBS zone but different between neighboring E-MBS zones, is configured. Synchronous transmissions of the same contents with common pilot from multiple ABS in one MBS zone would result in correct MBSFN channel estimation. The E-MBS zone specific pilot streams depends on the maximum number of streams within the E-MBS zone.

Pilot structures/patterns should be supported up to two streams.

The definitions of the E-MBS zone specific pilots are FFS.

The E-MBS pilot pattern is shown in Figure xxx.

Figure xxx Proposed E-MBS pilot pattern in a RU for 2 Stream

-------------------------------  Option 2 Text End  -----------------------------------------------------------------------