| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed E-MBS Zone Specific Pilot for MBSFN (E-MBS) |  |
| Date Submitted | 2009-09-17 |  |
| Source(s) | Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting ITRI | Voice : 886-3-5914854 E-mail: yutaohsieh@gmail.com E-mail: jefferywu@itri.org.tw |
| Re: | IEEE P802.16m/D1: EMBS DG |  |
| Abstract | The contribution proposes the text and figures for E-MBS zone specific pilots. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed E-MBS Zone Specific Pilot Patterns

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

In current SDD [1], the basic concept about E-MBS zone specific pilot is described in 11.5.3.2. However, the E-MBS zone specific pilot patterns have not yet been defined. According to current EMD [3], MBSFN channel models for inter-site distance (ISD) 1.5km and 5km have much larger delay spread than ITU-R VA and PB channel. Therefore, we may need to increase pilot density to accommodate severe frequency selectivity. In this contribution, we propose the E-MBS zone specific pilot patterns and corresponding text for AWD [2].

# References

[1] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document.”

[2] IEEE 802.16m-08/0010r2, “IEEE 802.16m Amendment Working Document.”

[3] IEEE 802.16m-07/004r5, “IEEE 802.16m Evaluation Methodology Document.”

# Text proposal for the 802.16m amendment

Insert the following text and figure in IEEE 802.16m/D1(E-MBS DG) in section 15.3.5.4.3:

============================== Proposed text start ===================================

E-MBS zone specific pilots are transmitted for multi-cell multicast broadcast single frequency network (MBSFN) transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilots that are common inside one E-MBS zone but different between neighboring E-MBS zones are configured. Synchronous transmissions of the same contents with common pilot from multiple ABS in one MBS zone would result in correct MBSFN channel estimation.The E-MBS zone specific pilots depend on the maximum number of Tx streams within the E-MBS zone. Pilot structures/patterns should be supported up to two Tx streams. The E-MBS zone specific pilot pattern is shown in Fig. ZZZ.

Fig. ZZZ E-MBS zone specific pilot pattern

============================== Proposed text end ===================================