| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_NBR-ADV message format (15.2.6.10) |  |
| Date Submitted | 2009-09-21 |  |
| Source(s) | Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Jaehee Cho, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Reply Comments regarding the 474 marked (unresolved) comments Section 15.2.6.10 AAI_NBR-ADV |  |
| Abstract | This contribution provides detailed format of the AAI_MC-ADV and AAI_NBR-ADV message |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_NBR-ADV message format (15.2.6.10)

Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Jaehee Cho, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides detailed format of the AAI_NBR-ADV message.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.6 AAI MAC Management Messages

15.2.6.10 AAI_NBR-ADV

…

Each AAI_NBR-ADV message carries

- AAI_NBR-ADV change count

- The number of total cell types

- Segment information for this AAI_NBR-ADV message

- System information of ABSs from one or more cell types, which is of variable size.

- Starting BS Index (i.e. this is the index offset from the last BS of the previous fragment. If this is first fragment, the Starting Index is 0. Hence, each fragment has one Index which corresponds to the first BS in that fragment.)

For each cell type, shared configuration parameters are carried as follows

- FA information

- BW information

- Duplex mode

- MAC protocol version

Within each cell type, a list of ABSs are provided and the following parameters are carried for each ABS

- Index of neighbor ABS in the whole neighbor list maintained by serving ABS

- 48bit BS-ID

- ABS A-Preamble index

- indication whether full system information or partial information is carried for this ABS, which includes

- SFH information

- ESI information

- Index to MC configuration,

where for ABS of macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type); and for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in the AAI_NBR-ADV message.

Table XXX—AAI_NBR-ADV message format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 8 | NBR-ADV change count |
| Number of cell types | 2 | — |
| Cell type in this message | 2 | — |
| Total Fragmentation | TBD | Indicates the total number of fragmentations |
| Fragmentation Index | TBD | Indicates the current fragmentation index. |
| Starting BS Index | 8 | BS index is calculated from this parameter. |
| FA information | 8 | — |
| BW information | 8 | — |
| Duplex mode | 2 | — |
| BS number (M) | TBD | Total number of BSs to be included in this segment |
| SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with TBD format 0b10: no SFH included 0b11: no specific information provided For macrocell ABS, the bitmap shall be either 0b00 or 0b01. |
| if (SFH_encoding_format != 0b11) { |  |  |
| for (i=0; i<M; i++) { | — | — |
| BSID | 48 | — |
| A-PREAMBLE info | TBD | — |
| S-SFH change count | 4 | The change count of this BS’s S-SFH IE |
| If( SFH_encoding_format == 00) { | — | — |
| Control_bitmap | 3 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 111. |
| If( Control_bitmap[0] ==1 ) { | — | — |
| SFH Subpkt 1 | TBD | — |
| } | — | — |
| If( Control_bitmap[1] ==1 ) { | — | — |
| SFH Subpkt2 | TBD | — |
| } | — | — |
| If( Control_bitmap[2] ==1 ) { | — | — |
| SFH Subpkt3 | TBD | — |
| } | — | — |
| } | — | — |
| if (SFH_encoding_format == 01) { | — | — |
| Delta information | — | Delta encoding, w.r.t. the serving BS |
| } | — | — |
| } |  |  |
| } | — | — |
| } | — | — |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------