| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MC-ADV message format (15.2.7.2.3.1.1) |  |
| Date Submitted | 2009-09-17 |  |
| Source(s) | Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Jaehee Cho, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Reply Comments regarding the 474 marked (unresolved) comments Section 15.2.7.2.3.1.1 AAI_MC-ADV |  |
| Abstract | This contribution provides detailed format of the AAI_MC-ADV and AAI_NBR-ADV message |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MC-ADV message format (15.2.7.2.3.1.1)

Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Jaehee Cho, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides detailed format of the AAI_MC-ADV message.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.6 AAI MAC Management Messages

15.2.6.x7.2.3.1.1 AAI_MC-ADV (Multi-Carrier Advertisement) Message

Table 707XXX —AAI_MC-ADV MAC Management Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_MC-ADV_Message_format() { | — | — |
| Management Message Type = MM | 8 | — |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| Uniformity Flag | 1 | All carriers have the same Protocol version, Carrier type, Duplexing mode and Transmit power. |
| Physical carrier index of current carrier | 6 | The carrier that the ABSs broadcasts this message. |
| for (i=1; i<=Number of Carrier Groups; i++) { | — | — |
| Multi-carrier configuration index | 6 | Index associated to Table 648 |
| Start frequency assigment index | 6 | Frequency assignment index of the first carrier in carrier group i |
| Uniformity Flag | 1 | 1: All carriers have the same Protocol version, Carrier type, Duplexing mode and Transmit power. 0: Otherwise |
| if (Uniformity Flag==1) { |  |  |
| Protocol version | 2 | 00: IEEE 802.16-2009 only mode 01: IEEE 802.16m and IEEE 802.16-2009 mixed mode 10: IEEE 802.16m only mode 11: reserved |
| Carrier type | 1 | 0: for fully configured 1: for partially configured |
| Dupexing mode | 1 | 0: for TDD 1: for FDD |
| Transmit power | 6 | Maximum transmit power level |
| } |  |  |
| Number of carriers | 4 | Number of carriers in carrier group i |
| for (j=i1; j<=Number of carriers’; j++){ | — | — |
| Physical carrier index | 63 | — |
| Preamble index | 6 | — |
| if (Uniformity flag==10) ¦¦ (i==1 && j==1) { | — | — |
| Protocol version | 2 | 00: IEEE 802.16-2009 only mode 01: IEEE 802.16m and IEEE 802.16-2009 mixed mode 10: IEEE 802.16m only mode 11: reserved |
| Carrier type | 1 | 0: for fully configured 1: for partially configured |
| Dupexing mode | 1 | 0: for TDD 1: for FDD |
| Transmit power | 6 | Maximum transmit power level |
| } | — | — |
| } | — | — |
| } | — | — |
| Number of neighbor MC BSs | 8 | — |
| for (i=0; i< Number of neighbor MC BSs; i++) { | — | — |
| Multi-carrier configuration index | 6 | Index associated to Table 648 |
| for (j=0; j< Number of carriers; j++) { | — | Number of carriers is derived from Multi-carrier configuration index |
| BSID | 48 | This BSID shall be presented in the AAI_NBR-ADV message. The BSID shall be presented in an ascending order of physical carrier index. |
| } | — | — |
| } | — | — |
| } | — | — |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------