| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of AAI_MC-ADV |  |
| Date Submitted | 2009-09-18 |  |
| Source(s) | Jaesun Cha, Juhee Kim, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Reply Comment on IEEE 802.16-09/0047r4 |  |
| Abstract | This contribution proposes the modification of AAI_MC-ADV message |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of AAI_MC-ADV Message

Jaesun Cha, Juhee Kim, Soojung Jung, Eunkyung Kim, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

AAI-MC-ADV message contains Protocol version, Carrier type, Duplexing mode and Transmit power of a certain carrier according to the value of ‘Uniformity Flag’ field, and ‘Uniformity Flag’ is used to indicate whether all carrier in all carrier groups have the same configuration. If an ABS supports a partially configured carrier, then all the parameters shall be included for all carriers. If ‘Uniformity Flag’ is used for each carrier group, we can avoid the redundant occurrence of the same parameters.

In addition, we modified the format of Table 707 to keep the consistency with other tables defined for MAC management messages.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 707 as follows]

| Name | Value | Usage |
| --- | --- | --- |
| Number of Carrier Groups | 0 ~ 15 | Groups of contiguous carriers |
| Uniformity Flag |  |  |
| Physical carrier index of current carrier | 0 ~ 63 | The carrier that the ABS broadcasts this message |
| Uniformity Flag | 1: all carriers in a carrier group have the same protocol version, carrier type, duplexing mode and transmit power 0: otherwise | This field shall be included for each carrier group |
| Multi-carrier configuration index | 0 ~ 63 | Index associated to Table 648 This field shall be included for each carrier group |
| Start frequency assignment index | 0 ~ 63 | Frequency assignment index of the first carrier in a carrier group This field shall be included for each carrier group |
| Number of carriers | 0 ~ 15 | Number of carriers in a carrier group This field shall be included for each carrier group |
| Physical carrier index | 0 ~ 63 | This filed shall be included for each carrier in each carrier group |
| Preamble index | 0 ~ 63 | This filed shall be included for each carrier in each carrier group |
| Protocol version | 00: IEEE 802.16-2009 only mode 01: IEEE 802.16m and IEEE 802.16-2009 mixed mode 10: IEEE 802.16m only mode 11: reserved | This field is included only for the first carrier in a carrier group if ‘Uniformity Flag’ of the carrier group is set to 1. Otherwise, this field is included for each carrier in a carrier group |
| Carrier type | 0: for fully configured 1: for partially configured | This field is included only for the first carrier in a carrier group if ‘Uniformity Flag’ of the carrier group is set to 1. Otherwise, this field is included for each carrier in a carrier group |
| Duplexing mode | 0: for TDD 1: for FDD | This field is included only for the first carrier in a carrier group if ‘Uniformity Flag’ of the carrier group is set to 1. Otherwise, this field is included for each carrier in a carrier group |
| Transmit power | 0 ~ 63 | Maximum transmit power level This field is included only for the first carrier in a carrier group if ‘Uniformity Flag’ of the carrier group is set to 1. Otherwise, this field is included for each carrier in a carrier group |

---------------------------------------------------------End of the Text----------------------------------------------------------