| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Clarification on UL Multi-BS MIMO section (15.3.14.4) |  |
| Date Submitted | 2009-09-19 |  |
| Source(s) | Rong Ran, Jihyung Kim, Suchang Chae, Wooram Shin and Dongseung Kwon | Sunny_rr@etri.re.kr |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes clarification on UL Multi-BS MIMO section |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clarification on the UL Multi-BS MIMO section (15.3.14.4)

Rong Ran, Jihyung Kim, Suchang Chae, Wooram Shin and Dongseung Kwon

ETRI

# Introduction

The concurrent UL multi-BS MIMO is not clear in the current standard. A UL sounding should be included in the feedback header for channel measurement. Moreover, the quantized RSSI difference should be feedback to the serving ABS for selecting AMSs. Thus, it is proposed to add clarification text on UL multi-BS MIMO section.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

- Proposed text

-------------------------------------------------------Start of the Text-------------------------------------------------------

[Add the following blue/underlined texts into section 15.3.14.4 on page 232 as follows]

15.3.14.4 UL Multi-BS MIMO

### 15.3.14.4.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques based on the MIMO schemes defined in << Section 15.3.10 >> applicable with UL MIMO modes 2 and 4 with adaptive codebook precoding, with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

### 15.3.14.4.1.1 DL Signaling

Table 55 shows the control parameters for UL multi-BS MIMO.

Table 55 - Control parameters for UL multi-BS MIMO

| - Parameter | - Description | - Value | - Control channel (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - UCCE | - Uplink codebook coordination enable | - 0b0: disable - 0b1: enable | - MAC management control message | - Indicates operation of PMI combination |
| - UTRU | - Uplink target resource unit | - TBD - Cell and user cyclic shift separation [TBD] | - MAC management messge - Sounding commend A-MAP | - Indicates resource units for measurement - Indicates AMSs to send sounding signals using all subcarriers within sounding allocation |
| - PMImin | - PMI minimizing interference |  | - MAC management control message | - Enable when UCCE=0b1 - [Preferred PMI minimizing inter-cell interference for the neighboring ABS] |

### 15.3.14.4.1.2 UL Signaling

Table 56 shows the feedback information for UL multi-BS MIMO.

Table56 - Feedback information for UL multi-BS MIMO

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is a member ID assigned to this ABS |
| Periodic feedback | Uplink sounding | Uplink sounding on the subband indicated by the ABS in sounding command A-MAP IE |
| Event driven feedback | RSSI difference level | Report RSSIdiff for ABS selecting AMS which should send sounding signals |

### 15.3.14.4.1.3 Operation procedure

Single BS precoding with Multi-BS Coordination proceeding is performed by combining two egoistic and altruistic PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells.  it can be used to mitigate inter-cell interference (ICI) without much increase of feedback overhead for CL SU MIMO. One of PMIs maximizes the transmission power of the serving cell; another minimizes interference generated to the neighboring cell.

The operation procedure of PMI combination follows the steps below:

- PMI combination may be triggered by an ABS in an unsolicited manner. ABSs may send UCCE (UL codebook coordination enable) in a MAC management control message to AMSs indicating operation with PMI combination.

- Once an AMS receives a this MAC management control message, it shall send a periodic feedback header with the requested information. This information may include a Temp_BSID (a member ID) and the corresponding RSSI difference level.

The BS_ID of the neighboring ABS which suffers the most interference caused by the AMS is selected by the AMS i through the following criterion: AMSs shall calculate RSSI difference between two RSSIs received from the serving and neighboring ABSs. If the RSSI difference value of an AMS is smaller than a threshold T, the AMS can select the neighboring ABS which suffers the most interference from it via the following criterion:

,

(176)

where i is the index of the AMS, p is the index of the serving ABS and q is the index of the neighboring ABS. RSSIp,iindicates the RSSI measurement result measured by the AMS i from the received  preamble transmitted by the ABS p.

For determining the level of RSSI difference denoted as RSSIdiff, the following procedure shall be performed by the AMS i after selecting the neighboring ABS:

(177)

Where T0 is larger than T. b1 and b2 are two quantized values of RSSI difference. Therefore, only one feedback bit is necessary.

- After receiving feedback from multiple AMSs, the serving ABS should select several AMSs according to RSSI difference level and the number of AMSs supported by the sounding channel. The selected AMSs should be indicated by the serving ABS to send sounding signals using all subcarriers within sounding allocation via UTRU (unit target resource unit) in a sounding command A-MAP. The UL sounding channel for multiple AMSs of multiple cells can be multiplexed through cell cyclic shift separation and user cyclic shift separation indicated in a sounding command A-MAP IE.

- By using UL sounding signals, the serving and neighboring ABSs shall measure channels of selected AMSs. Upon the channel measurements, the serving and neighboring ABSs shall determine PMI maximizing transmission power and PMImin minimizing interference, respectively. Then the neighboring ABS shall inform PMImin to the serving ABS via the backhaul network.

Two PMIs of the predefined codebook, managementone of which maximizes the transmissionpower for the serving ABS and another of which minimizes the interference power for the neighboring ABS nfrom (176) should be selected through based on the following equations procedure:

,                                                        (177)(178)

where  is a PMI.  and  denote information of channels from the AMS to its serving and corresponding neighboring ABSs, respectively.

- The serving ABS shall inform the PMI in A-MAP IE and PMImin in a MAC control message to the AMS.

The following procedure should be performed by the AMS to determine a PCR (PMI combination ratio) (K1, K2, K3) based on two fixed RSSI difference criteria (TBD).

(178) (179)

The transmitted precoder  combines combining two precoders (,) should be generated as follows， tcorresponding to PMImin and PMImax through

(179) (180)

--------------------------------------------------------End of the Text-------------------------------------------------------