| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for E-MBS Configuration indicators |  |
| Date Submitted | 2009-09-19 |  |
| Source(s) | Qi Wu, Kaushik Josiam, Jaejeong Shim Samsung Electronics Ke Yao, Kaiying Lv, Bo Sun ZTE Corporation | Email : qi.wu@samsung.com k.josiam@sta.samsung.com brian.shim@samsung.com E-mail: lv.kaiying@zte.com.cn |
| Abstract | This contribution proposes text for E-MBS Configuration indicators in the AAI_SCD |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for E-MBS Configuration indicators

Qi Wu, Kaushik Josiam, Jaejeong Shim

Samsung Electronics

Ke Yao, Kaiying Lv, Bo Sun

ZTE

The following text is proposed to add in 15.2.6.29 of 802.16m AWD.

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

15.2.6.29 AAI_System Configuration Descriptor(SCD) Message

E-MBS configuration information is transmitted periodically in AAI-SCD message and indicates the distribution information of E-MBS superframes and subframes, and the multiplexing information of E-MBS and unicast which both E-MBS AMSs and unicast AMSs should know. The E-MBS configuration information carried on the AAI-SCD message of a specific carrier shall only contain the information of the E-MBS services carried on the corresponding carrier.

The distribution of E-MBS at superframe level can be described by E-MBS indication bits and superframe bitmap. The E-MBS resource indicator gives not only the subframe level E-MBS distribution but also the E-MBS and unicast multiplexing pattern, which repeats in all frames of one superframe.

- An N-bit superframe bitmap indicates which superframe has E-MBS subframe(s) and N is assumed as periodicity of E-MBS configuration information.

- E-MBS subframe resource indicator indicates a particular E-MBS and unicast multiplexing pattern in a predefined pattern set in one frame. E-MBS and unicast multiplexing pattern describes the resources occupied by E-MBS at LRU level in each subframe.

2-bit indication is used to identify different combination of superframe bitmap(s) and E-MBS and unicast multiplexing pattern index(s), as illustrated in table XXX.

Table XXX1 –E-MBS configuration information in the AAI_SCD (This table should be merged into current AAI_SCD message)

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Format of E-MBS configuration information { |  |  |
| Indication bits | 2 | 0b00: No E-MBS during N superframes 0b01: each superframe has the same E-MBS and unicast multiplexing pattern 0b10: some superframes has the same E-MBS and unicast multiplexing pattern 0b11: some superframes has E-MBS, but with different E-MBS and unicast multiplexing pattern N is the periodicity of the E-MBS configuration information. |
| If (Indication bits >0b00) { |  |  |
| E-MBS Subframe Resource Indicator | 3 | Indicate the subframe resources in every frame of the superframe that carry E-MBS traffic. Given an EMBS_SUBFRAME_RESOURCE_INDICATOR value, the indices of the resources within the frame where E-MBS traffic is present is indicated via a predetermined look-up table (See Table XXX2). |
| } |  |  |
| If (Indication bits > 0b01) { | - | - |
| Superframe bitmap | N | indicates which superframe has E-MBS subframe(s) and N is assumed as periodicity of E-MBS configuration information. |
| } | - | - |
| If (Indication bits > 0b10) { | - | - |
| E-MBS Subframe Resource Indicators | 3*(K-1) | K-1 E-MBS Resource Indicators, where K is the number of “1” bits in superframe bitmap |
| } |  |  |
| } |  |  |

An example of E-MBS Resource Indicator is as follows.

If the E-MBS resource indicator <= 0b011 (This value could be changed), it is TDM, Otherwise it is FDM. The lookup table for TDM is as follows.

Table XXX2 –E-MBS resource indicator lookup table

| E-MBS_Resource_Indicator | EMBS RESOURCE indicated SF: Indicates the subframe reserved for E-MBS traffic FP_MBS: Frequency Partition reserved for E-MBS |
| --- | --- |
| 000 | SF = Last SF of the DL Zone FP_MBS = ALL Frequency Partitions |
| 001 | SF = Last 2 SFs of the DL Zone FP_MBS = ALL Frequency Partitions |
| 010 | SF = Last 3 SFs of the DL Zone FP_MBS = ALL Frequency Partitions |
| 011 | SF = Last 4 SFs of the DL Zone FP_MBS = ALL Frequency Partitions |
| 100 | SF = Last SF of the DL Zone FP_MBS = FP0 LRU_MBS = the first 4 CRUs |
| 101 | SF = Last 2 SFs of the DL Zone FP_MBS = FP0 LRU_MBS = the first 8 CRUs |
| 110 | SF = Last 3 SFs of the DL Zone FP_MBS = FP0 LRU_MBS = the first 12 CRUs |
| 111 | SF = ALL SFs of the DL Zone FP_MBS = FP0 LRU_MBS = the first 16 CRUs |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------