| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS MAP |  |
| Date Submitted | 2009-09-19 |  |
| Source(s) | Qi Wu, Kaushik Josiam, Jaejeong Shim Samsung Electronics | Email : qi.wu@samsung.com k.josiam@sta.samsung.com brian.shim@samsung.com |
| Re: |  |  |
| Abstract | This contribution defines the text and format of E-MBS MAP |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for E-MBS MAP

Qi Wu, Kaushik Josiam, Jaejeong Shim

Samsung Electronics

The following text is proposed to add in 15.3.6.2 of P802.16m/D1.

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

15.3.6.2 DL Control Channels

15.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries control information for enhanced multicast broadcast service.  The E-MBS MAP consists of non-user specific control information. Non-user specific control information is further divided into basic assignment information, persistent assignment information and composite persistent assignment information. The E-MBS MAP, when present, is transmitted at the starting place of an MSI.

The format of E-MBS MAP is as follows. The E-MBS_DATA_IE can be the Basic Assignment E-MBS DATA IE, the Individual Persistent Assignment E-MBS DATA IE, or the Composite Persistent Assignment E-MBS DATA IE.

Table XXX. E-MBS MAP Format

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| MSI Length (m) | [TBD] | The length of an MSI is 2m+3 superframes |
| E-MBS_IE_Count | [TBD] | Number of E-MBS_DATA_IE count |
| for (i=0; i≤E-MBS_IE_Count; i++) { | - | - |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| } |  |  |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------

The following text is proposed to add in 15.3.6.3 of P802.16m/D1.

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

15.3.6.3 Resource Mapping of DL Control Channels

15.3.6.3.3 Enhanced- Multicast Broadcast Services MAP (E-MBS MAP)

The coding chain for non-user specific E-MBS DATA IEs is TBD.

The E-MBS DATA IEs is encoded using TBCC with rate [TBD].

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------