| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Control Information Elements |  |
| Date Submitted | 2009-09-19 |  |
| Source(s) | Qi Wu, Kaushik Josiam, Jaejeong Shim Samsung Electronics | Email : qi.wu@samsung.com k.josiam@sta.samsung.com brian.shim@samsung.com |
| Re: |  |  |
| Abstract | This contribution defines the text and format of Control Information Elements |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Control Information Elements

Qi Wu, Kaushik Josiam, Jaejeong Shim

Samsung Electronics

The following text is proposed to add in 15.3.6.5 of P802.16m/D1.

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

15.3.6.5 DL Control Information Elements

15.3.6.5.5 E-MBS Control Information Elements

E-MBS DATA IE is defined as the basic element of E-MBS control.

15.3.6.5.5.1 Basic assignment E-MBS DATA IE

Table XXX4. Describes the fields in a basic assignment E-MBS DATA IE used for indicating resource assignment for E-MBS data in the DL.

Table XXX4. Basic Assignment E-MBS DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| E-MBS IE Type | 2 | E-MBS Basic Assignment IE |
| FID | 4 | E-MBS Flow ID |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| E-MBS MEF | 2 | E-MBS MIMO Encoding Format 0b00 – SFBC 0b01 – VE 0b10 – HE 0b11 - reserved |
| If (E-MBS MEF == 0b01){ |  |  |
| Mt | 1 | Number of streams 0b0 = 1 stream 0b1 = 2 streams |
| } |  |  |
| MBS Frame offset & Resource Indexing | [TBD] | Include the location and allocation size, |
| Padding | variable | Padding to reach byte boundary |
| } | - | - |

A 16 bit CRC is generated based on the contents of the E-MBS Basic Assignment IE. The CRC is masked by the multicast Station ID.

E-MBS MEF: MIMO encoder format for E-MBS data

MBS Frame offset & Resource Indexing: [TBD]

15.3.6.5.5.2 Individual Persistent Assignment E-MBS DATA IE

Table XXX5. Describes the fields in a persistent assignment E-MBS DATA IE used for indicating resource assignment for E-MBS data in the DL.

Table XXX5. Individual Persistent Assignment E-MBS DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| E-MBS IE Type | 2 | E-MBS Individual Persistent IE |
| FID | 4 | E-MBS Flow ID |
| Allocation Lifetime | [4] | The number of remaining E-MBS data transmissions that this allocation is valid. |
| Allocation Period | [6] | Frame offset for multiple transmission instances (in number of frames) |
| ISizeOffset | 5 | Offset used to compute burst size index (see 15.3.12.1.2) |
| E-MBS MEF | 2 | E-MBS MIMO Encoding Format 0b00 – SFBC 0b01 – VE 0b10 – HE 0b11 - reserved |
| If (E-MBS MEF == 0b01){ |  |  |
| Mt | 1 | Number of streams 0b0 = 1 stream 0b1 = 2 streams |
| } |  |  |
| MBS Frame offset & Resource Indexing | [TBD] | Include the location and allocation size, |
| Padding | variable | Padding to reach byte boundary |
| } | - | - |

A 16 bit CRC is generated based on the contents of the E-MBS Individual Persistent IE. The CRC is masked by the multicast Station ID.

Allocation Lifetime: the time interval indicated by the allocation lifetime. During the lifetime, an AMS does not need to decode the E-MBS MAP and wakes only at the E-MBS data.

Unlike the persistent A-MAP IE, which only appears at the starting and ending place of a persistent resource allocation, the E-MBS DATA IE for a persistent allocation has to be transmitted each time in an MSI having the associated E-MBS data due to the multicast/broadcast behavior. Figure YYY1 illustrates the behavior of an AMS under Allocation Lifetime = 0b101.

Figure YYY1. Behavior of an AMS under Allocation Lifetime = 0b101.

Allocation Period: the number of frame offsets by which different transmission instances of a single content are separated within an MSI as shown in Figure YYY2.

Figure YYY2. Example of allocation periods for differ E-MBS contents.

15.3.6.5.5.3 Composite Persistent Assignment E-MBS DATA IE

Table XXX3. describes the fields in a composite persistent assignment E-MBS DATA IE used for indicating resource assignment for E-MBS data in the DL.

Table XXX6. Composite Persistent Assignment E-MBS DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| E-MBS IE Type | 2 | E-MBS Composite Persistent Assignment IE |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| if (Persistent Flag == 1) { |  |  |
| Allocation Lifetime Indicator | 1 | If Allocation Lifetime Indicator is 1, it indicates that allocation lifetime is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation lifetime as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Lifetime Indicator == 1) { | - | - |
| Allocation Lifetime | 4 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Allocation Period | [6] | Frame offset for multiple transmission instances (in number of frames) |
| } |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that ISizeOffset is explicitly assigned for this allocation. Otherwise, this allocation will use the same ISizeOffset as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| } |  |  |
| E-MBS MEF | 2 | E-MBS MIMO Encoding Format 0b00 – SFBC 0b01 – VE 0b10 – HE 0b11 – reserved |
| If (E-MBS MEF == 0b01){ |  |  |
| Mt | 1 | Number of streams 0b0 = 1 stream 0b1 = 2 streams |
| } |  |  |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| MBS Frame offset & Resource Indexing | [TBD] | Include the location and allocation size, |
| } |  |  |
| } else if (RAI == 0b01) { |  |  |
| MBS Frame offset & Resource Indexing | [TBD] | It indicates the start position of resource region for this subburst |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------