| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for DSx MAC Message for E-MBS Transmission |  |
| Date Submitted | 2009-09-19 |  |
| Source(s) | Nan Li,Kaiying Lv ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: lv.kaiying@zte.com.cn |
| Re: | IEEE 802.16m-09/0037, “Call for Contributions on Project 802.16m Amendment Content”. Target topic: “E-MBS”. |  |
| Abstract | This contribution proposes E-MBS transmission for multicarrier deployment. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DSx MAC Message for E-MBS Transmission

Nan Li，Kaiying Lv

ZTE Corporation

The following text is proposed to add in 15.2.5.8 of 802.16m AWD.

---------------------------------------------------------Start of the Text--------------------------------------------------------

15.2.5.8 DSx MAC Management Message

The AMS chooses the channel package that it needs to subscribe to based on the higher layer reception of the EMBS program guide from the EMBS server in the network. The EMBS server can then use network mechanisms out of scope of this specification to trigger the network initiated DSA to this AMS via the serving BS.

The AMS and the ABS communicate through AAI_DSA-REQ/RSP messages to learn EMBS/MBS Service Flow Parameters. The parameters sent will exactly indicate to the AMS whether to listen to EMBS on a 16m zone or to listen to MBS on a legacy MBS zone. It also includes E-MBS multicast STIDs and FIDs, E-MBS zone IDs, E-MBS carrier information (physical carrier index). The AMS, which has acquired the carrier information of the specific E-MBS service from AAI_DSA message, shall proceed to decode AAI_SCD message sent on the corresponding carrier for E-MBS configuration information.

If the carrier indicated in AAI_DSA-REQ/RSP message has not been assigned to the AMS or been activated, the ABS can decide to assign and activate the carrier for the AMS. The assignment of E-MBS carriers can also be done by using AAI_CM-RSP message and activate the carrier with AAI_CM-CMD message. The AMS shall keep receiving the specific E-MBS service on the assigned carrier unless mapping relationship update is informed in AAI_DSC-REQ/RSP message.

15.2.5.8.2 DSA-RSP message contents

- MBS Mode = legacy or EMBS

- If (MBS mode = legacy)

  - 16e MCIDs and LCIDs

  - CS of these MCIDs

  - SF parameters of these MCIDs

  - 16e MBS Zone-ID

  - 16e MBS carrier info. Optionally included only if the MBS carrier is different from the serving carrier.

- If (MBS mode = EMBS)

  - 16m STIDs package with available FIDs

  - CS parameters for each of the STID

  - SF parameters for each of the <STID, FID> combination

  - EMBS Zone-ID

  - EMBS RF carrier info. Optionally included only if the EMBS carrier is different from the serving carrier.

.

---------------------------------------------------------End of the Text---------------------------------------------------------