| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Zone specific pilot patterns |  |
| Date Submitted | 2009-09-19 |  |
| Source(s) | Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting, Yan-Xiu Zheng ITRI David W. Lin and Ching-Yao Huang NCTU | Jefferywu@itri.org.tw |
| Re: | E-MBS DG document 09_2166 |  |
| Abstract | Comments on the initial text in Section 15.3.5.4.3 for E-MBS DG discussion. |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Zone specific pilot patterns

Jia-Hao Wu, Yu-Tao Hsieh, Pang-An Ting, Yan-Xiu Zheng

ITRI

David W. Lin, Ching-Yao Huang

NCTU

---------------------------------------------- Start of Proposed Text ------------------------------------

<Please add following text and figure in line 18 on page 1 in the section 15.3.5.4.3>

E-MBS zone specific pilots are transmitted for multi-cell multicast broadcast single frequency network (MBSFN) transmissions. An E-MBS zone is a group of ABSs involved in an SFN transmission. The E-MBS zone specific pilots are common inside one E-MBS zone. Synchronous transmission of the same pilots from multiple ABSs helps an AMS to estimate the composite channel response within an E-MBS zone.

The single E-MBS pilot pattern is robust for various cell size MBSFN deployments. The pilot supports channel estimation of up to two transmit streams within an E-MBS zone. The structures of the E-MBS zone specific pilot pattern are shown in Fig. XXX.

Fig. XXX E-MBS zone specific pilot pattern

---------------------------------------------- End of Proposed Text ------------------------------------