| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dedicated Carrier Supporting E-MBS in IEEE 802.16m (E-MBS) |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D1 E-MBS,” in response to the TGm Call for Contributions on Project 802.16m Amendment Content, IEEE 802.16m-09/0037 |  |
| Abstract | E-MBS Support in detail of dedicated carrier on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Dedicate Carrier Supporting E-MBS in IEEE 802.16m (E-MBS)

Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD)[1] describes the support for Enhanced Multicast Broadcast Service(E-MBS) and Section 13 in SDD describes some primitive concept of E-MBS, which is not enough to cover general concept of support E-MBS on the next version of IEEE 802.16m Amendment Draft Standard [2] . Therefore, in this contribution, we describe the dedicated carrier operation supporting E-MBS operation in IEEE 802.16m for the 802.16m Amendment Draft Standard.

# 2. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Add the following text in the 802.16m Amendment Draft Standard]

15.8.x E-MBS Dedicated Carrier

The multi-carrier AMS which is capable of processing multiple radio carriers at the same time may perform normal data communication at one carrier while receiving E-MBS data over another carrier. It may also receive multiple E-MBS streams from multiple carriers simultaneously.

The multi-carrier AMS which is incapable of processing multiple radio carriers at the same time may perform carrier switching mode. The carrier switching for E-MBS service on the dedicated carrier which is partially configured may be operated in consideration of the E-MBS scheduling interval and/or MBS MAP interval.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document,” July 2009

- IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 2009