| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for DL Control Channel Information Elements of E-MBS MAP in IEEE 802.16m (E-MBS) |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D1 E-MBS,” in response to the TGm Call for Contributions on Project 802.16m Amendment Content, IEEE 802.16m-09/0037 |  |
| Abstract | E-MBS Support in detail of Downlink Control Structure on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for DL Control Channel Information Elements of E-MBS MAP in IEEE 802.16m (E-MBS)

Eunkyung Kim, Jaesun Cha, Juhee Kim, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1. Text Proposal for the 802.16m Amendment Draft Standard

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 15.3.6.5.5 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)]

Table xxa – E-MBS MAP Message format

| Syntax | size (bits) | Notes |
| --- | --- | --- |
| E-MBS MAP Message format () { |  |  |
| [E-MBS Region IE ()] |  | See Table xxb. <<comment: It may be located separately with the E-MBS MAP message. In this case it does not included in this message>> |
| Next E-MBS MAP Point IE() |  | See Table xxc |
| # of MSTID (i) | [TBD] | # of E-MBS STID included in the E-MBS MAP |
| for(i=0; i<# of MSTID; i++){ |  |  |
| MSTID | 12 | Multicast STID |
| # of FID (k) | [TBD] | # of FIDs |
| for(k=0;k<#of FID;k++){ |  |  |
| FID | 4 | Multicast FID for MSTID |
| Frame offset for MBS Region | 2 | for the FID Frame location of the first E-MBS Region after transmitting E-MBS MAP message 1: current frame 2: (current frame + 1)th frame … n: (current frame + n)th frame |
| Subframe offset for MBS Region | 3 | for the FID Subframe location in the frame of the first E-MBS Region after transmitting E-MBS MAP message 0b000: 1st subframe of the frame … 0b111: (7+1)th subframe of the frame |
| MCS | [4] | MCS for the E-MBS region of FID Depends on supported modes, 16modes |
| Resource Information | [TBD] | Region Information of E-MBS region for the FID |
| Allocation pattern | [TBD] | Pattern to transmit such as periodicity |
| } |  |  |
| } |  |  |

Table xxb – E-MBS Region IE format

| Syntax | size (bits) | Notes |
| --- | --- | --- |
| E-MBS Region IE () { |  |  |
| Superframe Bitmap | [TBD] | Size is the same as MSI length in superframe unit 1: multicast is included in the superframe 0: unicast only in the superframe |
| Frame Bitmap | 4*( number of bit set in the superframe bitmap) | Maximum length = (number of frame in an superframe) * (length of superframe); Length = (number of frame in an superframe) * (number of set bit in the superframe bitmap) 1: multicast is included in the frame 0: unicast only in the frame |
| Subframe Bitmap | (number of subframe in an frame) *(number of bit set in the frame bitmap) | Maximum length = (number of subframe in an frame) * (Max. length of frame bitmap) Length = (number of subframe in an frame) * (length of frame bitmap) 1: multicast is included in the subframe 0: unicast only in the frame |
| } |  |  |

Table xxc – Next E-MBS MAP Point IE format

| Syntax | size (bits) | Notes |
| --- | --- | --- |
| Next E-MBS MAP Point IE () { |  |  |
| Superframe number | [TBD] | LSB of the superframe number of the next E-MBS MAP message |
| Frame Index | 2 | Frame location of the next E-MBS MAP message 0b00: 1st frame 0b01: 2nd frame 0b10: 3rd frame 0b11: 4th frame |
| Subframe Index | 3 | Subframe location in the frame of the next E-MBS MAP message 0b000: 1st subframe of the frame … 0b111: 8th subframe of the frame |
| MCS | [4] | MCS for the next E-MBS MAP message Depends on supported modes, 16modes |
| Resource Information | [TBD] | Region information of the next E-MBS MAP message |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]