| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for E-MBS MIMO Transmission (E-MBS) |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Chih-Yuan Lin, Pei-Kai Liao, Yu-Hao Chang, Paul Cheng | E-mail: chihyuan.lin@mediatek.com |
| Re: | Call for comments on C80216m-09/2166. |  |
| Abstract | E-MBS MIMO transmission schemes are not determined. This contribution proposes some texts to address this issue. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for E-MBS MIMO Transmission Schemes (E-MBS)

Chih-Yuan Lin, Pei-Kai Liao, Yu-Hao Chang, and Paul Cheng

Mediatek

# 1. Text Proposal for the 802.16m Amendment Draft Standard

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

15.3.7.4  MIMO Transmission Schemes for E-MBS

There are three MIMO transmission modes as listed in Table x1 for E-MBS MIMO transmissions.

Table x1. E-MBS MIMO Modes

| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| --- | --- | --- | --- |
| E-MBS MIMO Mode 0 | OL SU-MIMO | SFBC | Non-adaptive |
| E-MBS MIMO Mode 1 | OL SU-MIMO(SM) | VE | Non-adaptive |
| E-MBS MIMO Mode 2 | OL SU-MIMO(SM) | HE | Non-adaptive |

The allowed values of the parameters for each E-MBS MIMO mode are shown in Table x2.

Table x2. E-MBS MIMO Parameters

|  | Number of transmit antennas | STC rate per layer | Number of streams | Number of subcarriers | Number of Layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | Rate | Mt | NF | L |
| E-MBS MIMO Mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| E-MBS MIMO Mode 1 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
| E-MBS MIMO Mode 2 | 2 | 2 | 2 | 1 | 2 |
|  | 4 | 2 | 2 | 1 | 2 |
|  | 8 | 2 | 2 | 1 | 2 |

In E-MBS MIMO mode 2, the two layers correspond to two codewords. The transmission format shall include a primary codeword and a secondary codeword. The primary and secondary codewords may use different modulation and coding and/or power offsets in order to enable efficient interference cancellation at the mobile station.

Non-adaptive precoding for E-MBS without macro-diversity support

With MEF=TBD, the matrix W changes every N1 PRUs according to Equation (213).

Non-adaptive precoding for E-MBS with macro-diversity support

With MEF=TBD, the matrix W changes every N1 PRUs. The PMI of a  precoder matrix applied in the sth subband is derived by the following equation:

,  ,	(x)

where CellID denotes the cell ID and  is the number of subbands used for the E-MBS transmission.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]