| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for EMBS-MAP and EMBS MAP IE based ToC recommended by EMBS DG |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Etemad Kamran Muthaiah Venkatachalam Intel Corporation | Kamran.etemad@intel.com |
| Re: | Proposed text for section AWD 15.8 |  |
| Abstract | This contribution proposes AWD text for EMBS for session 63.5 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for EMBS-MAP and EMBS MAP IE

based ToC recommended by EMBS DG

Kamran Etemad, Muthaiah Venkatachalam

Intel Corporation

- Introduction

The EMBS Resource allocation in 16m should reuse the design principals of  MBS in 802.16-2009. Based on this design goal this proposal suggest using EMBS-MAP and EMBS MAP DATA IE’s which are similar to MBS MAP and MBS-DATA IE’s in 802.16-2009, with necessary changes to adopt 802.16m specific design.

The changes include:

- The use of Multicast STID’s and FID’s intead of MCID’s and LCID’s.

- Using fixed MSI and periodic allocation.

- Using compact resource indexing with MIMO support

- Allowing selective decoding with power saving on a per FID basis

- Apply message optimization to reduce the overhead when allocation attribute are similar across FID’s.

========================== Start of Proposed Text =========================

<< DG Notes:  Text that the DG group agrees on are indicated in blue with an underline.  Sentences/words that have comments on them are left in black.  >>

<<Add text to these existing sections>>

- 15.3.6.2 DL Control Channels

15.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries some configuration information for enhanced multicast broadcast service for one or more EMBS Zones.  The E-MBS MAP, when present, is transmitted at the starting place of an MSI

The E-MBS MAP message structure is defined in Table XXX1.

- Table XXX1. E-MBS MAP Format

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| No of E-MBS_Zone | [TBD] | Number of E-MBS_ Zones |
| for (i=0; i≤ No. of E-MBS_Zones; i++) { | - | - |
| EMBS-Zone-ID | 12 | The EMBS Zone ID to which this EMBS MAP apply |
| MSI Length (m) | [TBD] | The length of an MSI is 2m+3 superframes |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| } |  |  |

- 15.3.6.3 Resource Mapping of DL Control Channels

15.3.6.3.3 Enhanced- Multicast Broadcast Services MAP (E-MBS MAP)

- 15.3.6.5 DL Control Information Elements

15.3.6.5.5 E-MBS Control Information Elements

The resource mapping for EMBS allows are defined in EMBS-DATA_IE’s as defined in  Table XXX2.

- Table XXX2. E-MBS DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| No. of Multicast STIDs | 4 | Up to 16 Multicast STID in one EMBS-Zone |
| For (i=0; i< No. of Multicast STID’s, i++){ | - |  |
| STID | 12 | Multicast STID |
| PHY Similarity Flag | 1 | Set this to 0 for the first STID and Set this to 1 if the PHY parameters for this STID is the same as previous one. |
| If PHY Similarity Flag =”0” | 1 | If Flag =”1” skip MCS and MIMO Configuration and set them to be the same as those defined for previous STID listed in this EMBS DATA IE. |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| E-MBS MEF | 2 | E-MBS MIMO Encoding Format 0b00 – SFBC 0b01 – VE 0b10 – HE 0b11 - reserved |
| If (E-MBS MEF == 0b01){ | - |  |
| Mt | 1 | Number of streams 0b0 = 1 stream 0b1 = 2 streams |
| No. of FID’s | 4 | No. of FID’s for the Given Multicast STID |
| For (j=0; j< No. of FID’s for Given STID, i++){ | - |  |
| FID | 4 | FID of the channel in the package. |
| MBS Frame offset & Resource Indexing | [TBD] | Include the location and allocation size, |
| Persistence Flag | 1 | “0” One allocation per MSI no persistence “1” persistent allocation |
| If Persistence Flag==1 { Periodicity | 2 | Period of tranmission in units of MSI time “00” Half the MSI Period “01” One MSI Period “10” two MSI perid “11” 4 MSI Period |
| Persistence Window | 2 | Up to 4 Period |
| Padding | Variable | Padding to reach byte boundary |
| }} | - | - |

========================== End of Proposed Text =========================