| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for Group DSA for EMBS based on EMBS DG draft |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Etemad Kamran Muthaiah Venkatachalam Intel Corporation | Kamran.etemad@intel.com |
| Re: | Proposed text for section AWD 15.8 |  |
| Abstract | This contribution proposes AWD text for EMBS for session 63.5 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for Group DSA for EMBS based on EMBS DG draft

Kamran Etemad, Muthaiah Venkatachalam

Intel Corporation

- Introduction

The EMBS Framework in 802.16m needs to address the following deployment/usage scenarios:

- Access of 16m MS’s in a 16m carrier to EMBS transmissions in the same carrier

- Access of 16m MS’s in a 16m mixed carrier to MBS content sent in 16e zone of the same carrier.

If Multicarrier Switching is supported the following shall also be enabled:

- Access of 16m MS’s in a 16m carrier to MBS content sent on another 16e carrier or 16e zone of another carrier 16m mixed carrier.

- Access of 16m MS’s in a 16m carrier to EMBS in DL only 16m carriers

- Access of 16m MS’s in a 16m carrier to contents on an alternative Broadcast Only Technology (interworking)

While all scenarios are required, scenarios 1-2 are most basic ones to address, while scenario 3 is also critical for most deployments. Scenarios 4 and 5 are also needed to meet the requirement ut are considered as more advanced features.

The EMBS framework should be consistent with MBS definitions and procedures in IEEE802.16-2009 and should not require major changes to MCBCS network architecture in WiMAX Network Release 1.5.

In such framework, when MBS is supported the DSx based MBS service management at later 2 is required both in the air interface and in the network. Support for Group DSx is also part of MBS in Release 1.5 profile and supported by the network.

Based on these considerations this contribution proposed text to define Group DSx based service flow creation which also includes redirection of MS to 16e/16m zones of the same or other carriers as well as other technologies. Considering that redirection to other technologies and interaction for multicarrier feature needs further discussions in 16m thois contribution only defines place holders for such redirection and interaction with MC messages will be defined in a separate contribution.

========================== Start of Proposed Text =========================

<< DG Notes:  Text that the DG group agrees on are indicated in blue with an underline.  Sentences/words that have comments on them are left in black.  >>

<<Add text to these existing sections>>

15.2.5.8 AAI-DSx MAC Management Message

15.2.5.8.1 AMS initiated AAI-DSA-REQ:

AMS sends Group DSA-REQ to the ABS requesting EMBS/MBS service. The AMS may not know the type of EMBS (ie legacy or 16m or mixed) that is available when it sends the DSA-REQ. The ABS sends the DSA-RSP to the AMS containing the relevant EMBS/MBS parameters. The parameters sent will exactly indicate to the AMS whether to listen to EMBS on a 16m zone or to listen to MBS on a legacy MBS zone.

  - Service type: 4 bits

    - Value 0 =  EMBS flows based on 802.16m

    - Value 1 = MBS flows based on 802.16-2009

    - Values 2-15 = reserved for other technologies.

15.2.5.8.1 BS Initiated AAI-GDSA-RSP:

The AAI-GDSA-RSP is send by the ABS to activate new EMBS or legacy MBS  flows and it contains the following information:

  - Service type: 4 bits

    - Value 0 =  EMBS flows based on 802.16m

    - Value 1 = MBS flows based on 802.16-2009

    - Values 2-15 = reserved for other technologies.

  - If (service type = 1)

    - IEEE802.16-2006 based MCIDs and LCIDs

    - CS of these MCIDs

    - SF parameters of these MCIDs

    - IEEE802.16-2006  MBS Zone-ID

    - IEEE802.16-2006 MBS carrier info. Optionally included only if the MBS carrier is different from the serving carrier.

  - If (service type = 0)

    - 16m STIDs package with available FIDs

    - CS parameters for each of the STID

    - SF parameters for each of the <STID, FID> combination

    - EMBS Zone-ID

    - EMBS RF carrier info. Optionally included only if the EMBS carrier is different from the serving carrier

15.3.12.6  Idle Mode Support for MBS

- E-MBS Configuration Indicators

<<Note:  This may not be a new section>>

15.3.5.4.3  E-MBS Zone Specific pilot Patterns

- 15.3.5.6  Downlink Physical Structure for E-MBS Support

15.3.6 Downlink control structure

- 15.3.6.2 DL Control Channels

15.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

- 15.3.6.3 Resource Mapping of DL Control Channels

15.3.6.3.3 Enhanced- Multicast Broadcast Services MAP (E-MBS MAP)

- 15.3.6.5 DL Control Information Elements

15.3.6.5.5 E-MBS Control Information Elements

15.8 Support for Enhanced Multicast Broadcast Service

Enhanced Multicast and Broadcast Service (E-MBS) provides an efficient method for concurrent transport of DL data common to a group of users, using a common multicast STID (MSTID) and a FID. E-MBS service is offered in the downlink only and may be coordinated and synchronized among a group of ABS to allow macro-diversity.

Each multicast/broadcast connection is associated with a service flow provisioned with the QoS and traffic parameters for that service flow.

Service flows to carry E-MBS data are instantiated on individual AMSs participating in the service while in normal operation. During such instantiation the AMS learns the parameters that identify the service and associated service flows.

Each ABS capable of providing E-MBS belongs to a certain E-MBS Zone and one ABS can belong to multiple E-MBS zones. An E-MBS zone defined as a set of ABSs where the same MSTID and FID is used for transmitting the content of certain service flow(s). Each E-MBS Zone is identified by a unique E-MBS_Zone_ID.

To ensure proper multicast operation on networks of ABS employing E-MBS, the MSTIDs and FIDs used for common E-MBS content and service shall be the same for all ABSs within the same E-MBS Zone. This allows the AMS which has already registered with a service to be seamlessly synchronized with E-MBS transmissions within an E-MBS Zone without communicating in the UL or re-registering with other ABS within that E-MBS Zone. The E-MBS_Zone_IDs shall not be reused across any two adjacent E-MBS Zones.

- 15.8.1 E-MBS Transmission Modes

Continuous reception of EMBS transmissions within an EMBS Zone rely on some coordination among ABS’s in that EMBS Zone. Such coordination is applied to achieve frame level synchronization of EMBS in non-macro diversity transmission mode or to achieve symbol level synchronization with macro diversity transmission mode as described below.

An ABS may provide the AMS with E-MBS content locally within its coverage and independently of other ABSs. The single ABS provision of E-MBS is therefore a configuration where an E-MBS Zone is configured to consist of a single ABS only. This configuration may be provided as one of the possible cases of E-MBS. In this case, the ABS may use any  MSTID and FID for providing the E-MBS service, independently of other ABSs, so the AMS receives the E-MBS data from its serving ABS, and the AMS should not expect the service flow for this E-MBS connection to continue when the AMS leave the serving ABS. However, if the AMS moves to an ABS that is transmitting the same E-MBS flows in another E-MBS Zone and updates its Service Flow management encodings, the AMS may continue to receive the same E-MBS flows.

- 15.8.1.1 Non-Macro Diversity Mode

Non-macro diversity mode is provided by frame level coordination only, in which the transmission of data across ABSs in an E-MBS Zone is not synchronized at the symbol level. However, such transmissions are coordinated to be in the same frame. This MBS transmission mode is supported when macro-diversity is not feasible. For all ABSs that belong to the same E-MBS Zone, the following coordination shall be assured:

— The set of MAC SDUs carrying  E-MBS content shall be identical in the same frame in all ABS in the same E-MBS Zone;

— The mapping of MAC SDUs carrying E-MBS content onto MAC PDUs shall be identical in the same frame in all ABS in the same E-MBS Zone, meaning, in particular, identical SDU fragments and identical fragment sequence number (block sequence number) and fragment size

Coordination in the E-MBS Zone assures that the AMS may continue to receive E-MBS transmissions from any ABS that is part of the E-MBS Zone, regardless of the AMS operating mode—Active Mode, Sleep Mode and Idle Mode—without need for the AMS to register to the ABS from which it receives the transmission.

- 15.8.1.2 Macro Diversity Mode

In addition to coordination, E-MBS transmissions may optionally be synchronized across all ABSs within an E-MBS Zone. This option enables an AMS to extract macro-diversity gains in the multicast or broadcast transmission from multiple ABS, thereby improve the reliability of reception. When macro-diversity is used, the mapping of SDUs into the E-MBS Bursts is identical, and the same E-MBS bursts are transmitted using the same time-frequency resource in all involved ABS; additional parameters may also be required to be identical across ABSs if macro-diversity is used.

In macro-diversity mode, within one E-MBS Zone all ABSs participating in the same E-MBS service shall be time and frequency synchronized in the transmissions of common E-MBS data to allow macro diversity gain at the AMS. When macro-diversity is enabled the E-MBS bursts positions and dimensions as well as PHY parameters shall be the same across all ABS’s within the same E-MBS Zone. In addition to the coordination parameters such as E-MBS Zone ID, STID & FID, MSI, and Packet Classification Rule parameter(s), macro-diversity synchronization requires that all ABSs within the same E-MBS Zone shall use the same

—Transmission PHY parameters, MCS associated with each E-MBS Burst including FEC Type, Modulation Type, and Repetition Coding

— Mapping of SDUs to PDU (order of the SDUs and fragments) including Sub Headers )

— Mapping of PDUs to bursts

— Order of bursts in the zone/region

— E-MBS MAP construction

Mechanisms and procedures for multiple ABSs to accomplish the synchronized transmission (which implies performing functions like classification, fragmentation, scheduling at a centralized point like the E-MBS Server) are outside the scope of this standard.

15.8.2 E-MBS Operation

Establishment of E-MBSs for a specific AMS with respect to certain service flow, when needed, shall be performed while the AMS is in connected state.

E-MBS service flows are not dedicated to the specific AMS and are maintained even though the AMS is either in Active/Sleep mode or in the Idle mode. When an AMS is registered at an ABS for receiving E-MBS, multicast and broadcast service flows shall be instantiated as multicast connections. Data of multicast and broadcast service flows may be transmitted from ABS and may be received at AMS also regardless of what mode the AMS is currently in. The ABS may establish a DL E-MBS by creating a multicast and broadcast service flows when the service commences. Mapping of multicast and broadcast service flows to corresponding STIDs and FIDs shall be known and be the same for all ABSs belonging to the same E-MBS Zone. The method of making all ABS in the same E-MBS Zone aware of E-MBS flows and associated E-MBS service flows—including multicast STID and FID assignment, QoS parameter set, and Classification Rule(s)—is outside the scope of the standard. As the classification and transmission of E-MBS flows may be supported on an ABS in an E-MBS Zone regardless of the presence or absence of any AMS in Active mode receiving the service, the ABS may retain E-MBS service flow management encodings to do classification and scheduling of E-MBS flows, even when no AMS in Active mode receiving the service is registered at the ABS.

The AMS discovers the E-MBS services and registers with the ABS for receiving multicast and broadcast services through upper layer signaling which are outside the scope of this standard.  The EMBS user may choose and subscribe to the desired program package based on the program guide provided to the AMS through upper layer signaling from the EMBS server in the network. The user also can make changes to its selection of EMBS programs through upper layer signaling with such EMS server.  Such upper layer signaling with EMBS server in the network is outside the scope of this specification.

Following such registration at the ABS for receiving multicast and broadcast services, the ABS or AMS may initiate the DSA procedurewith respect to multicast and broadcast connections.

When supported the EMBS service flows are activated, changed or deleted through ABS/network Initiated DSA, DSC and DSD messaging respectively. The EMBS service flows may be activated, changed or deleted through DSA, DSC and DSD messaging respectively. The ABS initiated DSx messaging shall be supported and MS initiated DSA MAY also be supported by the AMS.  Considering that EMBS services typically involve multiple flows the compact form of Group DSx shall be used to reduce overhead and signaling delay associated with EMBS flow management.

The ABS sends the AAI-DSA-RSP to the AMS containing the relevant EMBS/MBS parameters. The parameters sent will indicate to the AMS whether to listen to EMBS on a 16m zone or to listen to MBS on a legacy MBS zone. It also includes E-MBS multicast STIDs and FIDs, E-MBS zone IDs, E-MBS carrier information (physical carrier index).  Subscription EMBS can be at the granularity of  EMBS-Packages/STID’s, while selective decoding of content is at the granularity of content channels or FID’s.

If multicarrier feature is supported by the AMS the ABS may also use DSA-RSP message to direct the MS to legacy MBS zones or 16m EMBS zone of other carriers. The AMS, which has acquired the service flow information for the specific E-MBS service from AAI_DSA message, shall proceed to decode AAI_EMBS_Conf. message sent on the target EMBS carrier for E-MBS configuration information.

After successful configuration, the AMS shall reuse the same configuration when it moves to another ABS in the same E-MBS Zone without re-configuration.

The AMS learns the E-MBS_Zone_ID to which the ABS belongs through  E-MBS Zone configuration message. The AMS may continue to receive E-MBS transmissions from any ABS that is part of the E-MBS Zone, regardless of the AMS operating mode—Active Mode, Sleep Mode, Idle Mode—without need for update to any service flow management encoding for the E-MBS flow.

To allow seamless transition from one E-MBS Zone to another without any interruption of E-MBS data service and operation, the AMS should update E-MBS service flow management encodings including MSTIDs and FIDs, Packet Classification Rule parameter(s), E-MBS Zone Identifier Assignment parameter, and E-MBS contents IDs. If the AMS has no MSTIDs and FIDs information regarding the new E-MBS Zone, then the AMS is required to acquire MSTIDs and FIDs context through the other procedures, i.e., location-update if AMS is in the idle mode and handover if MS is in connected mode.

- 15.8.2.1 E-MBS Operation in Connected State

<<DG Chair Notes:  TLV is currently called parameter in D1.  Unlike in 16e where REG-RSP is a TLV in RNG-RSP, there are two messages in D1 – AAI_REG-RSP and AAI_RNG-RSP.   We need to decide on one message and update the sentence accordingly. >>

When the AMS transits to a new E-MBS Zone while in Active Mode or Sleep Mode, and the E-MBS service flow management encodings of the AMS have not been updated, the ABS may include MSTID and FID Update in REG-RSP encoding TLV in the RNG-RSP to provide updated service flow management encodings for any affected E-MBS flow as part of the handover procedure.

- 15.8.2.2 E-MBS Operation in Idle State

When an AMS in Idle mode moves to an ABS which does not belongs to AMS’ previous E-MBS Zone, the AMS is expected to update the E-MBS service flow management encodings at that ABS to provide for further reception of E-MBS content. Such an update may include one or more of MSTIDs and FIDs, Packet Classification Rule parameters, and E-MBS Zone Identifier Assignment parameter. If the AMS has not received such information from the serving E-MBS Zone, the AMS may conduct location update to acquire updated E-MBS service flow management encodings, or may conduct re-entry from Idle mode. The ABS may include STID_Update in REG-RSP encoding TLV in the RNG-RSP to provide updated service flow management encodings for any affected E-MBS flow.

- 15.8.3 E-MBS Protocol Features and Functions

- 15.8.3.1 E-MBS Zone Configuration

<< DG Chairs comment: We need to identify where this configuration information is carried.  We have identified two options.  Members are requested to indicate their preferred choice or identify more options>>

Different MSTIDs and FIDs may be used in different E-MBS Zones for the same multicast and broadcast service flow. A multicast and broadcast zone identifier (E-MBS_ZONE_ID) is used to indicate a service area through which a MSTID and FID for a broadcast and multicast service flow are valid. An ABS that supports E-MBS shall include the E-MBS Zone identifier(s) as an E-MBS Zone identifier list in the [TBD] message. The E-MBS Zone identifier shall not be '0'.

When the E-MBS Zone identifier list appears in [TBD] message with only one value of '0', then the neighbor ABS is not affiliated with any E-MBS zone.

The ABS sends AAI_DSA for establishment of connection for E-MBS, E-MBS ZONE shall be encoded in the AAI-DSA message. If an AMS in idle mode moves into ABSs in the same E-MBS zone, the AMS does not have to re-enter the network to re-establish a connection. However, if an AMS moves into a different E-MBS zone, the AMS may need to update service flow management encodings for the multicast and broadcast service flow. One ABS may have multiple E-MBS Zone IDs for different E-MBS services.

- To support inter-MBS zone transition, AMSs need to get E-MBS_Zone_IDs with which ABS is associated.

- 15.8.3.2 E-MBS Scheduling Interval (MSI)

E-MBS scheduling interval can span several super-frames. For each MBS Zone there is an MBS Scheduling Interval (MSI), which refers to a number of successive frames for which the access network may schedule traffic for the streams associated with the MBS Zone prior to the start of the interval. The length of this interval depends on the particular use case of E-MBS. E-MBS MAP addresses the mapping of MBS data associated with an MBS Zone for an entire MSI.  The MBS MAP message is structured such that it may be used to efficiently define multiple transmission instances for a given stream within an MSI.

An AMS decodes only the E-MBS data bursts associated with user selected content.  The AMS wakes up in each E-MBS scheduling interval in order to check whether there is data to be decoded.

========================== End of Proposed Text =========================