| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Region Configuration Indicator and E-MBS MAP on E-MBS DG draft |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak LG Electronics | chegal@lge.com, sixs@lge.com, emptylie@lge.com, ksryu@lge.com, ronnykim@lge.com, samji@lge.com |
| Re: | P802.16m/D1 - Enhanced – Multicast Broadcast Service (E-MBS) Call for Contributions on Project 802.16m Amendment Content : IEEE 802.16m-09/0037 |  |
| Abstract | Proposes initial text for E-MBS DG discussion. |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Region Configuration Indicator and E-MBS MAP on E-MBS DG draft

Jeongki Kim, Youngsoo Yuk, Jinsoo Choi,  Kiseon Ryu, Yongho Kim and Jin Sam Kwak

- LG Electronics

<< DG Notes:  Text that the DG group agrees on are indicated in blue with an underline.  Sentences/words that have comments on them are left in black.  >>

<<Add text to these existing sections>>

15.2.5.8 DSx MAC Management Message

15.3.12.6  Idle Mode Support for MBS

- 15.3.12.6.1 MBS Location Update

E-MBS Region Configuration Indicators

E-MBS subframe configuration shall be included in the [S-SFH SP1 IE] to inform the number of Dl subframes allocated only for E-MBS region. E-MBS Region Configuration Indicator carries the information to configure the E-MBS Region and is included in the [S-SFH SP2 IE] [ABI/SCD]. Table x shows the example of E-MBS Region configuration indicator. If the E-MBS region is multiplexed with unicast region by TDM manner, all frequency partitions in a subframe is used for E-MBS region.

[Insert the following field [into table 675, page xxx][in the AAI_SCD message ]]

[Table 675—S-SFH SP2 IE format ]

| - Syntax | - Size (bit) | - Note |
| --- | --- | --- |
| - (E-MBS) subframe configuration | - [2][6] | - The number of the DL subframes of which whole resources are used for E-MBS region. - [We can add a new field or can be included as cases of existing subframe configuration.] |

[Table 675—S-SFH SP2 IE format ] [Table AAI SCD message]

| - Syntax | - Size (bit) | - Note |
| --- | --- | --- |
| - E-MBS Region Configuration Indicator | - [3] | - The mapping between value of this index and E-MBS Region configuration is listed in Table X. |

- Table X— E-MBS Region Configuration and Indexing

| - E-MBS Region Configuration Indicator | - Notes(The details is TBD) |
| --- | --- |
| - 000 | - No E-MBS region |
| - 001 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 010 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 011 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 100 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 101 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 110 | - TBD last DL subframes and TBD first LRUs in FP0 |
| - 111 | - Dynamic Allocation |

15.3.5.4.3  E-MBS Zone Specific pilot Patterns

15.3.5.6  Downlink Physical Structure for E-MBS Support

15.3.6 Downlink control structure

15.3.6.2 DL Control Channels

15.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

An E-MBS MAP carries control information to decode the E-MBS Bursts belonging to an E-MBS Zone. The E-MBS MAP is transmitted at the first E-MBS region in a E-MBS Scheduling Interval (MSI).

The size of E-MBS MAP is of variable by the number of E-MBS_DATA_IEs. If the method of E-MBS resource indication is static, the size of E-MBS is indicated by the E-MBS Control Header. An E-MBS Control Header is the same structure as non-user specific A-MAP defined in 15.3.6.4.1 and is transmitted at the beginning of the first E-MBS region in a MSI. If the E-MBS Control Header exists, an E-MBS MAP follows the E-MBS Control Header. Otherwise, E-MBS MAP is transmitted at the beginning of the first E-MBS region in a MSI.

If the dynamic allocation is used, the E-MBS MAP Assignment A-MAP IE is used to indicate the E-MBS region in the subframe which contains E-MBS MAP. An E-MBS MAP Assignment A-MAP IE is a type of the unicast control Assignment A-MAP IEs and contains the information to decode an E-MBS MAP (e.g., Resource allocation size, MCS, etc.). E-MBS MAP Assignment A-MAP IE is transmitted at the first Assignment A-MAP region of the first frame of a MSI.

15.3.6.3 Resource Mapping of DL Control Channels

15.3.6.3.3 Enhanced- Multicast Broadcast Services MAP (E-MBS MAP)

E-MBS Control Header

The E-MBS Control Header is encoded with a code rate of [1/12] using TBCC described in 15.3.12.2.

The encoded sequence of E-MBS Control Header is modulated using QPSK.

E-MBS MAP

A 16-bit CRC is generated based on the contents of the E-MBS MAP IE. Following randomization, the resulting sequence of bits shall be encoded by the CTC described in <<15.3.12. 2. >>

The code rates of E-MBS MAP shall be explicitly signaled in the E-MBS Control Header or E-MBS MAP Assignment A-MAP IE.

[The encoded sequence of E-MBS MAP is modulated using QPSK.]

15.3.6.5 DL Control Information Elements

- 15.3.6.5.4.13 E-MBS MAP Assignment A-MAP IE

Table xxx1 shows E-MBS MAP Assignment A-MAP IE format which contains the information to decode the E-MBS MAP IE for the corresponding E-MBS Zone. This is used only for dynamic allocation of E-MBS Region.

- Table xxx1. E-MBS MAP Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS MAP Assignment A-MAP IE () { | - | - |
| E-MBS IE Type | 4 | E-MBS MAP assignment A-MAP IE |
| E-MBS Region Size | [TBD] | Indicates the size of E-MBS region (the number of LRUs) |
| I_sizeoffset | [5] |  |
| [E-MBS MIMO Information] | TBD | TBD E.g.) E-MBS MIMO Encoding Format (SFBC/VE/HE/etc.), Number of stream |
| Resource Indexing | [TBD] | Include the location and allocation size of E-MBS MAP message |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

A 16 bit CRC is generated based on the contents of the E-MBS MAP Assignment A-MAP IE. The CRC is masked by the E-MBS Zone Station ID.

15.3.6.5.5 E-MBS Control Information Elements

15.3.6.5.5.1 E-MBS Control Header

- An E-MBS Control Header contains the information to decode E-MBS MAP as shown in Table xx2.

- Table xxx2. E-MBS Control Header format

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS Control Header format () { | - | - |
| Allocation size of E-MBS MAP | [7] | Number of LRUs which are used for E-MBS MAP transmission. |
| ISizeOffset | [5] | ISizeOffset |
| } |  |  |

15.3.6.5.5.2 E-MBS MAP IE

- An E-MBS MAP IE shall include one or multiple MBS_DATA IEs.

- Table xxx3 shows E-MBS MAP IE format.

- Table xxx3. E-MBS MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS MAP IE () { | - | - |
| Number of E-MBS Zone | [2] |  |
| For (i=0; i< Number of E-MBS Zone; i++) { |  |  |
| E-MBS Zone ID | [7] |  |
| MSI Length (m) | [2] | The length of an MSI is 2m+3 superframes |
| Number of E-MBS_DATA IEs | [5] |  |
| For (j=0; j<Number of E-MBS_DATA IEs; j++) { |  |  |
| E-MBS_DATA_IE () |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| CRC | [16] |  |
| } |  |  |

- 15.3.6.5.5.2.1 E-MBS_DATA IE

- An E-MBS MAP include one or more Basic Assignment MBS_DATA IEs, Persistent Fixed Assignment MBS_DATA IEs, and Persistent Flexible Assignment MBS_DATA IEs. An Basic MBS_DATA IE contains the resource assignment information for a MBS burst which is present once in a MSI, an Persistent Fixed Assignment MBS_DATA IE contains the resource assignment information for MBS bursts belonging to a E-MBS connection with Periodic traffic pattern and fixed payload size and an Persistent Flexible Assignment MBS_DATA IE contains the resource assignment information for MBS bursts belonging to a E-MBS connection with Periodic traffic pattern and variable payload size. Table xxx4 shows E-MBS_DATA IE format.

- Table xxx4. E-MBS_DATA IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS_DATA IE () { | - | - |
| MBS_DATA IE Type | 2 | 0b00: Basic Assignment E-MBS DATA IE 0b01: Persistent Fixed Assignment E-MBS DATA IE 0b10: Persistent Flexible Assignment E-MBS DATA IE 0b11: Reserved |
| Multicast STID | 12 |  |
| FID | 4 |  |
| MIMO Indicator | 1 | If MIMO Indicator is 1, it indicates that MIMO information (E-MBS MEF/Mt) is explicitly assigned for this allocation. Otherwise, this allocation will use the same MIMO Information (E-MBS MEF/Mt) as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (MIMO Indicator == 1) { |  |  |
| [E-MBS MIMO Information] | TBD | TBD E.g.) E-MBS MIMO Encoding Format (SFBC/VE/HE/etc.), Number of stream (1 or 2) |
| } |  |  |
| Frame offset Indicator | 1 | If Frame offset Indicator is 1, it indicates that E-MBS Frame offset is explicitly assigned for this allocation. Otherwise, this allocation will use the same E-MBS Frame offset as the previous subburst. If j is 0 then this indicator shall be 1 |
| if (Frame offset Indicator == 1) { |  |  |
| E-MBS Frame offset | [TBD] | Include the start [Super-frame/Frame/Subframe] offset of this allocation |
| } |  |  |
| If (MBS_DATA IE Type ==0b00) { |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that ISizeOffset is explicitly assigned for this allocation. Otherwise, this allocation will use the same ISizeOffset as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index, see 15.3.12.1.2 |
| } |  |  |
| Resource Indexing | [TBD] | Include the location and allocation size, |
| Long TTI |  |  |
| } else { |  |  |
| If (MBS_DATA_IE Type == 0b01) { |  |  |
| Allocation Period | [6] | Frame offset for multiple transmission instances (in number of frames) |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that I_SizeOffset is explicitly assigned for this allocation. Otherwise, this allocation will use the same ISizeOffset as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index, see 15.3.12.1.2 |
| } |  |  |
| Resource Indexing | [TBD] | Include the location and allocation size, |
| Long TTI |  |  |
| } else if (MBS_DATA_IE Type == 0b10){ |  |  |
| Allocation Period | [TBD] | Frame offset for multiple transmission instances |
| Number of bursts | [4] | The number of allocated bursts during MSI |
| For (i=0; I < Number of bursts; i++) { |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that ISizeOffset is explicitly assigned for this allocation. Otherwise, this allocation will use the same ISizeOffset as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index, see 15.3.12.1.2 |
| } |  |  |
| Resource Indexing | [TBD] | Include the location and allocation size, |
| Long TTI |  |  |
| } //for |  |  |
| } //else if |  |  |
| } //else |  |  |
| } | - | - |