| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for DSx MAC Management Message on E-MBS DG draft |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Yongho Kim and Jin Sam Kwak LG Electronics | chegal@lge.com, sixs@lge.com, emptylie@lge.com, ksryu@lge.com, ronnykim@lge.com, samji@lge.com |
| Re: | P802.16m/D1 - Enhanced – Multicast Broadcast Service (E-MBS) Call for Contributions on Project 802.16m Amendment Content : IEEE 802.16m-09/0037 |  |
| Abstract | Proposes initial text for E-MBS DG discussion. |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for DSx MAC Management Message on E-MBS DG draft

Jeongki Kim, Youngsoo Yuk,  Jinsoo Choi,  Kiseon Ryu, Yongho Kim and Jin Sam Kwak

- LG Electronics

15.2.5.8 DSx MAC Management Message

[Insert the E-MBS related parameters into Table 704 as follows.]

- Table 704—Service flow/convergence sublayer parameters

| Fields | Size (bits ) | Description |
| --- | --- | --- |
| Multicast STIDs | Variable | Multicast STIDs allocated for E-MBS |
| FIDs | Variable | FIDs allocated for E-MBS |
| E-MBS Zone Identifier | [7] |  |
| E-MBS Zone STID | 12 | It is used for AMS to decode the E-MBS MAP Assignment A-MAP IE. |
| MSI Length (m) | TBD | E-MBS Scheduling Interval The length of an MSI is [2m+3 ]Superframes |
| MSI start offset | 6 | The start position of MSI LSB 6bits of super frame number |