| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS MIMO |  |
| Date Submitted | 2009-09-20 |  |
| Source(s) | Kaushik Josiam, Qi Wu, Jaejeong Shim Samsung Electronics | Email : qi.wu@samsung.com kjosiam@sta.samsung.com brian.shim@samsung.com |
| Re: | E-MBS DG – call for reply comments |  |
| Abstract | This contribution defines the text and format for E-MBS MIMO |  |
| Purpose | For review and discussion by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for E-MBS MIMO

Kaushik Josiam, Qi Wu, Jaejeong Shim

Samsung Electronics

The following text is proposed to add in 15.3.7.4 of P802.16m/D1.

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

15.3.7.4 MIMO Transmission Schemes for E-MBS

There are three MIMO transmission modes as listed in Table xx1 for E-MBS MIMO transmissions.

Table xx1. E-MBS MIMO Modes

| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| --- | --- | --- | --- |
| E-MBS MIMO Mode 0 | OL SU-MIMO | SFBC | Non-adaptive |
| E-MBS MIMO Mode 1 | OL SU-MIMO(SM) | VE | Non-adaptive |
| E-MBS MIMO Mode 2 | OL SU-MIMO(SM) | HE | Non-adaptive |

The allowed values of the parameters for each E-MBS MIMO mode are shown in Table xx2.

Table xx2. E-MBS MIMO Parameters

|  | Number of transmit antennas | STC rate per layer | Number of streams | Number of subcarriers | Number of Layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | Rate | Mt | NF | L |
| E-MBS MIMO Mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| E-MBS MIMO Mode 1 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
| E-MBS MIMO Mode 2 | 2 | 2 | 2 | 1 | 2 |
|  | 4 | 2 | 2 | 1 | 2 |
|  | 8 | 2 | 2 | 1 | 2 |

In E-MBS MIMO mode 2, the two layers correspond to two codewords. The transmission format shall include a primary codeword and a secondary codeword. The primary and secondary codewords may use different modulation and coding and/or power offsets in order to enable efficient interference cancellation at the mobile station.

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------