| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes/Refinements to the References and Definitions in the IEEE 802.16m SDD (802.16m-09/0034r1) |  |
| Date Submitted | 2009-09-21 |  |
| Source(s) | Roshni Srinivasan, Jose Puthenkulam Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Change Request - 802.16m SDD, IEEE 802.16m-09/0034r1 |  |
| Abstract | This contribution proposes changes to the references and definitions in the 802.16m SDD, IEEE 802.16m-09/0034 r1. Changes are also proposed to text in sections 4, 12 and Appendix 2 for consistency with the revised definitions and references. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction

The contribution proposes text changes to the references and definitions in the 802.16m SDD, IEEE 802.16m-09/0034 r1.

It is proposed that the resolutions to comment #1629 and #1630 in IEEE 802.16m-09/0047r4 be implemented in the 802.16m SDD for consistency.

Changes are also proposed to text in sections 4, 12 and Appendix 2 for consistency with the revised definitions and references.

_____________________  Begin Proposed Text Changes to Section 2________________________________

# 2	References

- IEEE 802.16m PAR, December 2006, http://standards.ieee.org/board/nes/projects/802-16m.pdf , December 2006.

- IEEE 802.16 WG, “Five Criteria Statement for P802.16m PAR Proposal,” IEEE 802.16-06/055r3, November 2006 http://ieee802.org/16/docs/06/80216-06_055r3.pdf, November 2006.

- IEEE Std 802.16-2009: “IEEE Standard for Local and Metropolitan Area Networks – Part 16: Air Interface for Fixed Broadband Wireless Access Systems”, May 2009.

- Recommendation ITU-R M.1645: Framework and overall objectives of the future development of IMT-2000 and systems beyond IMT-2000, January 2003.

- Report ITU-R M.2135: Guidelines for evaluation of radio interface technologies for IMT-Advanced, (November 2008).

- New Report ITU-R M.2134: Requirements related to technical performance for IMT Advanced radio interface(s) (2008).

- IEEE 802.16m System Requirements, IEEE 802.16m-07/002r8 http://ieee802.org/16/tgm/#07_002, January 2009.

- WiMAX Forum™ Mobile System Profile, Release 1.0 Approved Specification (Revision 1.4.0: 2007-05-02), http://www.wimaxforum.org/technology/documents

- The WiMAX Forum Network Architecture Stage 2 - 3: Release 1.5, Version 1.20 5http://www.wimaxforum.org/resources/documents/technical/release

- ITU-T Recommendation X.680: Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation, July 2002.

- ITU-T Recommendation X.681: Information technology - Abstract Syntax Notation One (ASN.1): Information object specification, July 2002.

- ITU-T Recommendation X.682: Information technology - Abstract Syntax Notation One (ASN.1): Constraint specification, July 2002.

- ITU-T Recommendation X.683: Information technology - Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications, July 2002.

- ITU-T Recommendation X.691: Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER), July 2002

- WiMAX Forum: WiMAX Network Protocols and Architecture for Location Based Services DRAFT- T33- 110- R015- v01_ Protocols and Procedures for location based services and DRAFT-T33-110-R015v01_location-based-service (http://www.wimaxforum.org/resources/documents)

- Guidelines for use of a 48-bit Extended Unique Identifier (EUI-48™), http://standards.ieee.org/regauth/oui/tutorials/EUI48.html

- Wireless Priority Service for National Security/Emergency Preparedness: Algorithms for Public Use Reservation and Network Performance", August 2002, http://wireless.fcc.gov/releases/da051650PublicUse.pdf, August 2002.

- Internet Draft “Transmission of IP over Ethernet over IEEE 802.16 Networks” http://tools.ietf.org/html/draft-ietf-16ng-ip-over-ethernet-over-802-dot-16-08, January 2009.

- Internet Draft “Transmission of IPv4 packets over IEEE 802.16's IP Convergence Sublayer”

http://tools.ietf.org/html/draft-ietf-16ng-ipv4-over-802-dot-16-ipcs-04, October 2008.

[20] IEEE Std 802.16j-2009: “IEEE Standard for Local and Metropolitan Area Networks – Part 16: Air Interface for Fixed Broadband Wireless Access Systems, Amendment 1: Multihop Relay Specification”, May 2009.

_____________________  End Proposed Text Changes to Section 2_______________________________

_____________________  Begin Proposed Text Changes to Section 3________________________________

# 3 Definitions, Symbols, Abbreviations

# 3.1 Definitions

- WirelessMAN-OFDMA Reference System: A system compliant with a subset of the WirelessMAN-OFDMA capabilities specified by IEEE 802.16-2004 and amended by IEEE 802.16e-2005 and IEEE 802.16Cor2/D3, where the subset is defined by WiMAX Forum Mobile System Profile, Release 1.0 (Revision 1.4.0: 2007-05-02) [8], excluding specific frequency ranges specified in the Section 4.1.1.2 (Band Class Index) the WirelessMAN-OFDMA capabilities specified in Subclause 12.5 in IEEE Std 802.16-2009[1].

- Advanced WirelessMAN-OFDMA System: A system compliant with the the features and functions defined in  according to this document.

- YMS (Yardstick Mobile Station) WirelessMAN-OFDMA MS: A mobile station compliant with the WirelessMAN-OFDMA Reference System.

- RS (Relay Station): A relay station compliant with the IEEE Std 802.16-2009 [3] as amended by IEEE Std 802.16j-2009 [20]. WirelessMAN OFDMA specification specified by IEEE 802.16-2004 and amended by IEEE 802.16e-2005, IEEE 802.16Cor2/D3 and IEEE 802.16j

- YBS (Yardstick Base Station): WirelessMAN-OFDMA BS: A base station compliant with the WirelessMAN-OFDMA Reference System.

- MRBS (Multihop Relay Base Station): A WirelessMAN-OFDMA BS YBS implementing functionality to support RSs as defined in IEEE Std 802.16j -2009 [20].

- AMS: (Advanced Mobile Station) A mobile station capable of acting as a WirelessMAN-OFDMA MS YMS  and additionally implementing the protocol defined in IEEE 802.16m.

- ARS: A station implementing the relay station functionality defined in IEEE 802.16m.

- ABS: A base station capable of acting as a WirelessMAN-OFDMA BS YBS and additionally implementing the protocol defined in IEEE 802.16m.

- LZone: A positive integer number of consecutive subframes where during which ABS communicates with RSs or YMSs, WirelessMAN-OFDMA MSs and where an ARS or an RS communicates with one or more YMSs. WirelessMAN-OFDMA MSs.

- MZone: A positive integer number of consecutive subframes where during which an ABS communicates with one or more ARSs or AMSs, and where an ARS communicates with one or more ARSs or AMSs.

- Location-Based Service (LBS): A service provided to a subscriber based on the current geographic location of the AMS.

- LBS Application: The virtual entity that controls and runs the location based service, including location determination, and information presentation to the users.

- Location Server (LS): A server which that determines and distributes the location of the AMS in the WiMAX network. It may reside in the WiMAX network CSN, as defined by [15].

- Location Controller (LC): A controller which is responsible for coordinating the location measurements of the AMS. It may reside in the WiMAX network ASN, as defined by [15].

- Location Agent (LA): An agent which is responsible for the making measurements or optionally collecting and reporting of location related data to LC. LA function could reside entirely in the ABS, in the AMS or both, as defined by [15].

- LBS Zone: A configurable amount of consecutive resource units which are reserved for LBS purposes.

- LBS Pilots: A set of pilots which are periodically broadcasted by involved ABSs for LBS purposes.

- Time difference of arrival (TDOA): The measurement of the difference in arrival time of received signals.

- Time of arrival (TOA) : The time of arrival of a signal received by an AMS or ABS

- Angle of arrival (AOA): The angle of arrival of a received signal relative to the boresight of the antenna.

- Spatial Channel Information: Generalized set of measurements from the antennas (spatial channel estimation or a set of AOA's), which can be used for location estimation

- Round trip delay (RTD): The time required for a signal or packet to transfer from a AMS to a ABS and back again.

- Relative delay (RD): The delay of neighbor DL signals relative to the serving/attached ABS.

- Separate coding: Each unicast service control information element is coded separately

- Joint coding: Multiple unicast service control information elements are coded jointly

- E-MBS Zone: An E-MBS zone is a group of ABSs transmitting the same E-MBS content.

- E-MBS Region: An E-MBS region is a time/frequency region within a frame where E-MBS data is transmitted.

- Multicast Service: A Multicast Service is a service where users may dynamically join and leave a Multicast session. The network may monitor the number of users at each E-MBS Zone to decide on data transmission and its mode.

- Dynamic Multicast Service: In the Dynamic Multicast Service, the membership of the multicast group changes in time. Users may join and leave groups at any time. The transmission of the content may be turned on or off based on the number of users in the group.

- Static Multicast Service: In the Static Multicast Service, the content is always transmitted through one or more broadcast channel(s) irrespective of the number of users in the group. The broadcast channel(s) normally pre-established prior to the user(s) join and leave a Multicast session at each Multicast service area.

- Broadcast Service: The Broadcast Service is a special type of E-MBS service for which the content is always transmitted through broadcast channels by the access network without considering the number of users receiving the transmission.

- Subordinate link: a link between the ABS or ARS and its subordinate stations (ARSs or AMS)

- Superordinate link: a link between the ARS or AMS and its superordinate station (ABS or ARS)

- Time-division transmit and receive (TTR) relaying: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) is separated in time.

- Transparent ARS: a relay station that does not transmit A-PREAMBLE, SFH, A-MAP.

- Non-transparent ARS: a relay station that transmits A-PREAMBLE, SFH, A-MAP.

- Access station: A station (ARS or ABS) that provides a point of access into the network for an AMS or ARS.

- Access ARS: A relay station which serves as an access station.

- Centralized security mode: This mode is based on authentication and key management between AMS and ABS, without involving the access ARS.

- Distributed security mode: This mode is based on authentication and key management between AMS and an access ARS, and between the access ARS and the ABS.

- CSG (Closed Subscriber Group) Femtocell BS: A CSG Femtocell BS is accessible only to the AMSs,  which are member of the CSG, except for emergency services.

- OSG (Open Subscriber Group) Femtocell BS: An OSG Femtocell BS is accessible to any AMSs

_____________________  End Proposed Text Changes to Section 3________________________________

_____________________  Begin Proposed Text Changes to Section 4________________________________

4 Overall Network Architecture

The Network Reference Model (NRM) is a logical representation of the network architecture. The NRM

identifies functional entities and reference points over which interoperability is achieved between

functional entities. Figure 1 illustrates the NRM, consisting of the following functional entities: Mobile

Station (MS), Access Service Network (ASN), and Connectivity Service Network (CSN). The existing

network reference model is defined in WiMAX Network Architecture [8].

_____________________  End Proposed Text Changes to Section 4________________________________

_____________________Begin Proposed Text Changes to Section 12________________________________

12.1.1 LBS Network Reference Model

The LBS architecture is a functional model consistent with the WiMAX network reference model (NRM) [15].

The LBS architecture is shown illustrated in Figure 78.

_____________________End Proposed Text Changes to Section 12________________________________

_____________________Begin Proposed Text Changes to Appendix 2________________________________

Proposed change #1: Revise the first paragraph in Appendix 2 as shown below.

“The analyses in this appendix use the WiMAX network reference model [8] (WiMAX Forum Network Architecture Stage 2) specified by Network Working Group of WiMAX Forum. The details of the interfaces and

network entities can be found at http://www.wimaxforum.org/resources/documents/technical/release1. In

these analyses, it is assumed that each IEEE 802.16m 5 ms radio frame consists of eight subframes of 0.617

ms length and the transmission time interval (TTI) is equal to one subframe. The mobile station and base

station processing times are assumed to be three subframes (3*TTI).”

Proposed change #2: Delete Figure 104 and the following line that describes it. Figure 1 is identical to Figure 104.

Figure 104 illustrates the network reference model used for IEEE 802.16m

_____________________End Proposed Text Changes to Appendix 2________________________________