| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Management Message Reliability Text Clarification |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0044, IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution proposes text clarifications for the usage of length fields in various MAC headers, to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Management Message Reliability Text Clarification

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

Clarification of text on management message reliability section is needed.

# Text Proposal

[Modify the section 15.3.15 as shown below]

All MAC management messages shall be fragmentable. HARQ shall be applied to all the unicast MAC management messages.

Large messages can be fragmented for transmission. Retransmission timers shall may be defined for MAC management messages which require reliable transmission. The transmitter may retransmit a complete message or a fragment of the message if the retransmission timer expires while waiting for acknowledgement of successful transmission. Retransmission timer is started when MAC management message transmission is initiated by the transmitter to wait for the response message (for e.g. AAI_RNG_RSP is a response message for AAI_RNG_REQ). The retransmission timer is stopped if the response message is received from the receiver. If Local NAK for MAC management message is enabled, and if the HARQ is applied during the transmission of a MAC management message and if the HARQ process is terminated with an unsuccessful outcome before the expiration of the retransmission timer, the transmitter may initiate retransmission of the complete management message or the management message fragment, of transmitted in the failed HARQ burst. During retransmission initiated by Local NACK, the transmitter shall retransmit the complete management message or management message fragment with same information (FEH and management message information) as was transmitted during the initial transmission. The transmitter may retransmit the complete message, or one or more message fragments belonging to the failed message.

In addition, MAC layer acknowledgement may be used to provide reliable message delivery. When a MAC layer acknowledgement is used, the sender of the management message shall set the Polling bit to 1 in FEH of the MAC PDU containing the complete management message or the last pending fragment of the management message. When a transmitter sends a MAC PDU with complete management message or management message fragment with Polling bit  set to 1 in FEH, it shall start a ACK timer to wait for AAI_MSG_ACK message or Message ACK extended header. The ACK timer is stopped if AAI_MSG_ACK message or Message ACK extended header or response message is received from the receiver. If the ACK timer expires before an acknowledgement is received or the retransmission timer expires, the transmitter may initiate the retransmission of the entire message. The transmitter shall stop the ACK timer if the retransmission timer expires. The transmitter shall reset the retransmission timer, if it initiates the retransmission, after the ACK timer or retransmission timer expiry.

If a receiver receives management message or management message fragment in a MAC PDU with Polling bit set to ‘1’ in FEH, the receiver shall respond with requested MAC layer acknowledgement a AAI_MSG-ACK message or a Message ACK extended header or response message to indicate the reception of the complete management message. For the fragmented management messages, receiver shall sends AAI_MSG-ACK message or a Message ACK extended header after receiving all fragments of the management message must be received before the MAC layer acknowledgment is sent.

# Reference

[1] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”