| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Performance of TDOA Based on LBS Evaluation Group Assumptions/parameters |  |
| Date Submitted | 2009-09-21 |  |
| Source(s) | Kanghee Kim Kunmin Yeo ETRI 138 Gajeongno, Yuseong-gu, Daejeon, 305-700 KOREA | Voice: +82-42-860-5568 E-mail: kangheekim@etri.re.kr kunmin@etri.re.kr |
| Re: | LBS DG |  |
| Abstract | Performance analysis of downlink TDOA based on parameters/assumptions by LBS evaluation group at the first step |  |
| Purpose | Informative |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Performance of TDOA Based on LBS Evaluation Group Assumptions/Parameters

Kanghee Kim and Kunmin Yeo

ETRI

# 1. Introduction

This contribution provides performance of downlink TDOA based on parameters/assumptions [1] by LBS evaluation group to obtain the first step results. Simulation results are presented by cdf of location error and cdf of TDOA estimation error.

# 2. Simulation Environments

Most features of assumptions/parameters by LBS evaluation group are considered in simulation. Table 1 lists assumptions/parameters used here.

Wrap-around method is not applied to simplify simulation. MSs are uniformly generated only in the center cell. An MS selects the serving BS with the strongest correlation peak value among 19 cells. Therefore, the resultant difference is that 19 cells around an MS is used here though 19 cells around the serving BS is used for wrap-around method. There are two alternatives for selecting the serving BS. The first one is our choice which is proper for low speed since cell selection process follows Rayleigh fading channel. The second is to select the BS with the smallest distance loss plus shadowing loss excluding Rayleigh fast fading. It is proper for high speed. For the second choice, an instantaneous channel of the serving BS may suffer from deep fading. Thus, we expect that the performance of the first choice is better than that of the second one.

To find the first arrival path a proper threshold is used. A fixed multiple of an average correlation value is adopted. Averaging is performed over 1/3 of a preamble symbol length and starts from the beginning of the first arriving signal of the nearest BS. For the serving BS, searching is executed only over 21 samples assuming coarse synchronization. For the other two BSs, we find the strongest correlation peak timing first to locate the first path. After that, going back about tens of samples we start searching the first path above the threshold.

For positioning algorithm, least-squares method is used following literature [2].

Table 1. Simulation Assumptions/Parameters

| System Level Evaluation Assumptions/Parameters |  |  |  |
| --- | --- | --- | --- |
| Parameter/Assumption | Proposed Value | Notes/Comments | Resolution |
| OFDMA Parameters |  |  |  |
| FFT Size | 1024 | Compliant with IEEE 802.16m EMD | Agreed |
| Cyclic Prefix | 1/8 | Compliant with IEEE 802.16m EMD | Agreed |
| Bandwidth | 10 MHz | Compliant with IEEE 802.16m EMD | Agreed |
| Carrier Frequency | 2.5GHz | Compliant with IEEE 802.16m EMD | Agreed |
| Preamble | SA-Preamble, IEEE 802.16m D1 | Compliant with IEEE 802.16m D1 | Agreed |
| BS Equipment Model |  |  |  |
| Max TX Power per sector | 46 dBm | Compliant with IEEE 802.16m EMD | Agreed |
| Cable Loss | 2 dB | Compliant with IEEE 802.16m EMD | Agreed |
| Antenna gain | 17dBi, HPBW 70 deg. | Compliant with IEEE 802.16m EMD | Agreed |
| Number of TX antennas used on BS for transmitting LBS reference signals | 1 | Simplifies wideband channel measurements. Basic assumption for preamble transmission. Compliant with IEEE 802.16m D1. | Agreed |
| Antenna pattern front-to-back ratio | 20 dB | Compliant with IEEE 802.16m EMD | Agreed |
| Number of sectors | 3 | Compliant with IEEE 802.16m EMD | Agreed |
| Height | 32m | Compliant with IEEE 802.16m EMD | Agreed |
| MS Equipment Model |  |  |  |
| Number of RX antennas | 1 | To simplify efforts at least at 1st stage | Agreed |
| TX Power (not used) | 23 dBm | Compliant with IEEE 802.16m EMD | Agreed |
| Height | 1.5 m | Compliant with IEEE 802.16m EMD | Agreed |
| Noise Figure | 7 dB | Compliant with IEEE 802.16m EMD | Agreed |
| Cable Loss | 0 dB | Compliant with IEEE 802.16m EMD | Agreed |
| Antenna/gain | Omni, 0dBi | Compliant with IEEE 802.16m EMD | Agreed |
| Deployment |  |  |  |
| Wrap around (not applied) | On. Wrap around procedure is used during MS association. When association is completed the users attached to central cell are analyzed | Compliant with IEEE 802.16m EMD (at least for MS association phase) | Agreed |
| Shadow Fading (SF) | On | Compliant with IEEE 802.16m EMD | Agreed |
| SF type | Correlated shadowing factor with correlation coefficient equal to 0.5 is applied. | Compliant with IEEE 802.16m EMD | Agreed |
| SF Standard Deviation | 8 dB | Compliant with IEEE 802.16m EMD | Agreed |
| Network topology | Hexagonal deployment, 19 cells, 3 sectors | Compliant with IEEE 802.16m EMD | Agreed |
| Site to site distance | 1.5 km | Compliant with IEEE 802.16m EMD | Agreed |
| MS distribution (not applied) | Uniform distribution over area of 19 cells | Compliant with IEEE 802.16m EMD | Agreed |
| Signaling |  |  |  |
| Preamble Assignment | Unique IDcells within network | Practical assumption used during network planning | Agreed |
| Transmission | All sectors of the network transmit preamble synchronously (time aligned) | Compliant with IEEE 802.16m D1 | Agreed |
| Random OFDM data symbol | At least one random OFDM data symbol is added at the end of SA-Preamble transmission (TX power: 46 dBm) | To emulate the normal SA-Preamble transmission. Compliant with IEEE 802.16m D1. | Agreed |
| RX Processing Assumptions |  |  |  |
| MS is coarsely synchronized with serving sector | Initial synchronization error is uniformly distributed in the interval +/- 10 samples accuracy | To simplify analysis of the first arrival path timing estimation it can be assumed that MS is initially synchronized with serving BS/sector with specified ‘coarse’ synchronization accuracy | Agreed |
| First ray timing measurement | Delay of the first ray in channel impulse response carriers information about distance between BS and MS | For calibration of simulation results at the first step | Agreed |
| The first arrival path timing estimation algorithm is used for relative delay measurements | Relative delay measurements are performed using SA-preamble signals and the first arrival path timing estimation algorithm | Proponents are required to provide description of algorithm used for estimation of the first arrival path timing | Agreed |
| BS Advertisement (not applied) | MS knows coordinates and IDcells of the 19 BSs that surround its serving sector | Emulates LBS-Adv message, (Two tier advertisement) | Agreed |
| Selection of BS candidates for positioning | MS selects 3 candidate BSs | Propose to start evaluation from D-TDOA for 3 BSs. | Agreed |
| One – shot processing | RD measurements are performed using one SA-preamble symbol | For calibration of simulation results at the first step | Agreed |
| Positioning algorithm | Any TDOA based positioning algorithm can be used for the purpose of MS location. | Proponents are required to provide description of positioning algorithm used for evaluation | Agreed |
| Small Scale Сhannel Model Assumptions |  |  |  |
| Channel model | eITU PedB | For calibration of simulation results at the first step | Agreed |
| Pathloss model | Mandatory model from EMD | Compliant with IEEE 802.16m EMD | Agreed |
| Propagation delay | The real propagation delay from each BS to each MS shall be simulated | It is required to apply algorithm for estimation of the first arrival path timing | Agreed |
| Evaluation criteria/metrics |  |  |  |
| Location error | CDF of location error | Compliant with IEEE 802.16m SRD | Agreed |
| TDOA estimation error (additional metric) | CDF of TDOA estimation error | Useful metric to evaluate performance of RD estimation algorithm | Agreed |
| Statistic Accumulation |  |  |  |
| Number of MSs to be used for CDF plots | 10000 | To get accurate results | Agreed |

# 3. Performance Results

## 3.1 TDOA estimation error

Fig. 1 shows cdf of TDOA estimation error. The graph is drawn by 20,000 samples (2 samples/MS). The performance is not satisfactory.

Fig. 1. CDF of TDOA estimation error

## 3.2 Location error

Fig. 2 exhibits cdf of location error.

Fig. 2. CDF of location error

## 3.3 Ped-B and modified Ped-B

Fig. 3 gives performance difference between Ped-B and modified PedB. Moreover, additional results are shown using the strongest tap instead of the first tap. Though we cannot find any major difference or preference in this figure, the choice of a channel model may influence performance.

Fig. 3. CDF of location error for Ped-B and modified Ped-B

## 3.4 Threshold value

This section is for information. Fig. 4 shows location error results according to threshold values. We choose “x3” of an average correlation value as the threshold.

Fig. 4. CDF of location error for various threshold values

# 4. Conclusion

In this contribution, we provide performance of downlink TDOA based on parameters/assumptions by LBS evaluation group. Both TDOA estimation error and location error are not satisfactory. More elaborate and innovative algorithms may enhance the performance.

# 5. References

[1]	Assumptions/Parameters for System Level Evaluation of LBS Performance, LBS Evaluation Group v0.3, Sep. 2009.

[2]	A. H. Sayed, et al., “Network-Based Wireless Location,” IEEE Signal Processing Magazine, pp. 24-40, July 2005.