| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes in Section 15.2.11.1.1 Contention-based random access bandwidth request (15.2.11.1.1) |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek | kelvin.chou@mediatek.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Reply Comments regarding the 474 marked (unresolved) comments Section 15.2.11.1.1 Contention-based random access bandwidth request |  |
| Abstract | This contribution provides a proposed text for the bandwidth request section. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes in Section 15.2.11.1.1 Contention-based random access bandwidth request (15.2.11.1.1)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng
MediaTek Inc.

# Introduction

This contribution incorporates the comment #568, #569, #570, #571, #572, #573, #574, #575, #576 and #577.

# Proposed Text

Delete the text from line 23, pp. 152 to line 65, pp. 153 in P802.16m/D1

Add the following proposed text to line 23, pp. 152 in P802.16m/D1

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.11.1.1 Contention-based random access bandwidth request

As specified in 15.3.9.2.5, bandwidth request channel and bandwidth request preamble sequences shall be used for contention-based random access BRs. Each BR channel indicates a BR opportunity.

The AMS decides whether to use 3-step random access based procedure or 5-step random access based procedure.

The 3-step random access based BR procedure is illustrated in Figure 428—. At step 1, the AMS shall transmit a BR preamble sequence and a quick access message on a randomly selected opportunity. In response to the BR, the ABS may transmit an explicit acknowledgement, called BR-ACK A-MAP IE. This IE is defined in Table 715, and is transmitted to the AMS in the next DL frame. The ABS shall send a BR-ACK A-MAP IE in the next DL frame when it detects no BR preamble sequence, or it detects BR preamble sequence but no quick access message.

The BR-ACK A-MAP IE indicates

- the decoding status of each BR opportunity in the previous frame (no or at least one BR preamble sequence is detected)

- all the correctly received BR preamble sequences in all the BR opportunities of the previous UL frame, and

- the decoding status of the quick access message for each correctly received BR preamble sequence

If the BR-ACK Bitmap indicates no BR preamble sequence is detected at the BR opportunity selected by the AMS, or the AMS’s BR preamble sequence is not included at the selected BR opportunity in the BR-ACK A-MAP IE, the AMS shall consider that it has received a Negative-ACK.

If there is no BR-ACK A-MAP IE in the DL frame which is right after the UL frame where the AMS sent the BR, the AMS shall consider that it has received an Implicit-ACK.

Upon successfully decoding the quick access message, the ABS may provide an UL grant to the AMS, using the STID and the BR index provided in the quick access message.

An AMS shall start the BR timer at the DL frame which is right after the UL frame in which the AMS sent the BR if the AMS receives neither any UL grant nor Negative-ACK. If the BR-ACK A-MAP IE indicates successful reception of BR preamble sequence and quick access message, or the AMS receives an Implicit-ACK, the BR Timer value shall be set to the Differentiated BR timer acquired during the DSx transaction. For all other cases, the BR Timer value shall be fixed.

AMS shall stop the BR timer upon reception of the UL grant.

AMS considers the BR as failed and may restart the BR procedure when any of the following conditions is met:

- AMS receives a Negative-ACK;

- The BR timer expires.

Figure 428—3-step random access BR procedure

During the 3-step BR procedure of Figure 428, if the ABS is unable to decode the quick access message, the ABS falls back to the 5-step BR procedure illustrated in Figure 429—. In that case, in Step 2, the ABS shall provide an UL grant to the AMS using a BR ACK A-MAP IE or CDMA Allocation A-MAP IE. The ABS shall limit the maximum HARQ retransmission of the BR REQ header to 2 through the BR ACK A-MAP IE or CDMA Allocation A-MAP IE. In step 3, the AMS transmits a standalone BW REQ header or use the given UL resource for its uplink data transmission instead of the BW-REQ message. In both cases, STID shall be carried in the uplink transmission.

In case of the 5-step procedure, the AMS shall start the BR timer at step 3 when it receives a positive acknowledgement that is generated locally at the AMS. The BR Timer value shall be set to the Differentiated BR timer acquired during the DSx transaction.

AMS shall stop the BR timer upon reception of the UL grant.

The AMS may restart the BR procedure if BR timer is expired, or the AMS receives a negative acknowledgement that is generated locally at the AMS. Local positive/negative acknowledgement for BR header shall be enabled.

Figure 429—Example of 5-step random access BR procedure as a fallback mode for the 3-step BR procedure

In the regular 5-step random access BR procedure, an AMS shall send a BR preamble sequence only. If the AMS receives neither any UL grant nor Negative-ACK at the next DL frame after it has transmitted the BR preamble sequence, a fixed value timer shall be activated. The rest of the BR procedure shall be the same as the fallback 5-step procedure of Figure 429—.

------------------------------------------------------ End of Proposed Text ----------------------------------------------------