| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | System Level Evaluation Assumptions for LBS Performance Analysis |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Alexey Khoryaev, Alexander Maltsev Intel Corporation Lei Zhou, Sangheon Kim, Fangmin Xu Samsung Jinsoo Choi LGE Kanghee Kim, Kunmin Yeo ETRI | E-mail: alexey.khoryaev@intel.com alexander.maltsev@intel.com l.zhou@samsung.com sangheon.kim@samsung.com fangmin.xu@samsung.com emptylie@lge.com kangheekim@etri.re.kr kunmin@etri.re.kr |
| Re: | LBS DG |  |
| Abstract | This contribution describes assumptions/criteria for LBS evaluation |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

System Level Evaluation Assumptions for 
LBS Performance Analysis

Alexey Khoryaev, Alexander Maltsev

Intel Corporation

Lei Zhou, Sangheon Kim, Fangmin Xu

Samsung

Jinsoo Choi

LGE

Kanghee Kim, Kunmin Yeo

ETRI

This contribution defines guidelines that should be used for LBS system level evaluation. The initial version of the document was prepared during LBS DG meeting in Jeju, Korea (IEEE Session #63) and was refined during LBS DG conference calls. The document provides baseline assumptions that proponents of LBS solutions are recommended to follow for fair evaluation of LBS proposals.

The existing IEEE 802.16m EMD does not specify system level parameters and assumptions that shall be used for evaluation of LBS performance characteristics. These assumptions are essential for System Level Evaluation of LBS performance. The table below describes the parameters that were agreed during LBS DG meetings.

Table 1 Proposed Assumptions/Parameters to be used for LBS System Level Evaluation

| System Level Evaluation Assumptions/Parameters |  |  |
| --- | --- | --- |
| Parameter/Assumption | Proposed Value | Notes/Comments |
| OFDMA Parameters |  |  |
| FFT Size | 1024 | Compliant with IEEE 802.16m EMD |
| Cyclic Prefix | 1/8 | Compliant with IEEE 802.16m EMD |
| Bandwidth | 10 MHz | Compliant with IEEE 802.16m EMD |
| Carrier Frequency | 2.5GHz | Compliant with IEEE 802.16m EMD |
| Preamble | SA-Preamble, IEEE 802.16m D1 | Compliant with IEEE 802.16m D1 |
| BS Equipment Model |  |  |
| Max TX Power per sector | 46 dBm | Compliant with IEEE 802.16m EMD |
| Cable Loss | 2 dB | Compliant with IEEE 802.16m EMD |
| Antenna gain | 17dBi, HPBW 70 deg. | Compliant with IEEE 802.16m EMD |
| Number of TX antennas used on BS for transmitting LBS reference signals | 1 | Simplifies wideband channel measurements. Basic assumption for preamble transmission. Compliant with IEEE 802.16m D1. |
| Antenna pattern front-to-back ratio | 20 dB | Compliant with IEEE 802.16m EMD |
| Number of sectors | 3 | Compliant with IEEE 802.16m EMD |
| Height | 32m | Compliant with IEEE 802.16m EMD |
| MS Equipment Model |  |  |
| Number of RX antennas | 1 | To simplify efforts at least at 1st stage |
| TX Power | 23 dBm | Compliant with IEEE 802.16m EMD |
| Height | 1.5 m | Compliant with IEEE 802.16m EMD |
| Noise Figure | 7 dB | Compliant with IEEE 802.16m EMD |
| Cable Loss | 0 dB | Compliant with IEEE 802.16m EMD |
| Antenna/gain | Omni, 0dBi | Compliant with IEEE 802.16m EMD |
| Deployment |  |  |
| Wrap around | On. Wrap around procedure is used during MS association. When association is completed the users attached to central cell are analyzed | Compliant with IEEE 802.16m EMD (at least for MS association phase) |
| Shadow Fading (SF) | On | Compliant with IEEE 802.16m EMD |
| SF type | Correlated shadowing factor with correlation coefficient equal to 0.5 is applied. | Compliant with IEEE 802.16m EMD |
| SF Standard Deviation | 8 dB | Compliant with IEEE 802.16m EMD |
| Network topology | Hexagonal deployment, 19 cells, 3 sectors | Compliant with IEEE 802.16m EMD |
| Site to site distance | 1.5 km | Compliant with IEEE 802.16m EMD |
| MS distribution | Uniform distribution over area of 19 cells | Compliant with IEEE 802.16m EMD |
| Preamble Assignment | Unique IDcells within network | Practical assumption used during network planning |
| Transmission | All sectors of the network transmit preamble synchronously (time aligned) | Compliant with IEEE 802.16m D1 |
| Random OFDM data symbol | At least one random OFDM data symbol is added at the end of SA-Preamble transmission (TX power: 46 dBm for data symbols and allow boosting for preamble as per 802.16m/D1 draft ) | To emulate the normal SA-Preamble transmission. Compliant with IEEE 802.16m D1. |
| RX Processing Assumptions |  |  |
| MS is coarsely synchronized with serving sector | Initial synchronization error is uniformly distributed in the interval +/- 10 samples accuracy | To simplify analysis of the first arrival path timing estimation it can be assumed that MS is initially synchronized with serving BS/sector with specified ‘coarse’ synchronization accuracy |
| First ray timing measurement | Delay of the first ray in channel impulse response carriers information about distance between BS and MS | For calibration of simulation results at the first step |
| The first arrival path timing estimation algorithm is used for relative delay measurements | Relative delay measurements are performed using SA-preamble signals and the first arrival path timing estimation algorithm | Proponents are required to provide description of algorithm used for estimation of the first arrival path timing |
| BS Advertisement | MS knows coordinates and IDcells of the 19 BSs that surround its serving sector | Emulates LBS-Adv message, (Two tier advertisement) |
| Selection of BS candidates for positioning. MS should be able to synchronize to the selected BS. | MS selects 3 candidate BSs | Propose to start evaluation from D-TDOA for 3 BSs. |
| One – shot processing | RD measurements are performed using one SA-preamble symbol | For calibration of simulation results at the first step |
| Positioning algorithm | Any TDOA based positioning algorithm can be used for the purpose of MS location. | Proponents are required to provide description of positioning algorithm used for evaluation |
| Small Scale Сhannel Model Assumptions |  |  |
| Channel model | eITU PedB | For calibration of simulation results at the first step |
| Pathloss model | Mandatory model from EMD | Compliant with IEEE 802.16m EMD |
| Propagation delay | The real propagation delay from each BS to each MS shall be simulated | It is required to apply algorithm for estimation of the first arrival path timing |
| Evaluation criteria/metrics |  |  |
| Location error | CDF of location error | Compliant with IEEE 802.16m SRD |
| TDOA estimation error (additional metric) | CDF of TDOA estimation error | Useful metric to evaluate performance of RD estimation algorithm |
| Statistic Accumulation |  |  |
| Number of MSs to be used for CDF plots | 10000 | To get accurate results |