| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m LBS Drafting Group Output text |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Jose Puthenkulam Intel Corporation (LBS Drafting Group Chair) | Voice: +1-503-264-6121 E-mail: jose.p.puthenkulam@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions for LBS |  |
| Abstract | Text intended for section “15.6 Support for Location Based Services (LBS)” |  |
| Purpose | For adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

[Insert the following text into the sections as shown and create the sub sections as necessary:]

4. Definitions

[Insert the following abbreviations]

LBS      Location Based Services

15.2.6.5 AAI_REG-REQ

[Modify and update the following table 684 in P80216m /D1 page 52 as indicated]

Table 684—Parameters for AAI_ REG-REQ

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capabilities Negotiation Process | ARQ parameters | ARQ_SN_MODULUS | The number of unique SN value; default=2^10 |
|  |  | ARQ_WINDOW_SIZE | The maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks. |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEO UT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | LBS capabilities | GPS capability | 0: AMS is GPS capable. 1: AMS is GPS incapable. |
|  |  | Reserved | TBD |
|  | Co-located coexistence capability supported |  | Indicates if colocated coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |
| Convergence sublayer capa- bilities | Classification/PHS options and SDU encapsu- lation support |  | Indicates which classification or PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classification rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | Indicates the level of PHS support. |

15.2.6.5 AAI_REG-RSP

[Modify and update the following table 685 in P80216m /D1 page 54 as indicated]

Table 685—Parameters for AAI_ REG-RSP message

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capabilities Negotiation Process | ARQ parameters | ARQ_SN_MODULUS | The number of unique SN value; default=2^10 |
|  |  | ARQ_WINDOW_SIZE | The maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks. |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with Rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | The maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEO UT | The time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | LBS capabilities | TBD | TBD |
|  | Co-located coexistence capability supported |  | Indicates if colocated coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |
| Convergence sublayer capa- bilities | Classification/PHS options and SDU encapsulation support |  | Indicates which classification or PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classification rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | Indicates the level of PHS support. |

[Insert the following text into the section 15.6 and relevant subsections as shown]

15.6 Support for Location Based Services (LBS)

The location determination feature includes support for over the air downlink and uplink measurements and reporting of 802.16m based transmissions to help relevant entities on the AMS and/or in the network to determine AMS’s location. Location capabilities shall be supported in basic mode and may also be supported in enhanced mode as defined in the following:

The Basic LBS support (15.6.2) capability involves support of similar functionality and framework as IEEE802.16-2009 adapted to new frame and control channel structure in 802.16m. This mode of operation reuses the generic PHY and does not require any location specific PHY layer transmission.

Enhanced LBS support capability (15.6.3) may also be supported, involving over the air transmissions and measurements schemes which are specifically designed to optimize measurements needed for more accurate location determination.

15.6.1 Location Determination Capability Negotiation

The capability negotiation for LBS support and its mode are communicated through AAI_REG-REQ/RSP. (See sections 15.2.6.5 and 15.2.6.6)

All AMSs shall support basic location measurement capabilities including RSSI, RD and RTD measurements and report.

The capability negotiation also includes indications on whether the AMS supports special satellite based positioning capability, e.g. GPS.

15.6.2 Basic LBS Support

15.6.2.1  Basic functions supported using AAI-LBS-ADV message

AAI_LBS-ADV: is a MAC management message broadcast by ABS to provide AMS with geo-location of neighboring ABS’s, which can be used by the AMS for triangularization or trilaterization to enable location determination. This message may also contain time and frequency information to aid satellite based, e.g. GPS receivers for improved performance.  The content of this message and its functionality is consistent with LBS-ADV message in IEEE Std. 802.16-2009.

# 15.6.2.2 Measurements and Reporting for Location Determination

The Location measurement and report capabilities needed to support Basic LBS are the following:

- The ABS ability to provide AMS with, and the AMS’s ability to process, the AAI_LBS-ADV identifying the neighboring ABS’s which need to be scanned by the AMS as well as their locations.

- ABS capability to direct AMS to start scanning using a MAC management message, with indication that is for location determination, and to report the results to ABS using a MAC management message. This direction shall include information about which parameter the AMS to measure and report, e.g. RSSI, RD, etc., and it may also include a flag to indicate if Enhanced LBS measurements should be used.

- AMS capability to request ABS for scanning time for LBS.

- AMS’s capability for downlink scanning of SA-Preambles identified by a MAC management message to measure RSSI and RD.

- AMS and ABS capability to enable measurement of RTD based on ranging channel transmission (UL-TDOA and TOA).

- AMS providing scanning report to ABS providing measurements results based on LBS specific direction in a MAC management message.

- A MAC management message shall be used by ABS to trigger measurements in support of location. These a MAC management messages include indication that the purpose of scanning and report is for location calculation.

# Assistance for Satellite Based Location Determination

The 802.16m support to assist with satellite based location involves two functions:

The support of AAI_LBS-ADV which contain optional fields providing time and frequency information to aid satellite based, e.g. GPS, receivers for improved performance.  The content of this message and its functionality is consistent with LBS-ADV message in IEEE Std 802.16-2009.

To further assist with satellite based location determination, the AAI_L2_XFER messages may be used using the following Transfer-Types: GNSS assistance and  LBS measurement. (See section 15.2.6.28).

# 15.6.2.4 LBS Message formats

# 15.6.2.4.1 AAI_LBS-ADV Message

An ABS that supports LBS shall use the AAI_LBS-ADV message to broadcast the LBS related configuration information. The message may be broadcast periodically without solicitation by the AMS. The periodicity of this transmission is TBD.

[Insert the following Table X in this section and number it appropriately]

Table X: AAI-LBS-ADV Message Format

| Name | Value | Usage |
| --- | --- | --- |
| Number_of_BS | TBD | Number of neighbor BSs included in this message that are identified using the BSID |
| BSID | TBD | The Base Station ID parameter in the A-MAP message of the Serving BS or Neighbor BS |
| Number_of_BS_Index | TBD | Number of neighbor ABSs included in this message that are identified using an index to their position in the NBR-ADV message |
| Configuration change count for AAI_NBR-ADV | TBD | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |
| Neighbor_BS_Index | TBD | Index that corresponds to the position of the ABS in the NBR-ADV message. |
| Absolute Position (Long Format) | TBD | The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and if the Absolute Position (Short Format) is not used. |
| Absolute Position (Short Format) | TBD | The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS and if the Absolute Position (Long Format) is not used. |
| Relative Position | TBD | This Relative Position is used to provide the absolute position of an ABS. A Neighbor ABS’s coordinates. When this Relative Position is included it provides the position of a neighbor ABS relatively to the transmitting ABS. |
| GPS Time | TBD | Information about GPS time and time accuracy. |
| Frequency Accuracy | TBD | Information about the frequency accuracy |

# 15.6.2.4.1 LBS Measurement Message formats

IEEE 802.16m defines MAC control messages to assign the unavailable interval for the AMS to measure its location by receiving the DL reference signals from the candidate ABSs.

The location measurement procedure may be initiated by either the AMS or the network, which is requested by the application associated with the AMS or the client attached at the core network.

An ABS may trigger a location measurement by sending a MAC management message to the AMS. The AMS responds to this with a MAC management message with the parameters that enable location determination.

15.6.3 Enhanced LBS Support

The Enhanced LBS (ELBS) is an optional LBS capability involving the following functions:

•	ABS ability to transmit special ELBS waveforms to be used for location specific measurements to enable more accurate location determination.

•	ABS’s ability to signal to the AMS the location of the special ELBS waveforms.

•	AMS’s ability to detect, measure and report relevant measurements to the ABS.