| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Configuration Indicators |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Kaushik Josiam, Qi Wu and JaeJeong Shim Samsung Electronics Chun-Yen Wang ITRI Kaiying Lv ZTE Youngsoo Yuk LG Electronics Etemad Kamran Intel Corporation | Email : qi.wu@samsung.com k.josiam@sta.samsung.com brian.shim@samsung.com ChunYen@itri.org.tw lv.kaiying@zte.com.cn sixs@lge.com Kamran.etemad@intel.com |
| Re: |  |  |
| Abstract | This contribution defines the text and format of Control Information Elements |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for E-MBS Configuration Indicators

- Kaushik Josiam, Qi Wu, Jaejeong Shim

- Samsung Electronics

Chun-Yen Wang

- ITRI

- Kaiying Lv

- ZTE

Youngsoo Yuk

LG Electronics

Etemad Kamran

- Intel Corporation

---------------------------------------------------- Start of Text Proposal-1 ------------------------------------------------

The following text is proposed to be added to Sec 15.8.3 after line 25, Page 5 in C802.16m-09/2166

- 15.8.3.1 E-MBS Configuration Indicators

The information regarding E-MBS configuration is transmitted to the AMSs using E-MBS configuration indicators. While most E-MBS configuration indicators necessary for E-MBS operation are of interest only to the AMSs interested in E-MBS, a few E-MBS related configuration information may be broadcasted to all AMSs.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isizeoffset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content.  Some of these E-MBS configuration indicators are transmitted in a new MAC management message called the AAI-E-MBS_CFG message to be read only by the E-MBS AMSs.

-----------------------------------------------------End of the Text Proposal - 1 ------------------------------------------------

---------------------------------------------------- Start of Text Proposal-2 ------------------------------------------------

Make the following changes to the text (indicated in red) in Section 15.8.3.1 E-MBS Zone configuration beginning at line 26, Page 5 in C802.16m-09/2166

15.8.3.12 E-MBS Zone Configuration

Different MSTIDs and FIDs may be used in different E-MBS Zones for the same multicast and broadcast service flow. A multicast and broadcast zone identifier (E-MBS_ZONE_ID) is used to indicate a service area in which a MSTID and FID for a broadcast and multicast service flow are valid.  An ABS that supports E-MBS shall include the E-MBS Zone identifier(s) to which it belongs in the AAI-E-MBS_CFG message. The E-MBS Zone identifier shall not be '0'.

When the E-MBS Zone identifier list appears in AAI_NBR_ADV message with only one value of '0' for some of ABS’s  it indicates  that the corresponding neighbor ABSs are not affiliated with any E-MBS zone.

The ABS sends AAI_DSA for establishment of connection for E-MBS, E-MBS ZONE shall be encoded in the AAI-DSA message. One ABS may have multiple E-MBS Zone IDs for different E-MBS services.

- To support inter-MBS zone transition, AMSs need to get E-MBS_Zone_IDs to which the neighboring ABS’s belong as well as any flow continuity across such neighboring zone .

15.8.3.23 E-MBS Scheduling Interval (MSI)

-----------------------------------------------------End of the Text Proposal - 2 ------------------------------------------------