| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SON DG Text |  |
| Date Submitted | 2009-09-23 |  |
| Source(s) | Baowei Ji SON DG Chair | baowei.ji@samsung.com |
| Re: | P802.16m/D1 - Self-Organizing Network Call for Contributions on Project 802.16m Amendment Content : IEEE 802.16m-09/0037 |  |
| Abstract | The final SON DG text. |  |
| Purpose | To be discussed and adopted by TGm into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SON DG Text

Baowei Ji

SON DG Chair

----------------------- Start of DG Text ------------------------------------------

[Editor notes: to add this new Section 15.7]

15.7  Support for Self-organization

Self Organizing Network (SON) functions are intended for ABSs (e.g. Macro, Relay, Femto) to automate the configuration of ABS parameters and to optimize network performance, coverage and capacity.

15.7.1  Self-Organization State Diagram

ABS could perform self-organizing procedures to automate the configuration of ABS parameters and optimize network performance, coverage and capacity optimization. Self-Organization consists of two states, self-configuration and self-optimization, as illustrated in figure x1.

Figure x1. State Transition Diagram of Self-Organization

15.7.2  Self Configuration

Self-configuration is the process of initializing and configuring ABSs automatically with minimum human intervention.

15.7.2.1  Femto ABS Neighbor Discovery

Femto ABS may discover its surrounding network topology by collecting information from the core network, inter-BS network air-signaling and AMS’s scan reports.

Upon installing a Femto ABS, the Femto ABS may receive a location based neighbor list from the core network. Based on the initial neighbor list, the Femto ABS should schedule a scanning interval to scan neighbor ABSs. During the scanning interval, the Femto ABS listens to neighbor ABSs preamble. The Femto ABS broadcasts the AAI_SON-ADV message to inform such scanning intervals to AMSs.

15.7.2.2  Macro ABS Self-Configuration

Cellular networks today still require much manual configuration of neighboring macro ABS to ensure neighboring cell sites work properly and hand off connections successfully that have greatly burdened the operators. Neighboring Macro ABS Self-Configuration provides a mechanism to enable an ABS to automatically update its neighboring ABS list and their associated attributes, as neighboring ABSs are going online / offline dynamically. Since the deployment of macro ABS is planned, and their locations are known by operators, its neighbors can be automatically identified by cell site and Sector Bearing. Neighboring Macro ABS Self-configuration is to support the following 3 scenarios.

- An ABS is going online

- An ABS is going offline

- An ABS has attribute changes

ABS should report BSID, location of ABS (i.e. longitude, latitude, and sector bearing – indicating the direction where the sector is pointing), and ABS attributes, as defined in AAI_NBR-ADV messages, in order to initiate Neighbor Macro ABS Self-configuration function. In response, neighbor ABS attributes (as defined in AAI_NBR-ADV messages) in the ABS should be updated.

15.7.3  Self Optimization

Self-optimization is the process of analyzing the reported SON measurement from the ABS/AMS and fine-tuning the ABS parameters in order to optimize the network performance which includes QoS, network efficiency, throughput, cell coverage and cell capacity

SON functions can automate the configuration of ABS parameters for optimizing network performance, coverage and capacity. The air interface support for SON functions is to perform measurement/reporting of air interface performance metrics and the subsequent adjustments of ABS parameters.

The SON server is assumed be available in backhaul network when SON functions are enabled. The SON server can exchange information among different ABSs through backhaul network for inter-BS coordination.

15.7.3.1  Support of Interference Mitigation

When supporting femto interference mitigation by radio resource blocking, the allocation of the radio resource region among ABSs should be coordinated. An ABS can request to block a radio resource region identified by sub-frame index and sub-band CRU index for a target ABS. The target ABS may receive a response indicating if the target ABS should block such radio resource region that is identified by sub-frame index and sub-band CRU index.

15.7.3.2  Support of Multi-BS MIMO

Multi-BS MIMO operation may be supported by SON to coordinate the transmission among multiple ABSs. The ABS should report the feedback results defined in 15.3.14.3 received from AMSs for initiating the coordination to support multi-BS MIMO. The ABSs within the diversity set will be selected and a common zone will be assigned for those ABSs operating with DL or UL Multi-BS MIMO. The common zone used by the ABSs shall be aligned over the same time-frequency radio resource region. In case of Co-MIMO operation, this zone is the Co-MIMO zone.

15.7.4  Support of Reconfigurations and Restart

SON may trigger ABS(s) to reconfigure and/or restart. The ABS may announce the upcoming action of its reconfiguration and/or restart in advance using AAI_SON_ADV message.

[Editor notes: insert the following new Section 15.2.6.x]

15.2.6.x   AAI_SON-ADV message

This message is used by an ABS to broadcast relevant SON information for action types as defined below. Action types 1 and 2 can be sent by any ABS. Action types 3 and 4 are specific to Femto ABS only.

This message contains the following parameters:

- Action type

    - 1st value: ABS Reconfiguration

    - 2nd value: ABS Restart

    - 3rd value: ABS Scanning

    - 4th value: ABS Reliability

- If(Action type == 1st , 2nd , or 3rd value)

    - Unavailability start time (UST)

    - Unavailable Time Interval (UTI) of the ABS

- If(Action type == 4th value)

    - Reason

      - 0b0: Power down

      - 0b1: Power reduction

    - If(Reason == 0b1) dB value of Tx power reduction

    - Expected down time

    - Expected uptime

    - Recommended BSID list

[Insert the following text in P80216m /D1 page 207 line 52]

15.3.12.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement or SON measurement.

[Insert the following text in P80216m /D1 page 207 line 63]

The Paging message also includes an action code directing each AMS notified via the inclusion of its identifier as appropriate:

•0b00: Perform network re-entry

•0b01: Perform ranging to establish location

•0b10: Perform LBS measurement

•0b11: Perform SON measurement

[Modify Table 729 in P80216m /D1 as indicated]

Table 729—Parameters for AAI_PAG-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Temporary Identifier | 0~1023 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Perform SON measurement | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a pre-determined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |

[Insert the following text to P80216m /D1 15.3.12.8 as indicated]

15.3.12.8 Idle Mode Support for SON/Femto

Idle AMS may support SON measurement. The ABS signals an idle AMS that is requested to perform SON measurement through setting the Action Code to 0b11 in the AAI_PAG-ADV message. After receiving the paging message with the Action Code of 0b11, the paged AMS shall perform network reentry. The ABS shall request the AMS to perform SON measurement and report.

----------------------- End of DG Text ------------------------------------------