| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for DL HARQ buffering capability and operation |  |
| Date Submitted | 2009-10-25 |  |
| Source(s) | Tom Harel, Amir Rubin Intel Corporation | tom.harel@intel.com amir.rubin@intel.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes the text additions for defining the DL HARQ buffering manner |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for HARQ buffering capability report

Tom Harel, Amir Rubin

Intel Corporation

- Introduction

This contribution defines the manner of reporting DL HARQ buffering capability by the MS and the manner of considering this report by the BS.

In 802.16m, the reporting manner of HARQ DL buffering capability and the resulting BS consideration of it are expected to be significantly different than in 802.16e. This is the result of 2 main operational differences:

- In 802.16m the expected content of the transmissions is based on incremental redundancy mechanism from a mother code with rate of 1/3 whereas in 802.16e the content of the transmissions is according to chase combining operation.

- In 802.16m there is CRC on each FEC block and on the entire HARQ burst while in 802.16e there’s CRC only on the entire HARQ burst.

As a result of these 2 differences it seems clear that while storage of failing bursts in 802.16e is with soft bits for further combining, in 802.16m for efficient buffer utilization, the buffering of all or some of the successful FEC blocks of failing bursts would be with data bits and not with soft bits. Therefore, the capability reported of buffer size in soft bits is not entirely reflecting the buffering capability of the MS.

Additionally, in 802.16e the BS is forced to bound soft bits amount in RTT frames to the reported buffer size for guaranteed buffer availability. It leads to an implied peak throughput of: Buffer size*5/6*(200/RTT).
In 802.16m the code rate of each burst may eventually be 1/3. Trying to operate similarly to 802.16e, in a manner that guarantees storage availability for every soft bit would limit the amount of data bits in RTT frames to 1/3 of the buffer. This would impose a significant and unnecessary upper bound of the throughput. Bearing in mind the high values of throughput aimed by 802.16m system, this concept seems as not fitting to 802.16m. This understanding is consistent with the clear statement in paragraph 11.13.2.6 (Aggressive HARQ Transmission) in reference [3] that allows the ABS to transmit bits in an amount that may lead to exceeding buffer size.

Therefore, the text changes herein:

- Specifying the DL HARQ buffering capability of the AMS in the group of reported capabilities in the REG-REQ. The reported capability is the steady amount of soft bits that the MS can receive per frame in a specific defined benchmark scenario while providing a specific combining gain.

- Specifying the operation manner upon conditions of no available buffer for storing soft bits in a failing burst and upon DL HARQ buffer occupancy of 80% and above.

- References

[1] IEEE P802.16m/D2, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems” October 2009.

[2] IEEE 802.16-2009, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”.

[3] IEEE 802.16m System Description Document (SDD)

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

<Add to table 676, page 42, line 36 – Parameters for AAI_REG-REQ and to table 677, page 44, line 44 – Parameters for AAI_REG-RSP a row as follows >

| AMS HARQ buffering capability | Concatenate the following capabilities to 1 field: - The amount of steady DL units of 1024 soft bytes per frame at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 15.2.14.2.1.3. - The amount of soft bits with guaranteed buffer availability. |
| --- | --- |

<Add in page 199 line 3, section 15.2.14.2.1.3 HARQ buffering capability >

15.2.14.2.1.3 DL HARQ buffering capability

The MS shall report its buffering capability by stating the steady amount of 1024 soft byte units per frame it can receive while providing the aimed combining gain in the following benchmark scenario:

- AWGN 0Km/h channel

- Throughput is equally carried over at least 8 HARQ channels

- Burst error rate of first transmission is not higher than 50%

When there is no available buffer for storage of soft bits, the AMS shall still attempt to receive the burst and to report to the ABS on the success or failure of reception.

“Buffer overflow” is defined as a condition of no available buffer for storing soft bits in a failing burst.
Upon buffer overflow and unless the previous fast feedback was used for “buffer overflow” indication the AMS shall indicate to the ABS the overflow condition with EDI (Event Driven Indication) as defined in paragraph <<15.3.9.3.1.1>>. Upon reception of such indications the ABS may adapt its transmissions and retransmissions configurations for optimized air-link utilization.

“Buffer 80% full” is defined as a condition in which 80% or more of the soft bits in the buffer are occupied.
The AMS may report a condition of “buffer 80% full” with EDI as defined in paragraph <<15.3.9.3.1.1>>upon occurrence and unless the previous fast feedback was used for “buffer overflow” or for “buffer 80% full” indication. Upon reception of such indications the ABS may adapt its transmissions and retransmissions configurations for optimized air-link utilization.

---------------------------------------------------  Text End  ---------------------------------------------------