| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for DL HARQ buffering capability and operation, 16.2.14 |  |
| Date Submitted | 2009-12-10 |  |
| Source(s) | Tom Harel, Amir Rubin Intel Corporation | tom.harel@intel.com amir.rubin@intel.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D3 |  |
| Abstract | The contribution proposes the text additions for defining the DL HARQ buffering manner |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for HARQ buffering capability report

Tom Harel, Amir Rubin

Intel Corporation

- Introduction

This contribution defines the manner of reporting DL HARQ buffering capability by the AMS and the derived manner of operation of both the ABS and AMS.

In 802.16m, the reporting manner of HARQ DL buffering capability and the resulting BS consideration of it are expected to be significantly different than in 802.16e. This is the result of 2 main operational differences:

- In 802.16m the expected content of the transmissions is based on incremental redundancy mechanism from a mother code with rate of 1/3 whereas in 802.16e the content of the transmissions is according to chase combining operation.

- In 802.16m there is CRC on each FEC block and on the entire HARQ burst while in 802.16e there’s CRC only on the entire HARQ burst.

As a result of these 2 differences it seems clear that while storage of failing bursts in 802.16e is with soft bits for further combining, in 802.16m for efficient buffer utilization, the buffering of all or some of the successful FEC blocks in failing bursts would be with data bits and not with soft bits. Therefore, the capability reported of buffer size in soft bits is not entirely reflecting the buffering capability of the MS.

Additionally, in 802.16e the BS is forced to bound soft bits amount in RTT frames to the reported buffer size for guaranteed buffer availability. It leads to an implied peak throughput of: Buffer size*5/6*(200/RTT).
In 802.16m the code rate of each burst may eventually be 1/3.

Trying to operate similarly to 802.16e, in a manner that guarantees storage availability for every soft bit would limit the amount of data bits in RTT frames to 1/3 of the buffer. Considering the high values of throughput aimed by 802.16m system, this would impose either a significant and unnecessary upper bound of the throughput or an addition of HARQ DL buffer that has no noticed performance gain over a much smaller one.
This understanding is consistent with the clear statement in paragraph 11.13.2.6 (Aggressive HARQ Transmission) in reference [3] that allows the ABS to transmit bits in an amount that may lead to exceeding buffer size.

Although considering typical error rate of HARQ bursts and of HARQ FEC blocks leads to the understanding that overflow conditions are expected to be quite rare, the expected operation manner upon overflows is defined herein.

If overflow conditions of the DL HARQ buffer occur, the AMS is required to maintain ongoing flawless operation trying to receive its allocated bursts and to transmit feedback on the reception success. However, the manner of optimizing buffer utilization upon overflows is left for the AMS implementation.

The ABS shall consider all soft bits of failing bursts transmitted in the period between AMS DL processing time before the previous fast feedback report and AMS DL processing time before the overflow report as not buffered. The main motivation for this worst case assumption is to allow the ABS to retransmit bursts with the information bits that may have not been buffered due to their significance for CTC decoding. The ABS may consider the overflow indications and adapt the used MCS or downlink throughput. Yet, it seems that anyhow typical operation of ABS would adapt the used MCS upon such rate of errors that has lead to the overflow.

It is also proposed to indicate UL buffering capabilities of the AMS in a similar format to that of the DL. Since the UL buffer should be able to contain the entire set of information bits that may have to be retransmitted the reported capability is only with a field indicating the amount of information bits that can be stored regardless of a benchmark scenario.

Therefore, the text changes herein:

- Specify the DL HARQ buffering capability of the AMS in the group of reported capabilities in the REG-REQ and REG-RSP.

- Specify the operation manner upon conditions of overflow (no available buffer for storing all soft bits) in a failing burst.

- References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems” December 2009.

[2] IEEE 802.16-2009, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”.

[3] IEEE 802.16m System Description Document (SDD)

- Proposed changes for 802.16m draft amendment

--------------------------------------------Begin Proposed Text Change 1--------------------------------------------------

<Add to table 679, page 54, line 64 – Parameters for AAI_REG-REQ and to table 680, page 58, line 64 – Parameters for AAI_REG-RSP a row as follows >

| AMS DL HARQ buffering capability | Bits [6:0]: The amount of information bits in units of 600 bytes that the AMS can process every sub-frame. Bits [11:7]: The steady amount of aggregated DL HARQ information bits per frame in units of 9600 bytes at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| --- | --- |
| AMS UL HARQ buffering capability | Bits [6:0]: The amount of information bits in units of 600 bytes that the AMS can process every sub-frame. |

--------------------------------------------End of Proposed Text Change 1 -------------------------------------------------

--------------------------------------------Begin Proposed Text Change 2--------------------------------------------------

<Add in page 229 line 30, section 16.2.14.2.1.3 HARQ buffering capability >

16.2.14.2.1.3 DL HARQ buffering capability

The MS shall report its buffering capability by stating the steady amount of 4096 soft byte units per frame it can receive while providing the aimed combining gain in the following benchmark scenario:

- AWGN 0Km/h channel.

- Throughput is equally spread over 16 HARQ channels.

- Burst error rate of first transmission is not higher than 50%.

- Round trip time of 5mSec.

When there is no available buffer for storage of soft bits, the AMS shall still attempt to receive the burst and to report to the ABS on the success or failure of reception.

“Buffer overflow” is defined as a condition of no available buffer for storing soft bits in a failing burst.
Upon buffer overflow the AMS shall indicate to the ABS the overflow condition with EDI (Event Driven Indication) as defined in paragraph <<16.3.9.3.1.1>>.

Upon reception of overflow EDI from the AMS, the ABS may consider all soft bits of failing bursts, in the period from AMS DL Tproc before the previous FFB report from that AMS until AMS DL Tproc before the overflow EDI, as not buffered in the DL HARQ buffer. The ABS shall consider these transmissions as part of the maximum amount of retransmissions (N_MAX_ReTx). Consequently, to make sure that the systematic bits are used for decoding by the AMS, the ABS may retransmit with SPID=0 failing bursts transmitted for the first time in that period.

The ABS may adapt its DL link adaptation in a manner that aims lowering the likelihood of near reoccurrence of buffer overflows. For example such manners can be: to allocate bursts with SPIDs that were already buffered by the AMS to prevent new soft bits occupation or to lower the MCS of bursts in the near period to obtain successful reception and buffer evacuation.

--------------------------------------------End of Proposed Text Change 2--------------------------------------------------