| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to DL Subband Partitioning (15.3.5.2.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Lei.Huang@sg.panasonic.com Yoshii.Isamu@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a: P802.16m/D2 |  |
| Abstract | This contribution proposes some modifications on Section 15.3.5.2.1 (Subband Partitioning) for simplifying specification and implementation |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to DL Subband Partitioning (15.3.5.2.1)

- Lei Huang

- Panasonic Singapore Laboratories

- Isamu Yoshii

- Panasonic Corporation

- Introduction

In [1], Table 768, Table 769, and Table 770 are used to show the mapping between a 5, 4, or 3-bit signaling field DSAC (Downlink Subband Allocation Count) and KSB (i.e., number of subbands) for the 20 MHz, 10 MHz, and 5 MHz bands, respectively. In this contribution, we propose using three equivalent formulas instead of Table 768, Table 769 and Table 770 to represent the mapping between DSAC and KSB for simplifying IEEE 802.16m specification and implementation.

- Reference

- IEEE P802.16m/D2, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, October 2009

- Suggested text modification

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.3.5.2.1 Subband partitioning

Deleting Table 768, Table 769 and Table 770 and making the following modifications on line 14-16, page 289

Table 768 through Table 770 Equation (xxx-1) through Equation (xxx-3) show the mapping between DSAC and KSB for the 20 MHz, 10 MHz, and 5 MHz bands, respectively.

(xxx-1)

(xxx-2)

(xxx-3)

-------------------------------------------------------------End of the Text--------------------------------------------------------