| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to DL CRU/DRU Allocation (15.3.5.3.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Lei.Huang@sg.panasonic.com Yoshii.Isamu@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a: P802.16m/D2 |  |
| Abstract | This contribution proposes some modifications on Section 15.3.5.3.1 (CRU/DRU Allocation) |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to DL CRU/DRU Allocation (15.3.5.3.1)

- Lei Huang

- Panasonic Singapore Laboratories

- Isamu Yoshii

- Panasonic Corporation

- Introduction

In [1], Equation (191) in Page 297 defines the mapping between a 5, 4 or 3-bit signaling field DCASSB,0 and the number of subband-based CRUs for FP0 as follows:

LSB-CRU, FP0 = N1 DCASSB,0

However, from Table 768, Table 769, and Table 770, it is clearly shown that the total number of subbands KSB is upper bounded by 21, 10, and 4 for the 20 MHz, 10 MHz, and 5 MHz bands, respectively. So the number of subband-based CRUs for FP0, in a unit of subbands, is also upper bounded by 21, 10, and 4 for the 20 MHz, 10 MHz, and 5 MHz bands. Therefore, the Equation (191) is inaccurate. In this contribution, we propose some modifications on Equation (191) to accurately define the mapping between DCASSB,0 and the number of subband-based CRUs for FP0.

- Reference

- IEEE P802.16m/D2, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, October 2009

- Suggested text modification

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.3.5.3.1 CRU/DRU allocation

Making the following modifications on line 15-20, page 297

The number of subband-based CRUs for FP0 is given by the  Equation (191).  Equation (191a), Equation (191b), and Equation (191c) for 20 MHz, 10 MHz, and 5 MHz bands, respectively.

LSB-CRU, FP0 = N1 DCASSB,0                                    (191)

(191a)

(191b)

(191c)

-------------------------------------------------------------End of the Text--------------------------------------------------------