| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes to the DL/UL Persistent Allocation A-MAP IEs in IEEE P802.16m/D2 ( Sections 15.3.6.5.2.8 and 15.3.6.5.2.9) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Yi Hsuan, Aran Bergman Intel Corporation Hyunkyu Yu, Seho Kim, David Mazzarese Samsung Electronics Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny Youngho Kim, JimSam Kwak LG Electronics | roshni.m.srinivasan@intel.com hk.yu@samsung.com chegal@lge.com, sixs@lge.com |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | This document proposes the deletion of text related to the DL and UL Persistent A-MAP IEs |  |
| Purpose | To be discussed and adopted by the IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes to the DL/UL Persistent Allocation A-MAP IEs in IEEE P802.16m/D2 ( Section 15.3.6.5.2.8 and 15.3.6.5.2.9)

Roshni Srinivasan, Hujun Yin, Yi Hsuan, Aran Bergman

Intel Corporation

Hyunkyu Yu, Seho Kim, David Mazzarese

Samsung Electronics

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Ronny Youngho Kim, JimSam Kwak

LG Electronics

# Introduction

Contribution C802.16m-09/2224 [1] proposes the elimination of the mechanism to support persistent scheduling of multiple connections in P802.16m/D2 [2] based on an analysis of the additional complexity introduced in system design. Specifically, the contribution proposes the deletion of text associated with the MAC procedure for persistent scheduling of multiple connections in Subclause 15.2.7 of P802.16m/D2 [2]

This contribution complements contribution C802.16m-09/2224 in the PHY and proposes deletion of the DL and UL Composite PA A-MAP IEs Subclause 15.3.6.5.2.8 and 15.3.6.5.2.9 of IEEE P802.16m/D2 [2].

Additional cleanup of the DL/UL Persistent A-MAP IEs as a result of the deletion of the DL/UL Composite PA A-MAP IEs is also proposed.

# References

[1] “Proposed Text Changes to the MAC Procedure for Persistent Scheduling in the Advanced Air Interface in IEEE P802.16m/D2 ( Section 15.2.7)”, C802.16m-09/2224, November 2009.

[2] IEEE P802.16m/D2, October 14, 2009.

[3] Candidate IMT-Advanced RIT based on IEEE 802.16 (IEEE Contribution to ITU-R Working Party 5D), http://www.ieee802.org/16/liaison/index.html#09_010X, October 2009.

# Instructions to IEEE P802.16m/D2 Editor

This contribution proposes deletion of the DL and UL Composite PA A-MAP IEs Subclause 15.3.6.5.2.8 and 15.3.6.5.2.9 of IEEE P802.16m/D2 [1] starting on page 378, line 24.

The text to be deleted has been specified below using red font with strikeout. Text to be added has been underlined in blue font.

The following changes are proposed:

- Delete table 811, “DL Persistent A-MAP IE”

- Since persistent scheduling now only applies to individual connections, there is no need for headings specific to “individual” persistent scheduling and table 812 should be renumbered and renamed “DL Persistent A-MAP IE”.

- Since Table 811 is deleted, add text related CRC unmasking below renumbered table 812.

- Since Table 811 is deleted, add a row for A-MAP IE type in renumbered table 812.

- Change HFA value from 5 bits to 6 bits in renumbered table 812.

- Changed all the “Reserved bits” in renumbered table 812 from “TBD” to indicated values.

- Update the text following the table 812 as shown ( note the equation for Tproc is changed with the <= sign added).

- Delete the “DL Composite Persistent A-MAP IE” and table 813.

- Delete table 814, “UL Persistent A-MAP IE”

- Since persistent scheduling now only applies to individual connections, there is no need for headings specific to “individual” persistent scheduling and table 815 should be renumbered and renamed “UL Persistent A-MAP IE”.

- Since Table 814 is deleted, add text related CRC unmasking below renumbered table 815.

- Since Table 811 is deleted, add a row for A-MAP IE type in renumbered table 815.

- Change HFA value from 5 bits to 6 bits in renumbered table 815.

- Change all the “Reserved bits” in renumbered table 815 from “TBD” to indicated values.

- Delete the “UL Composite Persistent A-MAP IE” and table 816.

Following implementation of the proposed text changes, Subclause 15.3.6.5.2.8 should contain only one table (currently Table 812) describing the DL Persistent A-MAP IE and Subclause 15.3.6.5.2.9 should contain only one table (currently Table 815) describing the UL Persistent A-MAP IE.

A ‘clean’ version of the tables and text between the tables with all the proposed changes implemented has also been provided for reference.

____________________________Begin proposed text with markup____________________________

15.3.6.5.2.8 DL PA A-MAP IE

The DL persistent allocation A-MAP IE is specified in Table 811.

Table 811: DL Persistent A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent A-MAP IE |
| if MCRC is masked with Station ID { |  |  |
| DL Individual Persistent A-MAP_IE() |  | Refer to Table 812 |
| } else if MCRC is masked with Composite ID { |  |  |
| DL Composite Persistent A-MAP_IE() |  | Refer to Table 813 |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the contents of the DL Individual or Composite Persistent A-MAP IE and the CRC is masked by Station ID or the Composite ID (well-known ID specified in the system, TBD) respectively.

### DL Individual PA A-MAP IE

The DL individual persistent A-MAP IE is specified in Table 812.

Table 812 811: DL Individual Allocation Persistent A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | TBD 16 | Reserved bits |
| } else if (Allocation!= 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| } |  |  |
| Reserved | TBD 1 | Reserved bit |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the DL Persistent Allocation A-MAP IE and the CRC is masked by Station ID.

The Resource Index field in the DL Individual Persistent Allocation A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

The maximum HARQ retransmission delay for persistent allocation, PA_Max_ReTx_Delay can be computed from N_Max_ReTx, the maximum number of retransmission and PA_ReTx_Interval, the allowable delay between consecutive retransmission of persistent allocation as follows.

PA_Max_ReTx_Delay = N_Max_ReTx * PA_ReTx_Interval

where PA_ReTx_Interval is determined from Long TTI Indicator, Tproc, the data burst processing time, and Frame_length, the frame length as follows.

If Tproc3 and Long_TTI_Indicator = 0, PA_ReTx_Interval = Frame_length, else PA_ReTx_Interval = 2*Frame_length.

### DL Composite PA A-MAP IE

The DL composite persistent A-MAP IE is specified in Table 813.

Table 813: DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| } |  |  |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| if (Persistent Flag ==1 & Allocation Period==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| }else{ |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that ISizeOffset is explicitly assigned for this allocation. Otherwise, this allocation will use the same ISizeOffset as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| } |  |  |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mp | 2 | Modulation constellation of the paired user 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| } else{ |  |  |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| } |  |  |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } |  |  |
| } else if (RAI == 0b01) { |  |  |
| Resource offset | 7 | It indicates the start position of resource region for this subburst |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation |
| } |  |  |
| } |  |  |
| } |  |  |

The Resource Index field in the DL Composite Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

### 15.3.6.5.2.9 UL PA A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 814 812.

Table 814: UL Persistent A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent A-MAP IE |
| If MCRC is masked with Station ID { |  |  |
| UL Individual Persistent A-MAP_IE() |  | Refer to Table 668 |
| } else if MCRC is masked with Composite ID { |  |  |
| UL Composite Persistent A-MAP_IE() |  | Refer to Table 669 |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the contents of the UL Individual or Composite Persistent A-MAP IE and the CRC is masked by Station ID or the Composite ID (well-known ID specified in the system, TBD) respectively.

### UL Individual PA A-MAP IE

The UL individual persistent A-MAP IE is specified in Table 679.

Table 814 812: UL Individual Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation Period != 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| else if ( TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| Reserved | 1 | Reserved bit |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 5 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| } |  |  |
| Reserved | TBD2 | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Persistent Allocation A-MAP IE and the CRC is masked by Station ID.

The Resource Index field in the UL Individual Persistent Allocation A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

### UL Composite PA A-MAP IE

The UL composite persistent A-MAP IE is specified in Table 816.

Table 816: UL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| } |  |  |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| if (Persistent Flag ==1 && Allocation Period==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| }else{ |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| } |  |  |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| else if (TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } else if (RAI == 0b01) { |  |  |
| Resource offset | 7 | It indicates the start position of resource region for this subburst |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation |
| } |  |  |
| } |  |  |
| } |  |  |

The Resource Index field in the UL Composite Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

___________________________________End proposed text with markup_____________________________

______________________________Begin ‘clean’ version of proposed text_____________________________

15.3.6.5.2.8 DL PA A-MAP IE

The DL persistent allocation A-MAP IE is specified in Table 811.

Table 811: DL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation!= 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| Reserved | 1 | Reserved bit |
| } | - | - |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the DL Persistent Allocation A-MAP IE and the CRC is masked by Station ID.

The Resource Index field in the DL Persistent Allocation A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

The maximum HARQ retransmission delay for persistent allocation, PA_Max_ReTx_Delay can be computed from N_Max_ReTx, the maximum number of retransmission and PA_ReTx_Interval, the allowable delay between consecutive retransmission of persistent allocation as follows.

PA_Max_ReTx_Delay = N_Max_ReTx * PA_ReTx_Interval

where PA_ReTx_Interval is determined from Long TTI Indicator, Tproc, the data burst processing time, and Frame_length, the frame length as follows.

If Tproc3 and Long_TTI_Indicator = 0, PA_ReTx_Interval = Frame_length, else PA_ReTx_Interval = 2*Frame_length.

### 15.3.6.5.2.9 UL PA A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 812.

Table 812: UL Persistent A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation Period != 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| else if ( TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| Reserved | 1 | Reserved bit |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| Reserved | 2 | Reserved bits |
| } | - | - |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Persistent A-MAP IE and the CRC is masked by Station ID.

The Resource Index field in the UL Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

______________________________End ‘clean’ version of proposed text_____________________________