| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial Changes to the DL Basic Assignment A-MAP IE in IEEE P802.16m/D2 (15.3.6.5.2.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Roshni Srinivasan, Yi Hsuan, Hujun Yin Intel Corporation Wookbong Lee, Jinsoo Choi LG Electronics Hyunkyu Yu, David Mazzarese Samsung Electronics | roshni.m.srinivasan@intel.com wbong@lge.com, emptylie@lge.com hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 WG Letter Ballot Recirc #30a |  |
| Abstract | The contribution proposes editorial text changes IEEE P802.16m/D2 to the DL Basic Assignment A-MAP IE (15.3.6.5.2.2) |  |
| Purpose | To be discussed and adopted by the IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial Changes to the DL Basic Assignment A-MAP IE in IEEE P802.16m/D2  (15.3.6.5.2.2)

Roshni Srinivasan, Yi Hsuan, Hujun Yin

Intel Corporation

Wookbong Lee, Jinsoo Choi

LG Electronics

Hyunkyu Yu, David Mazzarese

Samsung Electronics

Resolutions adopted by TGm to the following comments in the commentary database 802.16m-09/0047r5 from session #63 and session 63.5 were not implemented in P802.16m/D2. The associated contributions that were adopted as part of the comment resolution are also listed below.

- Comment #1095 with editorial changes to Basic DL A-MAP IE (C802.16m/1928). Since the field related to the modulation constellation of the paired user “Mp” was deleted, the definition is no longer required.

- Comment #1101 suggesting deletion of expansion of “RI” in the definitions to “Resource Index”. The definition of “resource Index” has been updated in this contribution for clarification.

- Comment #1103 with suggesting deletion of redundant definition of “Resource Allocation”.

- Comment #1104 suggesting deletion of “TBD” related to HFA.

The following changes have also been made

- HFA field should have a value of 3 bits based on the HARQ Feedback Indexing described in P802.16m/D2. The update has been made in this contribution.

- SPID and CoRe version are specified separately.

Text changes with editorial markup corresponding to the above comments are proposed in this contribution.

Instructions to Editor

Table 801 appears on Page 358, starting on line 54.. Text to be deleted has been struck out and marked with red font. Text to be added has been underlined in blue font.

A ‘clean’ version of the table with all the proposed changes implemented has also been provided for reference.

_______________________________Begin proposed text with markup________________________________

< Table 801 appears on Page 358, starting on line 54. Adopt the proposed changes as shown.>

# 15.3.6.5.2.2. DL basic assignment A-MAP IE

Table 801 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL .

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 801.

Table 801 - DL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE() DL_Basic_Assignment_A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) Nt: Number of transmit antennas at the ABS. 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bits |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mp | 2 | Modulation constellation of the paired user 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: other modulation information not available. |
| } else { |  |  |
| Si | 4 | Index used when Nt= 4 or 8, to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] 3 | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | TBD 2 | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the DL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between IEs used for the UL/DL, and IEs used for resource allocation and control signaling, basic/extended IEs. Additional IE types are reserved for future use.

MEF: MIMO Encoder Format

PSI: Allocated pilot stream index for horizontal encoding

MtMt: Number of streams in transmission. The DL pilot pattern with MtMt streams shall be used in the allocated resource.

Mp: Modulation constellation of the paired user for 2 stream MU-MIMO operation with Nt = 2

Si: Index used when Nt= 4 or 8, to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission

RA: Resource Allocation information is used to signal the type of resource unit allocated (DRU/CRU), the location (start/end) and allocation size.

Resource Index: Resource Index with starting LRU index and size of a single allocation spanning contiguous LRUs.

Long TTI Indication: Indicator to signal allocations span multiple subframes in time.

HFA: TBD a Allocation for HARQ feedback.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

SPID: HARQ subpacket identifier for HARQ IR.

CRV: Constellation Rearrangement Version.

RI: Resource Index with location and size of one instance of a resource allocation.

________________________________End proposed text with markup________________________________

______________________________Begin ‘clean’ version of proposed text_____________________________

# 15.3.6.5.2.2. DL basic assignment A-MAP IE

Table 801 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 801.

Table 801 - DL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL_Basic_Assignment_A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission (Mt <= Nt ), Nt: Number of transmit antennas at the ABS. 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| Si | 4 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 2 | Reserved bits |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the DL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between IEs used for the UL/DL, and IEs used for resource allocation and control signaling. Additional IE types are reserved for future use.

MEF: MIMO Encoder Format.

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

Si: Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission.

Resource Index: Resource Index with starting LRU index and size of a single allocation spanning contiguous LRUs.

Long TTI Indication: Indicator to signal allocations span multiple subframes in time.

HFA: Allocation for HARQ feedback.

SPID: HARQ subpacket identifier for HARQ IR.

CRV: Constellation Rearrangement Version.

______________________________End ‘clean' version of proposed text_____________________________