| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the A-MAP IE Types in IEEE P802.16m/D2 (15.3.6.5.2.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Roshni Srinivasan, Yi Hsuan, Hujun Yin Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D2 related to the A-MAP IE Types currently defined in the draft. |  |
| Purpose | To be discussed and adopted by the IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the A-MAP IE Types in IEEE P802.16m/D2

Roshni Srinivasan, Yi Hsuan, Hujun Yin

Intel Corporation

Introduction

A-MAP IE Types have been updated to reflect changes in the draft IEEE P802.16m/D2. The following updates have been made:

- Two types reserved for DL and UL Group Configuration IEs have been deleted since Group Configuration will now be done through MAC control messages.

- The Feedback Polling IE has been added to the list of A-MAP IE types.

- The Broadcast A-MAP IE has been added to the list of A-MAP IE types.

Text changes with editorial markup corresponding to the above comments are proposed in this contribution.

Instructions to Editor

Table 800 appears on Page 358, starting on line 9. Text to be deleted has been struck out and marked with red font. Text to be added has been underlined in blue font.

A ‘clean’ version of the table with all the proposed changes implemented has also been provided for reference.

____________________________Begin proposed text with editorial markup____________________________

< Table 800 appears on Page 358, starting on line 9. Adopt the proposed changes as shown.>

Table 800 describes Assignment A-MAP IE Types

Table 800 -- A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | DL Subband Assignment A-MAP |
| 0b0011 | UL Subband Assignment A-MAP IE |
| 0b0100 | Feedback Allocation A-MAP IE |
| 0b0101 | UL Sounding Command A-MAP IE |
| 0b0110 | CDMA Allocation A-MAP IE |
| 0b0111 | DL Persistent A-MAP IE |
| 0b1000 | UL Persistent A-MAP IE |
| 0b1001 | DL Group Resource Allocation Configuration A-MAP IE |
| 0b1010 | UL Group Resource Allocation Configuration A-MAP IE |
| 0b1011 | DL Group Resource Allocation Feedback Polling A-MAP IE |
| 0b1100 | UL Group Resource Allocation BR-ACK A-MAP IE |
| 0b1101 | Reserved Broadcast A-MAP IE |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

____________________________End proposed text with editorial markup____________________________

______________________________Begin ‘clean’ version of proposed text_____________________________

Table 800 describes Assignment A-MAP IE Types

Table 800 -- A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | DL Subband Assignment A-MAP |
| 0b0011 | UL Subband Assignment A-MAP IE |
| 0b0100 | Feedback Allocation A-MAP IE |
| 0b0101 | UL Sounding Command A-MAP IE |
| 0b0110 | CDMA Allocation A-MAP IE |
| 0b0111 | DL Persistent A-MAP IE |
| 0b1000 | UL Persistent A-MAP IE |
| 0b1001 | DL Group Resource Allocation A-MAP IE |
| 0b1010 | UL Group Resource Allocation A-MAP IE |
| 0b1011 | Feedback Polling A-MAP IE |
| 0b1100 | BR-ACK A-MAP IE |
| 0b1101 | Broadcast A-MAP IE |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

______________________________End ‘clean’ version of proposed text_____________________________