| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial Changes to the UL Basic Assignment A-MAP IE in IEEE P802.16m/D2 (15.3.6.5.2.3) |  |
| Date Submitted | 2009-11-16 |  |
| Source(s) | Roshni Srinivasan, Yi Hsuan, Hujun Yin, Alexei Davydov, Huaning Niu Intel Corporation Wookbong Lee, Jinsoo Choi LG Electronics Hyunkyu Yu, David Mazzarese Samsung Electronics | roshni.m.srinivasan@intel.com wbong@lge.com, emptylie@lge.com hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 WG Letter Ballot Recirc #30a |  |
| Abstract | The contribution proposes editorial text changes IEEE P802.16m/D2 to the UL Basic Assignment A-MAP IE (15.3.6.5.2.3) |  |
| Purpose | To be discussed and adopted in TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial Changes to the UL Basic Assignment A-MAP IE in IEEE P802.16m/D1 (15.3.6.5.2.3)

Roshni Srinivasan, Yi Hsuan, Hujun Yin, Alexei Davydov, Huaning Niu

Intel Corporation

Wookbong Lee, Jinsoo Choi

LG Electronics

Hyunkyu Yu, David Mazzarese

Samsung Electronics

Introduction

The following changes have been made to the UL Basic Assignment A-MAP IE.

- The IE syntax has been updated from “UL-MAP_IE” to “UL_Basic_Assignment_A-MAP_IE”

- Reserved bits have been compute in order to facilitate accurate parsing of the fields of the IE.

- The HFA field should have a value of 3 bits based on the HARQ Feedback Indexing described in P802.16m/D2. The “TBD” in the HFA description has been deleted.

- The codebook size for the UL 2 Tx codebook is changed from 3 bits to 4 bits

- “Padding” has been removed since alignment with the byte boundary is not required.

Text changes with editorial markup corresponding to the above comments are proposed in this contribution.

Instructions to Editor

Table 802 appears on Page 363, starting on line 29. Text to be deleted has been struck out and marked with red font. Text to be added has been underlined in blue font.

A ‘clean’ version of the table with all the proposed changes implemented has also been provided for reference.

____________________________Begin proposed text with editorial markup____________________________

< Table 802 appears on Page 363, starting on line 29. Adopt the proposed changes as shown.>

# 15.3.6.5.2.3 UL basic assignment A-MAP IE

Table 802 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 802 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset ISizeOffset | 5 | Offset used to compute burst size index |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if (CSM-PMI Indicator == 0b0){ |  |  |
| Mt Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b0){ |  |  |
| Reserved | 5 | Reserved bits |
| else if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 | Reserved bits |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of the AMS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if (CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | [4] 3 | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | TBD1 | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

____________________________End proposed text with editorial markup____________________________

______________________________Begin ‘clean’ version of proposed text_____________________________

# 15.3.6.5.2.3 UL basic assignment A-MAP IE

Table 802 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 802 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if (CSM-PMI Indicator == 0b0){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled. 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b0){ |  |  |
| Reserved | 5 | Reserved bits |
| else if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 | Reserved bits |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if (CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

______________________________End ‘clean’ version of proposed text_____________________________