| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS Supporting Multi-resolution Transmissions with MIMO Scheme |  |
| Date Submitted | 2009-11-6 |  |
| Source(s) | Ren-Jr Chen, Chung-Lien Ho, Yan-Xiu Zheng, Yu-Chuan Fang, Chang-Lan Tsai, Hsi-Min Hsiao , Chun-Yen Wang, ITRI | rjchen@itri.org.tw zhengyanxiu@itri.org.tw tsaichangl@itri.org.tw |
| Re: | Call for contribution on P802.16m/D2, E-MBS |  |
| Abstract | The contribution proposes the text of supporting E-MBS multi-resolution transmission for P802.16m/D2. |  |
| Purpose | To be discussed and adopted by IEEE 802.16 TGm for P802.16m/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS Supporting Multi-resolution Transmissions with MIMO Scheme

Ren-Jr Chen, Chung-Lien Ho, Yan-Xiu Zheng, Yu-Chuan Fang, Chang-Lan Tsai, Hsi-Min Hsiao, Chun-Yen Wang

ITRI

# Introduction

In this contribution, a multi-resolution E-MBS transmission with MIMO is presented. In E-MBS transmission, the design goal is try to transmit more and more broadcasting information in a limited bandwidth and to support large groups of users as possible in a limited power. However, for mobile MSs, they have the capability to go to any place and change in time, thus, each MSs will have different SINR. If BSs want to support large groups of users, the most robust modulation and coding scheme (MCS) should be applied to guarantee the successful reception of MSs with low SINR. But it will sacrifice the spectral efficiency. Because there is always a trade-off in one of service bit-rate versus signal robustness, it is a problem to provided E-MBS to large groups of users with high spectral efficiency. MIMO technology can be used to increase the spectral efficiency or signal robustness depends on which mode is used. Spatial multiplexing (SM) mode is used to increase spectral efficiency, transmit diversity (TxD) mode is used to increase spectral. For example, if BSs have two antennas, and MSs have two antennas, BSs can decide to use SM mode to increase two times of spectral efficiency or TxD mode to increase the signal robustness. If BSs transmit SM mode signal, MSs can decode the SM mode signal in two conditions. One is the MSs should be equipped with more than two antennas, and the other is in higher SINR. On the other hand, MSs cannot decode SM signal with one antenna or in low SINR. If BSs transmit TxD mode signal, MSs can decode TxD signal with more than one antenna or even in low SNR, but the spectral efficiency is half of SM mode. There is also a trade-off between signal robustness and throughput.

In this contribution, a multi-resolution E-MBS transmission with MIMO is presented to adapt different MSs conditions, i.e. number of antenna in a MS, or SINR in a MS.

# Detail description of multi-resolution E-MBS transmissions with MIMO scheme

In Fig. 1, the multi-media data are separated by source encoding (like MPEG) into two information streams. One is basic information streamprovided basic quality information and the other enhancement information streamprovided higher quality information.

Fig.1 Block diagram of multi-resolution E-MBS transmissions with MIMO scheme

The detail mapping from two codewords tox1(n)andx2(n)of modulation and MIMO scheme was shown in Fig.2.

Fig.2 Detail mapping from codeword to modulation symbol

and  are the mapping from , and

s1(k)=f1(am,am+1), s2(k)=f1(am+2,am+3), s3(k)=f1(am,bm,am+1,bm+1)and s4(k)=f1(am+2,bm+2,am+3,bm+3), respectively, where , where f1(.)is QPSK mapping and f2(.)is 16QAM mapping as defined in 16m. And the mapping of MIMO scheme is

x1(n)x1(n+1)x2(n)x2(n+1)=s1(k)−s4H(k)s2(k)s3H(k).

The mapping of MIMO scheme is like to TxD mode as defined in 16m. Although,s1(k)is not exactly equal to s3(k), the mapping from two codewords tosi(k)was intended to lets1(k)close tos3(k)ands2(k)close tos4(k).

From received side, the receiver signal at MS side with two received antennas can be modeled as

r1(n)r1(n+1)r2(n)r2(n+1)=h11h12h21h22s1(k)−s4H(k)s2(k)s3H(k)+v1(n)v1(n+1)v2(n)v2(n+1)

where is the channel from th antenna at BS to th antenna at UE and v(k) is AWGN.

The receiver structure at MS is shown in the following figure

Fig. 3: Receiver structure with two antennas

UE can decide to use TxD mode detector or SM detector depends on the MS condition, like SINR, error rate performance, number of antenna. If MS is in bad condition, i.e. low SINR scenario, or poor error rate performance, MS can treat the received signal as TxD mode and decide to use TxD demapper to only retrieve the soft-value of base information stream  by the TxD demmper. The algorithm of TxD demmper as shown in the following, the MS can first process the received signal as

,

where and is the estimation of  and from first antenna and second antenna at MS, respectively. Then, the soft-value of and fromand can be retrieved by the SISO demapper, where from first antenna received signal and  from second antenna received signal. The soft-value of can be obtained by adding  and , i.e. .

If MS is in good condition, i.e. high SINR scenario, or good error rate performance, MS can treat the received signal as SM mode and decide to use SM demapper to both retrieve the soft-value of base information stream  and enhanced information stream by the SM demmper. The received signal at time index n and n+1 can be written respectively as

, and

.

This is the standard form of 2x2 MIMO model. MS can retrieve the soft-value of  and  by using any kind of MIMO demapper like, MMSE or ML .. etc.

If the MS only equip with one antenna, only the soft-value of base information stream can be retrieved. The received signal at MS with one antenna can be written as

r1(n)r1(n+1)=h11h12h21h22s1(k)−s4H(k)s2(k)s3H(k)+v1(n)v1(n+1)

The algorithm of TxD demmper is the same as in the MS with two antennas, and is shown in the following,

.

The soft-value of  can be retrieved by the SISO demapper based on .

Based on the multi-resolution E-MBS transmissions with MIMO scheme as described above, BS can transmit a MBMS signal to adapt multiple MS’s conditions with very small modification.

# The Performance of proposed multi-resolution E-MBS transmissions with MIMO scheme

Fig. 4. Performance comparison between conventional TxD, SM and ITRI’s MIMO scheme

The simulation results show the performance comparison between conventional TxD, SM and ITRI’s MIMO scheme. If MS has two antennas at receiver side, MS can decide to use TxD receiver method when MS is at bad conditions. The performance of ITRI’s MIMO scheme is less than the conventional TxD about 1 dB as shown in the Fig. 4 (blue solid line with mark *). Otherwise, when MS is at good conditions, MS can decide to use SM receiver method not only to decode basic information data but also to decode the enhanced information data. Because the basic information should be more important than the enhanced information data, more energy was put into the basic information data in ITRI’s MIMO scheme. The performance of ITRI’s MIMO scheme for basic information data is better than the conventional SM MIMO scheme with QPSK, since as shown in Fig.4 (red solid line with mark *). The performance of ITRI’s MIMO scheme for enhanced information data can be shown in Fig. 4 (red solid line with mark *). If MS only has one antenna at receiver side, MS only can use TxD receiver scheme to receive basic information data. The performance is shown in Fig. 4 (blue dot line with mark *).

The advantage of ITRI’s MIMO scheme is that it has the capability to adapt MS’s conditions (SNR condition, number of antenna condition) with very minor modification of existing system. We suggest to adapt ITRI’s MIMO scheme when transmit multi-media data in MBSFN.

# Appendix

The simulation parameters were shown bellow.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/4 |
| Resource Unit | RU (18x6) |
| Channel condition | ISD 1500 120km/h, ISD 5000 120km/h |
| The number of antennas | 2Tx, 2Rx/1Rx |
| Channel estimation for demodulation | Prefect channel estimation |
| Receiver | ML receiver |
| Coding | CTC, 1/3 code rate |
| Bit length of basic and enhanced information data | 1024 |
| Channel model | 3km/ Ped B |

# Text proposal for inclusion in the P802.16m/D2

-------------------------------  Text Start  ------------------------------------------------------------------------------------

15.3.7.4.1  Support for Multi-Resolution E-MBS Transmission

IEEE 802.16m supports multi-resolution E-MBS transmission. In this case, the multi-media data are separated into two information streams. One is basic information stream, , providing basic quality information and the other is enhanced information stream, , providing higher quality information. Every eight bits of both streams are modulated as four modulation symbols.

s1(k)=f1(am,am+1),s2(k)=f1(am+2,am+3),s3(k)=f2(am,bm,am+1,bm+1)s4(k)=f2(am+2,bm+2,am+3,bm+3)

where k = 0, 1, …, M/4-1, m = 4k ,and f1(.) and f2(.)represent QPSK and 16-QAM modulations respectively. The MIMO scheme transmits symbols for 2 antennae as the following mapping

x1(n)x1(n+1)x2(n)x2(n+1)=s1(k)−s4H(k)s2(k)s3H(k)
where the conjugate transpose of x is denoted by xH.

-------------------------------  Text End  ------------------------------------------------------------------------------------