| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femto ABS self-configuration |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Joey Chou, Xiangying Yang, Muthaiah Venkatachalam, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for the Femto ABS self-configuration. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femto ABS self-configuration

Joey Chou, Xiangying Yang, Muthaiah Venkatachalam

Intel

- Introduction

Operators can’t pre-provision the MAC / PHY attributes of Femto ABS to insure seamless integration with its neighbors, due to the following factors:

- The deployment of Femto ABS is not planned; therefore, the operators do not know where the Femto ABSs are located

- The number of Femto ABS can be too huge to be managed by operators efficiently

- Femto ABS can go up and down without operators noticed

The contribution proposes the Femto ABS self-configuration function that will choose the right set of MAC / PHY attributes, and configure Femto ABS automatically without the need truck-roll or operator intervention. This contribution is also intended to support Cell Initialization function that was adopted in SDD.

- Proposed Text

-------- Proposed text start -----------

15.7.2.3 Femto ABS Self-Configuration

In the initialization, Femto ABS will operate in the AMS mode to scan its neighbors. The ABS is not to scan its neighbors blindly, since Femto ABS can only operate in the licensed spectrum or the PHY modes pre-determined by the operators. After the scanning, the Femto ABS will report BSID and DL RSSI attributes to the SON server. The SON server will choose and download the MAC and PHY parameters that will enable the Femto ABS to work harmoniously with its neighbors.

--------------------- Proposed text end ------------