| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS assisted Femto ABS neighbor list update |  |
| Date Submitted | 2009-11-10 |  |
| Source(s) | Joey Chou, Xiangying Yang, Muthaiah Venkatachalam, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for the MS assisted Femto ABS neighbor list update. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS assisted Femto ABS neighbor list update

Joey Chou, Xiangying Yang, Muthaiah Venkatachalam

Intel

- Introduction

D2 draft includes a subclause 15.7.2.1 Femto ABS Neighbor Discovery to provide the neighbor list for a Femto ABS in the self-configuration. It is deemed this neighbor list is preliminary, and can not reflect the up-to-date status of neighbor list, as Femto ABS can be powered on or off intermittently at its owner’s wish. Since the neighbor list is intended to assist MS handoff, MS is in the best position to discover the neighbor list. This contribution proposes the MS assisted Femto ABS neighbor discovery mechanism to update the Femto ABS neighbor dynamically.

- Proposed Text

15.7.5 MS assisted Femto ABS neighbor list update

Neighbor list as provided by Femto ABS Neighbor Discovery in subclause 15.7.2.1 during the self-configuration,  can not reflect the up-to-date neighbor list, as Femto ABS can be powered on or off intermittently at its owner’s wish. Since the neighbor list is intended to assist MS handoff, MS is in the best position to discover the neighbor list. ABS should send AAI_SCN-RSP periodically to ask an AMS to scan neighbors that are not limited to BSID provided in AAI_NBR-ADV. The AMS is not to scan its neighbors blindly, since Femto ABS can only operate in the licensed spectrum or the PHY modes pre-determined by the operators. In each MS scanning, the ABS should collect and report MS measurements from AAI_SCN-REP, MS location, and time of the scanning to the SON server. The SON server should update the neighbor list of a Femto ABS based on multiple AMS measurements.