| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS assisted Femto ABS neighbor list update |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Joey Chou, Xiangying Yang, Muthaiah Venkatachalam, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for the MS assisted Femto ABS neighbor list update. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS assisted Femto ABS neighbor list update

Joey Chou, Xiangying Yang, Muthaiah Venkatachalam

Intel

- Introduction

Femto ABS Neighbor Discovery, as described in subclause 16.7.2.1 is to provide the neighbor list for a Femto ABS at the time of Femto ABS initialization. A Femto ABS can be powered on or off intermittently at its owner’s wish, so its neighbor list has to be updated periodically. Since the neighbor list is intended to assist MS handoff, MS is in the best position to discover the neighbor list. Therefore, this contribution proposes a method for MS to assist Femto ABS neighbor list update.

- Proposed Text (1)

--------------------------------- Start of proposed text -----------------------------

10.1 Global values

Table 554—Parameters and constants

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| ABS | Neighbor List Update Interval | Time between Femto ABS neighbor list updates | 10 minute | 60 minutes | 1440 minutes |

16.7.5 MS assisted Femto ABS neighbor list update

Femto ABS Neighbor Discovery, as described in 16.7.2.1 is to provide the neighbor list for a Femto ABS at the time of Femto ABS initialization. A Femto ABS can be powered on or off intermittently at its owner’s wish, so its neighbor list has to be updated periodically. Since the neighbor list is intended to assist MS handoff, MS is in the best position to discover the neighbor list. Figure xxx shows the control flow of Femto ABS neighbor list update. At each Neighbor List Update Interval, as defined in Table 554 , ABS may send AAI_SCN-RSP periodically to ask an AMS to scan neighbors that are not limited to BSID provided in AAI_NBR-ADV. The AMS is not to scan its neighbors blindly, since Femto ABS can only operate in the licensed spectrum or the PHY modes pre-determined by the operators. In each MS scanning, the ABS should collect and report MS measurements from AAI_SCN-REP, MS location, and time of the scanning to the SON server.

Figure xxx Femto ABS neighbor list update control flow

--------------------------------- End of proposed text -----------------------------