| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of Delay-intolerant MAC control message |  |
| Date Submitted | 2009-11-08 |  |
| Source(s) | Xiangying Yang, Joey Chou, Muthaiah Venkatachalam, Intel | E-mail: xiangying.yang@intel.com joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for the Compact MAC header. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support of Delay-intolerant MAC control message

Xiangying Yang, Joey Chou, Muthaiah Venkatachalam

Intel

- Introduction

D2 draft only includes a FID for MAC control message that is not able to distinguish delay-intolerant and delay-tolerant message. This contribution adds a channel ID in the FEH to support delay-intolerant and delay-tolerant messages.

- Proposed Text

15.2.2.2.2 Fragmentation extended header (FEH)

The FEH shall be used when MAC PDU contains payload from a single management connection. that are The FEH format is defined in Table 660.

Table 660—FEH Format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| FEH () { |  |  |
| BC |  | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Channel ID | 1 | 0: basic MAC management channel to transport delay-intolerant messages 1: primary MAC management channel to transport delay-tolerant messages |
| SN Indicator |  |  |
| If (SN Indicator = 0) { |  |  |
| Reserved | 65 | For byte alignment |
| else { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see <<<table TBD>>>) |
| SN | 8 | Payload sequence number |
| Reserved | 32 | For byte alignment |
| } |  |  |
| } |  |  |