| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification for the Feedback Allocation A-MAP IE (15.3.6.5.2.11) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Heewon Kang, Hokyu Choi Samsung Electronics | E-mail: d.mazzarese@samsung.com |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | The contribution proposes modifications to the feedback allocation A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification for the Feedback Allocation A-MAP IE (15.3.6.5.2.11)

David Mazzarese, Sangheon Kim, Heewon Kang, Hokyu Choi

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides modifications for feedback allocation A-MAP IEs in order to specify the fast feedback channel index size and the UL frame and UL subframe where the feedback resource is located.

- 6 bits are reserved for the channel index, which allows to to indicate any of the maximum 48 fast feedback channels allowed to be present in the uplink control resource region, which size is defined in S-SFH SP3 by UL Fast FB Size using 4 bits to indicate a maximum of 16 LRUs.

- Frame offset is reduced to 2 bits to indicate feedback in the current frame (frame offset = 0), or 1, 2 or 3 frames later. This offers enough flexibility in the scheduling. The previous choice of 8 frames is unnecessary. Allowing the feedback in the same frame is important for CQI reporting in sleep mode during listening window.

- Subframe index is introduced to indicate the UL subframe in which the feedback channel is allocated. 3 bits allow to indicate any UL subframe in all TDD and FDD cases.

This contribution also proposes changes to the MIMO parameters:

- Indication of parameters CCE and CS for MFM 7 are separated from MFM 3 and MFM 6. MFM 7 with wideband beamforming does not need to indicate CM, because it makes no sense to feedback a wideband PMI from the transformed codebook.

- The note explaining parameter CS is clarified, because it should be interpreted differently for MFM 3, 6 and for MFM 7. When the codebook subset feedback is enabled, the PMI may be a base PMI or a transformed PMI for MFM 3 and MFM 6, but it should only be a base PMI for MFM 7.

- “Standard mode” is replaced by “Base mode”.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

- Text Proposal

<Modify section 15.3.6.5.1.11 on page 391 line 52 as shown below>

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.6.5.2.11 Feedback Allocation A-MAP IE

Table 818 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating an UL fast feedback control channels (including both PFBCH and or SFBCH) to an AMS. If an AMS has an existing fast feedback control channel and receives a new feedback channel allocation, the original fast feedback channel is de-allocated automatically.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 818.

Table 818 - Feedback Allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| - A-MAP IE Type | [4] | - Feedback Allocation A-MAP IE = 0b0010 |
| - Channel Index | Variable 6 | - Feedback channel index within the UL fast feedback control resource region - (Dependent on LFB,FPi defined in section 15.3.8.3.3.2). |
| Short-term Feedback Period (p) | [3] | A feedback is transmitted on the FBCH every 2p frames. |
| Long-term Feedback Period (q) | [2] | A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunity. If q = 0b00, long-term feedback is not used. |
| - Frame offset | [3] | The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the AMS should starts reporting in eight frames. |
| - Subframe index | - 3 | - Indicate the UL subframe index in the UL portion of the frame |
| - Allocation Duration(d) | [3] | A FBCH is transmitted on the FBCH channels indexed by Channel Index for 82d frames. If d = 0b000, the FBCH is deallocated. If d=0b111, the AMS should shall report until the ABS command for the AMS to stop. |
| - ACK Allocation Flag | [1] | Indicate if one ACK channel is allocated to acknowledge the successful detection of this IE. |
| - If (ACK Allocation Flag == 0b1){ |  |  |
| - HFA | [3] | - HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } |  |  |
| - MFM | - 3 | - MIMO Feedback Mode as defined in Table 831 |
| MaxMt MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt Nt - If Nt Nt =2 (SU-MIMO and MU-MIMO Any MFM): - 0b0: 1 - 0b1: 2 - If Nt Nt =4 (SU-MIMO and MU-MIMO Any MFM): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt Nt =8 (SU-MIMO MFM 0, 1, 2, 3, 4): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - 0b00: 1 - 0b01: 2 - 0b10: 4 - 0b11: 8 - If Nt Nt =8 (MU-MIMO MFM 5, 6, 7): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| - MFM | [3] | - MIMO Feedback Mode as defined in Table 671 |
| If (MFM = 2, 3, 5, 6) { |  |  |
| - Feedback Format | [2] |  |
| } |  |  |
| If(MFM=0,1,4,7){ |  |  |
| FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM == 0,1AND long term feedback q!=0b00) |  |  |
| { |  |  |
| Long term FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | [2] | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard base mode with CCE disabled 0b01: transformation mode with CCE disabled 0b10: differential mode with CCE disabled 0b11: standard base mode with CCE enabled |
| CS | [1] | Codebook subset if Nt=4, otherwise CS shall be ignored. 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from the codebook subset or transformed codebook subset |
| } |  |  |
| If(MFM==0, 1, 2, 5){ |  |  |
| Measurement Method Indication | [1] | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt MaxMt streams for CQI measurements |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

- A 16 bit CRC is generated based on the contents of the Feedback Allocation A-MAP IE. The CRC is masked by the Station ID.

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index channel index and the AMS.

ACK Allocation Flag: BS may set ACK Allocation Flag to 0b1 if Allocation Duration equals 0b00. BS may set ACK Allocation Flag to 0b1 if Allocation Duration doesn’t equal 0b00 and the channel index of the newly allocated FBCH is the same as the channel index of the deallocated FBCH.

Short-term Feedback Period (p): A short-term feedback is transmitted on the FBCH every 2p frames

Long-term Feedback Period (q): A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunity. If q = 0b00, long-term feedback is not used.

Allocation duration (d): A FBCH is transmitted on the FBCH channels indexed by Channel Index for 82d frames. If d = 0b000, the FBCH is deallocated. If d=0b111, the AMS shall report until the ABS command for the AMS to stop.

MFM: MIMO feedback mode, defined in Table 831.

Feedback Format: This field specifies the feedback format index when reporting fast feedback information in FBCH. Feedback format definitions for different MIMO feedback modes are described in 15.3.9.3.1.4.

FPI: The frequency partition over which the short term period report shall be measured by the AMS.

Long term FPI: The frequency partition over which the long term period report shall be measured by the AMS.

MaxMt MaxMt: This field specifies the maximum rank to be fed back by the AMS if MFM=0,1,2,3,4 (which indicates a SU MIMO feedback mode for SM), or it specifies the maximum number of users scheduled on each RU at the BS if MFM=5,6,7 (which indicates a MU MIMO feedback mode)

CCE: Codebook Coordination Enable. When CCE is enabled, MS finds PMI within whole broadcasted codebook type entry; when the CMI CM field is set to 0b1, it means AMS finds rate-1 PMI within broadcasted codebook entries indicated by BC_SI and codebook subset indication CS.

CM: this field specifies codebook feedback mode, which is needed only for MFM 3 and 6.

Measurement Method Indication: This field indicates the use of midamble or pilots for CQI measurement.

----------------------------------------- End of the Proposed Amendment Text --------------------------------------------