| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE for single-BS feedback (15.3.6.5.2.14) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | Modifications are proposed for the feedback polling A-MAP IE for single-BS feedback |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to the feedback polling A-MAP IE for single-BS feedback (15.3.6.5.2.14)

David Mazzarese, Sangheon Kim, Heewon Kang, Hokyu Choi

Samsung Electronics Co., Ltd.

# Introduction

Modifications are proposed for the feedback polling A-MAP IE operation in order to allow:

- Assignment of MEF for uplink transmission of the feedback for both single-BS feedback and multi-BS feedback, whereas it is only feasible for single-BS feedback in [1].

Additionally, some changes are made to the parameters that configure the feedback information:

- The number of best subbands requested for feedback is reduced to 1, 6, 12 and all subbands. Other values previously possible do not offer a real benefit compared to what is achievable with the SFBCH allocated with the Feedback Allocation A-MAP IE.

- The maximum number of subbands is changed from Nsub to YSB.

- Similar to a proposal on the Feedback Allocation A-MAP IE, signaling of CS and CCE for MFM 7 is separated from MFM 3 and MFM 6. There is no need to feedback a PMI from the transformed codebook, since both PMI and correlation matrix are wideband in this case because MFM 7 operates in NLRU.

- Similar to a proposal on the Feedback Allocation A-MAP IE, the description of CS field is clarified. The PMI should be a base PMI or a transformed PMI, depending on the indicated in CM, except for MFM 7.

- FPk is corrected to k = floor(IDCell/255256).

- The rules for the usage of extended header of MAC control message are clarified.

Some additional minor editorial changes and clarifications are also proposed.

The introduction part is clarified by using bullet points.

- References

[1] DRAFT amendment to IEEE standard for local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.6.5.2.14 Feedback Polling A-MAP IE” on line 19 page 397 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

15.3.6.5.4.12 Feedback Polling A-MAP IE

The information element shown in Table 821 is used by the ABS to schedule MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Dedicated UL Allocation bit is set to 0b1, a dedicated UL allocation shall be included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Dedicated UL Allocation bit is set to 0b0, no dedicated UL allocation shall be included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE.

The MS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The MS shall send the feedback in a MAC control message when the following feedback information is requested:

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

- multi-BS feedback

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

If the feedback includes only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas, or only the wideband information for MIMO feedback modes 0, 1, 4 and 7, or only the subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6, then the MS shall use an extended header. In other cases, for the feedback of the quantized transmit correlation matrix when the ABS is equipped with 8 transmit antennas, or for the feedback of subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6, or for multi-BS feedback, the MS shall use a MAC control message. In case of feedback for MIMO feedback modes 0, 1, 4 or 7, the MS shall feedback the CQI for FP0 if FPCT equals 1 or 4, or for FPk if FPCT is not equal to 1 or 4, where FPk is determined by k = floor(IDCell/255256).

If MIMO_feedback_IE_type = 0b1, the AMS shall transmit using MIMO mode 0 if it has multiple transmit antennas, or using MIMO mode 1 with Mt=1 stream if it has a single transmit antenna. If MIMO_feedback_IE_type = 0b0, the AMS shall follow the MEF instruction in the IE.

Table 821 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| A-MAP IE Type { | [4] | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000){ |  | Feedback allocation |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 t24 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 1 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b0: SFBC 0b1: VE with Mt = 2 if Nt > 1, or VE with Mt = 1 if Nt = 1 Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Period (p) | 3 | Transmit feedback header every 4p frame. The first report shall start at the next frame. |
| MIMO_feedback_IE_type | 1 t29 | - 0b0: feedback for single-BS MIMO operation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| If (Dedicated UL allocation == 0b1){ |  |  |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding with Mt = 2 if Nt=2 or 4, or Mt = 1 if Nt=1 - Non-adaptive precoding shall be used at the MS. Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Transmit_Correlation_Matrix | 1 t30 | - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit correlation matrix shall be feedback if CM = 0b1 |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt MaxMt | Variable 1 or 2 T32 | Variable number of bits - depends on number of transmit antenna Nt Nt at the ABS - If Nt Nt=2 (SU-MIMO and MU-MIMO Any MFM): - 0b0: 1 - 0b1: 2 - If Nt Nt =4 (SU-MIMO and MU-MIMO Any MFM): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt Nt =8 (SU-MIMO MFM 0, 1, 2, 3, 4): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - 0b00: 1 - 0b01: 2 - 0b10: 4 - 0b11: 8 - If Nt Nt =8 (MU-MIMO MFM 5, 6, 7): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| - MFM | - 3 t35 | - MIMO Feedback Mode for which the AMS shall transmit feedback. CQI and CSI to the ABS |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 3 2 t37 | 0b000: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 0b001~0b111: Number of best subbands to report 1< Num_best_subbands ≤ Nsub YSB |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | Feedback of CQI and CSI for CL SU and MU MIMO |
| CM | [TBD] | [TBD] |
| CS | 1 t40 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  | Indication of report for OL region |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| CPI | 1 | Concatenating PMI Feedback indication; 0b0: feedback CPMI for N_multiBS_reports-1 diversity set members; 0b1: no feedback CPMI |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Reserved | 1 |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | 16 | 16 bit CRC masked by Station ID |

A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID.

Parameters in the Feedback Polling A-MAP IE should be interpreted as for the Feedback Allocation A-MAP IE.

-------------------------------  End of Text Proposal  ---------------------------------------------------