| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to the uplink feedback formats (15.3.9.3.1.4) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Chiwoo Lim, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: d.mazzarese@samsung.com |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | This contribution proposes change to the feedback formats for uplink fast feedback channels |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications to the uplink feedback formats (15.3.9.3.1.4)

David Mazzarese, Chiwoo Lim, Sangheon Kim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes changes to the feedback formats:

- The bit size column is removed from the tables. Instead, the feedback parameters are listed in the introduction paragraph and their bit size is described there.

- The information bit ordering is specified in the tables when feedback information is sent for multiple subbands, using a for loop

- MFM 4 is removed, because it can be operated with MFM 7 with MaxMt=1

- When only one feedback period is specified, we use the “long period” rather than the “short period”, because the long period signaled in the Feedback Polling A-MAP IE can take all the values of the short period, and in addition in can take larger values than the largest value possible with the short period.

- The reference for the differential CQI is changed to the average CQI at all feedback times, rather than to the previously computed CQI based on previous differential CQI expression. The effect is that the differential feedback sent over several consecutive frames is referenced to the fixed average CQI value of the long period.

- Editorial clean-up is also provided in this contribution

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

# Proposed AWD text

<Modify the section 15.3.9.3.1.4 on line 44, page 502 as shown >

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Feedback format

The detailed format for MIMO feedback mode 3 with differential codebook is in Table 892. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M > 1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M = 1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE.

Feedback formats define the information content carried by the fast feedback channels. The format of the content of the PFBCH is determined by the PFBCH encoding type. The SFBCH payload information bits may carry subband index, STC rate, wideband STC rate, PFBCH indicator, subband CQI , subband avg_CQI, differential CQI, wideband PMI, subband PMI, base PMI and differential PMI. The SFBCH payload information bits a0a1...al-1 are defined in the order specified in Tables 887 to 895.

Subband index shall be encoded withbits, where KSB is determined by USAC, which is transmitted in SFH, depending on system bandwidth.  is the combination operation.

STC rate and wideband STC rate shall be encoded with 1, 2 or 3 bits when the ABS has 2, 4 or 8 transmit antennas, respectively.
Subband CQI and subband avg_CQI shall be encoded with 4 bits corresponding to the nominal MCS of Table 886.
Differential CQI shall be encoded with 2 bits indicating values {-1, 0, +1, +2}.
Wideband PMI, subband PMI and base PMI shall be encoded with 3, 4 or 6 bits according to the codebook size.
Differential PMI shall be encoded with 2, 4 or 4 bits when the ABS has 2, 4 or 8 transmit antennas, respectively.

# Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 1, 4, and 7 are listed in Table 887. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report if MFM = 4 or 7, or by puncturing the 2q-th short period report if MFM = 0 or 1, where q is defined in the feedback allocation A-MAP IE. The wideband CQI is one average CQI corresponding to the MIMO mode signaled by the combination MFM and STC rate, with averaging over the whole band. In the case where the number of DL frequency partitions FPCT > 1, the wideband CQI is one average CQI over the corresponding frequency partition.

Table 887 shows feedback formats for MIMO feedback mode 0, 1, 4, and 7 when Measurement Method Indication = 0b0 in Feedback Allocation A-MAP IE (operation outside the open-loop region):

Table 887 - Feedback formats for MIMO feedback mode 0, 1, 4, and 7

| MFM | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 1 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 4 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |
| 7 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |

Table 888 shows Feedback formats for MIMO feedback mode 0, 1, 4, and 7 when Measurement Method Indication = 0b1 in Feedback Allocation A-MAP IE (operation inside the open-loop region)

Table 888 - Feedback formats for MIMO feedback mode 0, 1, 4, and 7

| MFM | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | Short Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
| 1 | PFBCH | 1 | Short Long | Wideband CQI | N/A | CQI for STC rate = 1/2 Encoding type 3 |

# Feedback format for MFM 2

The detailed format is listed in Table 889. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 889 shows Feedback formats for MIMO feedback mode 2 when Measurement Method Indication = 0b0 in Feedback Allocation A-MAP IE (operation outside the open-loop region)

Table 889 - Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and STC rate (rate=1 and 2) | N/A | Joint encoding of CQI and STC rate with PFBCH PFBCH encoding Type 0 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Long | Subband index Subband CQI STC rate PFBCH indicator | 3, 4, or 5 4 1~3 1 | Subband index for 5, 10, or 20MHz Support of STC rate 1 to 8 |
| 2 (M = 3 5) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | 4 23 = 6 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 5, 8 or 11 1~3 1 |  |
| 3 (M = 5 10) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | 4 25 = 10 |  |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 3, 10, or 16 1~3 1 |  |

Table 890 shows Feedback formats for MIMO feedback mode 2 when Measurement Method Indication = 0b1 in Feedback Allocation A-MAP IE (operation inside the open-loop region).

Table 890 - Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI for STC rate=1/2 | N/A | Joint encoding of CQI with STC rate =1/2 PFBCH encoding Type 3 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Long | Subband index Subband CQI PFBCH indicator | 3, 4, or 5 4 1 | STC rate = 1/2 |
| 2 (M = 3) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | 4 23 = 6 | Subband index for 5, 10, or 20MHz STC rate = 1/2 |
|  |  |  | Long | Subband index PFBCH indicator | 5, 8 or 11 1 |  |
| 3 (M = 5) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | 4 25 = 10 |  |
|  |  |  | Long | Subband index PFBCH indicator | 3, 10, or 16 1 |  |

# Feedback format for MFM 3

The detailed format is listed in Table 891. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after every report of subband index. Subband CQI and subband PMI are then transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted using encoding type 0 by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 891 - Feedback formats for MIMO feedback mode 3

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 STC rate = 1 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 7 or 9 1~3 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 5, 8 or 11 1~3 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 9 or 14 1~3 4 1 |  |

The detailed format for MIMO feedback mode 3 with differential codebook is in Table 892. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M > 1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M = 1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE.

Table 892 - Feedback formats for MIMO feedback mode 3 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Padding | 4 2-4 0~1 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) Padding 1 bit: If Nt=2 0 bit: Otherwise |
|  |  |  | Long | Subband index STC rate Base PMI PFBCH indicator | 3, 4 or 5 1 to 3 3-6 1 | Subband index for 5, 10, or 20MHz PMI for reset. |
| 1 (M=2) | SFBCH | 3 | Short | Diff CQI Differential PMI for (m=1:M){ Subband differential CQI Subband Differential PMI } | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Avg CQI subband avg_CQI PFBCH indicator | 4, 7 or 9 4 1 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | for (m=1:M){ Base PMI } Wideband STC rate | 2×3~2×6=6~12 1~3 |  |
| 2 (M =3) | SFBCH | 3 | Short | Diff CQI Differential PMI for (m=1:M){ Subband differential CQI Subband Differential PMI } | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Wideband STC rate Avg CQI subband avg_CQI PFBCH indicator | 8 or 10 1~3 4 1 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | for (m=1:M){ Base PMI } | 3×3~3×6=9~18 |  |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI for (m=1:M){ Subband differential CQI Subband Differential PMI } | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Wideband STC rate Avg CQI subband avg_CQI PFBCH indicator | 4, 9 or 14 1~3 4 1 | Subband index for 5, 10, or 20MHz (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | for (m=1:M){ Base PMI } | 4×3~4×4=12~16 |  |

# Feedback format for MFM 5

The detailed format is listed in Table 893. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 893 - Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 1 | Short Long | Subband index Subband CQI Stream Index PFBCH indicator | 3, 4, or 5 4 1~2 1 | Subband index for 5, 10, or 20MHz |
| 1 (M=2) | SFBCH | 2 | Short | Subband avg_CQI for (m=1:M){ Subband differential CQI Stream Index } | 4 22=4 (1~2) 2 |  |
|  |  |  | Long | Subband index PFBCH indicator | 4, 7, or 9 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Subband avg_CQI for (m=1:M){ Subband differential CQI Stream Index } | 4 23=6 3 (1~2) |  |
|  |  |  | Long | Subband index PFBCH indicator | 5, 8, or 11 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =5) | SFBCH | 3 | Short | Subband avg_CQI for (m=1:M){ Subband differential CQI Stream Index } | 4 25=10 (1~2) 5 |  |
|  |  |  | Long | Subband index PFBCH indicator | 3, 10, or 16 1 | Subband index for 5, 10, or 20MHz |

# Feedback format for MFM 6

The detailed format is listed in Table 894. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after every report of subband index. Subband CQI and subband PMI are then transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted using encoding type 0 by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 894 - Feedback formats for MIMO feedback mode 6

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 7 or 9 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 5, 8 or 11 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 9 or 14 4 1 |  |

The detailed format for MIMO feedback mode 6 with differential codebook is in Table 895. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M>1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M=1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE.

Table 895 - Feedback formats for MIMO feedback mode 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Padding | 4 2~4 0~1 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) Padding 1 bit: If Nt=2 0 bit: Otherwise |
|  |  |  | Long | Subband selection index Base PMI PFBCH indicator | 3, 4 or 5 3-6 1 | PMI for reset. |
| 1 (M=2) | SFBCH | 3 | Short | for (m=1:M){ Subband differential Diff CQI Subband Differential PMI } | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection index Avg CQI subband avg_CQI PFBCH indicator | 4, 7 or 9 4 1 |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | 2×3~2×6=6~12 |  |
| 2 (M =3) | SFBCH | 3 | Short | for (m=1:M){ Subband differential Diff CQI Subband Differential PMI } | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection index Avg CQI subband avg_CQI PFBCH indicator | 5, 8 or 10 4 1 |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | 3×3~3×6=9~18 |  |
| 3 (M =4) | SFBCH | 3 | Short | for (m=1:M){ Subband differential Diff CQI Subband Differential PMI } | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Avg CQI subband avg_CQI PFBCH indicator | 4, 9 or 14 4 1 | Subband index for 5, 10, or 20MHz (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | for (m=1:M){ Base PMI } | 4×3~4×4=12~16 |  |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------