| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Blind Area Detection and Reporting in SON operation (15.7.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jung-Mao Lin, Chie Ming Chou, Frank Ren, Yung-Han Chen, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | E-mail: moumou3@itri.org.tw whsheen@itri.org.tw |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes the blind area detection and reporting in SON operation (Section 15.7.3) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Blind Area Detection and Reporting in SON operation

Jung-Mao Lin, Chie Ming Chou, Frank Ren, Yung-Han Chen, Chun-Yuan Chiu

ITRI

Wern Ho Sheen

Chaoyang University of Technology / ITRI

- Introduction

Base on SDD, one of SON objectives is to optimize the coverage and capability. To support that, how to detect and resolve the blind area is an important issue. However, the details of the blind area detection and resolution are not clear in current P802.16m/D2. In this contribution, we propose the blind area detection and reporting procedure for an AMS. The blind area resolution by SON operation is proposed in C802.16m_09-2263.

- Discussion and proposal

Depend on different blind area scenarios; network can employ different approaches to resolve the blind area. For example, if the blind areas appear on the cell boundary, an ABS may reconfigure the transmission power to extend its coverage to cover the blind area. Oppositely, if the blind areas do not appear on the cell boundary, operator may deploy new Femto or relay ABS to cover this blind area. To differentiate which blind area scenario was happened, the detection and reporting procedures are discussed:

  - Blind area appears on the cell boundary：

When blind area appears on the cell boundary, it means that this area was not been covered by any ABS. This kind of blind area may happen when ABSs reduce transmission power for interference mitigation. In this scenario, AMS can detect this kind of blind areas by the scanning process. Figure 1 is an example: AMS1 moves from ABS1 to ABS2 and there is a blind area between ABS1 and ABS2. Due to the decrease of signaling quality, AMS1 or ABS1 will initiate the scanning procedure at time T1. Based on the neighboring list, ABS2 will be one of the scanning targets. However, AMS1 can not receive any signaling, and no reporting will be made (in event-trigger report mode) or AMS will send AAI_SCN-REP with N_Neighbor_BS_Index=0 (in periodic-trigger report mode). By the way, the HO process may be unable to apply and AMS1 will lose its connection soon. Through this discussion, when an AMS moving away from its serving ABS and can not scan any other available ABSs, the serving ABS can decide that blind area was happened on its cell boundary by checking the scan report. And the resolution of blind area is needed.

Figure 1, an example of blind area on the cell boundary

  - Blind area does not appear on cell boundary：

If blind area does not appear on the cell boundary, an AMS will unexpectedly disconnect with ABS when it moves into the blind area. Under this scenario, AMS can not notify ABS the blind area information before the disconnection. After disconnection, AMS will perform cell selection to find an available ABS for its network re-entry If the AMS has LBS capability, it could initiatively report the location information to the new serving ABS (can be another ABS or original ABS before disconnection). With the location information, ABS may try to configure its transmission power or notify network to develop a new ABS (Pico ABS, Relay ABS, or OSG-Femto ABS) to resolve this blind area. To report for location information after network entry, AMS can send AAI_RNG-REG message with the ranging purpose indication (bit #3 set to 1) to acquire the reporting resources during its re-entry. The modified ranging purpose indication parameter in AAI_RNG-REG message is represented at table 1.

Table 1, ranging purpose indication parameter in AAI_RNG-REG message

| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, it indicates that the AMS is currently attempting HO reentry, or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS. In this case, Bit#1 shall be 0. If Bit#1 is set to 1, it indicates that the AMS is initiating the idle mode location update process. In this case, Bit#0 shall be 0 If Bit#2 is set to 1, ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#3: is set to 1, it indicates that the AMS attempts to report its location information If bit #5 is set to 1, it indicates that the AMS is initiating location update for transmission to DCR mode from idle mode. If bit #6 is set to 1 in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting re-entry from DCR mode. <<<Editor’s note: reserved bits below are invalid>>> Bit#4, 7: reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| --- | --- | --- |

When receiving the AAI_RNG-REQ message with ranging purpose indication (bit #3 set to 1), ABS will grant resources for this AMS after its successful re-entry. And then AMS will send a LOC-REP message to report the last location information before its disconnection and the new location information after its re-entry. The format of LOC-REP is described by table 2. According to the AAI_LOC-REP message, ABS can identify the rough blind area location and perform corresponding resolved method. Figure 2 is an example of blind area detection and reporting when blind area does not appear on the cell boundary.

Table 2, AAI_LOC-REP message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| LOC-REP(){ |  | - |
| The newest location information | TBD | the newest location information after AMS network entry |
| The last location information | TBD | the last location information before AMS disconnection |
| } |  | - |

Figure 2, an example of blind area detection and reporting

- Proposed Text

[Accept the following modifications marked in blue.]

---------------------------------------------------- Start of Proposed Text----------------------------------------------------

15.7.3 Self Optimization

[Insert the new sub-section with following text]

15.7.3.3  Support of blind area detection and reporting

When an AMS with bad signal quality sends the AAI_SCN-REP message with none of target to its serving ABS, the ABS may initiate the blind area resolution.

If an AMS supports LBS capability, it can unsolicited make the location reporting after a sudden disconnection. An AMS can send AAI_RNG-REQ with indicated ranging purpose to ABS for this reporting during its re-entry. After getting associated allocation, the AMS can send AAI_LOC-REP to its serving ABS. When receiving the AAI_LOC-REP, ABS can initiate the blind area resolution.

15.2.3 MAC Control messages

      - AAI_RNG-REQ

[Modify the table with following text]

Table 674—Parameters for AAI_ RNG-REQ

| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, it indicates that the AMS is currently attempting HO reentry, or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS. In this case, Bit#1 shall be 0. If Bit#1 is set to 1, it indicates that the AMS is initiating the idle mode location update process. In this case, Bit#0 shall be 0 If Bit#2 is set to 1, ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If Bit#3: is set to 1, it indicates that the AMS attempts to report its location information If bit #5 is set to 1, it indicates that the AMS is initiating location update for transmission to DCR mode from idle mode. If bit #6 is set to 1 in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting re-entry from DCR mode. <<<Editor’s note: reserved bits below are invalid>>> Bit#3-7 4,7: reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| --- | --- | --- |

[Insert the new subsection]

15.2.3.40.x AAI_LOC-REP

The AAI_LOC-REP message format is defined in Table xxx. This message is used by AMS as a report to feedback the location of blind area.

Table xxx, AAI_LOC-REP message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| LOC-REP(){ |  | - |
| The newest location information | TBD | the newest location information after AMS network entry |
| The last location information | TBD | the last location information before AMS unexpectedly interrupts service |
| } |  | - |

---------------------------------------------------- End of Proposed Text ------------------------------------------------