| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Coverage and Capacity Optimization (15.7.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jung-Mao Lin, Chie Ming Chou, Frank Ren, Chun-Yuan Chiu ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | E-mail: moumou3@itri.org.tw whsheen@itri.org.tw |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes the power configuration procedures in SON operation (Section 15.7.3) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Coverage and Capacity Optimization

Jung-Mao Lin, Chie Ming Chou, Frank Ren, Chun-Yuan Chiu

ITRI

Wern Ho Sheen

Chaoyang University of Technology / ITRI

- Introduction

Based on current SDD, the SON functions require automatically resolving the blind areas to achieve the coverage and capacity optimization. In this contribution, we discuss the methods to resolve the blind areas and hope to achieve the coverage and capacity optimization.

- Discussion and proposal

Network can employ two approaches to extend the cell coverage: one is the operator can deploy a new ABS. The other is an ABS can reconfigure its transmission power. Those two approaches are described as follows.

  - Deploy new ABS to extend the cell coverage:

The blind areas or called coverage holes may happen due to some geographic features like tunnel or basement. Once AMS goes into in the coverage hole, it can not be served by any ABSs. This will degrade the QoS performance. If the location of coverage hole can be identified and be not on the cell boundary (refer to C802.16m_09-2262), the operator can deploy a new type of ABS such as a Pico ABS, or an ARS, or an OSG Femto ABS at this blind area by the location information.

  - Reconfigure ABS transmission power:

In most cases, an ABS can’t use the unlimited power to transmit data. This is because it may cause serious interference to neighbor ABS and inaccessible subscribers. An ABS, especial a Femto ABS may be requested to reduce its power when an AMS suffers serious interference from it. Once ABS reduces its transmission power, its cell coverage and capacity will be reduced and some blind areas may occur on the cell boundary. Besides, when Femto ABSs are introduced into the network, Femto ABSs may power down or temporarily stop services (e.g., Femto ABS goes into low duty mode). In this case, some operating ABSs may have chance to reconfigure its transmission power without the risk to raise interference. To reconfigure the transmission power of ABS, an inter-BS coordination power configuration is proposed. This configuration can be ABS-triggered or network-triggered:

  - ABS-triggered:

When there are some blind areas on the cell boundary discovered by the services AMS (ref to C802.16m_09-2262), the ABS may trigger power reconfiguration to cover the blind areas. When network receives this request from one ABS, the core network will negotiate with the neighbor ABSs whose geological locations are close to the requested ABS and operate at the same FA. During negotiation, the network will coordinate a sounding region for involved ABSs. Network will further inform those neighbor ABSs a dedicated sounding signal which would be transmitted at the sounding region. The sounding signal can be a PA/SA-preamble used by this ABS. After that, the network will request the triggered ABS to transmit sounding signal at this coordinated sounding region and resources with recommended transmitting power. To support the process, the involved other ABSs will also create the sounding region for measuring the signal quality (e.g. RSSI) of this sounding signal at that region. After measurements, neighbor ABSs will report the results to the network. The network will then calculate an acceptable transmission power and inform the results to the triggered ABS. An ABS can advertise the new value of ABS EIRP in its SP1 IE to notify serving AMSs that change. The whole procedures are described in figure 1. The details of sounding region are described at the following section.

Requested ABS

Neighbor ABSs

Network

Coordinate a common time interval, resources and a dedicated signal

Sounding signal

Make measurements

Report measurement result

Decide a suitable transmission power

Power configuration results

Operation

Inter-BS coordination power reconfiguration request

Create sounding region

Create sounding region

Figure 1, Inter-BS coordination power configuration (ABS-triggered)

  - Network-triggered:

Other than blind area resolution, network can also configure each ABS’s power to achieve network capacity optimization. During network-triggered case, network will coordinate the involved ABSs to reconfigure power by turns to improve the network performance. The configuration of target ABSs may include Macro and Femto ABS with same FA. During configuration, each ABS will transmit a specific sounding signaling at a coordinated sounding region sequentially. The sounding signal can be a PA/SA-preamble. The involved ABSs will create a coordinated sounding region based on the coordination to prepare to transmit or receive the sounding signal. Involved ABSs which receive the sounding signal will measure the signal quality (e.g. RSSI) of sounding signal and report to the network. The network can calculate the suitable transmission power for every involved ABSs and give commend to reconfigure the power. The ABS can advertise the new value of ABS EIRP in its SP1 IE to notify their serving AMSs the change. The whole procedures are described in Figure 2.

ABS

Network

Neighbor ABS

Coordinate a common time interval, resources and a dedicated signal

Sounding signal 1

Make measurements

Report measurement result

Sounding signal 2

Make measurements

Report measurement result

Decide suitable transmission power

Power configuration results

Power configuration results

Operation

Operation

Create sounding region

Create sounding region

Create sounding region

Create sounding region

Figure 2, Inter-BS coordination power configuration (network-triggered)

  - Sounding region to support inter-BS coordination power configuration:

When inter-BS coordination power configuration is triggered, core network will coordinate one radio frame in which the last DL subframe will be assigned as a sounding region. In this region, the involved ABSs will not schedule any data transmission and give any associated A-MAP for their serving AMSs. One involved ABSs will transmit a sounding signal in this region. Figure 3 is an example of sounding region (Tx side) when DL to UL ratio is 4:4. Once AMS can’t decode the A-MAP at DL subframe, it should ignore that DL subframe. On the other side, other involved ABSs need to receive and measure the sounding signal at the same sounding region. Figure 4 is an example of sounding region (Rx side) when DL to UL ratio is 4:4. To receive the sounding signal at sounding region, an S-TTG is required to make DL/UL translation. The length of S-TTG is equal to the TTG. Once the S-TTG appears in the frame structure, the ABSs do not need to switch Tx to Rx again at original TTG but reserve this gap and keep idle to make align with original start timing of UL for AMSs. If AMS can not decode the A-MAP at DL subframe, it should ignore that DL subframe. If the sounding signals which are transmitted at the sounding region are A-preamble, new incoming AMSs may be confused and synchronize with that sounding signal. In this case, AMS doesn’t detect following SFH, and it will try to perform synchronization again.

Figure 3, an example of the sounding region (Tx side)

Figure 4, an example of the sounding region (Rx side)

With this mechanism, one or multiple ABSs could be able to configure its transmitting power by themselves. Based on the sounding performed by involved ABSs, the power configuration results expected to be able to ensure the satisfied interference level and optimize the coverage and capacity of the network.

- Proposed Text

---------------------------------------------------- Start of Proposed Text----------------------------------------------------

15.7.3 Self Optimization

[Insert the new sub-section with following text]

15.7.3.x Support of Coverage and Capacity Optimization

When wanting to resolve blind area and optimize the coverage and capacity, an inter-BS coordination power configuration can be triggered.

With inter-BS coordination, the SON server will coordinate a sounding region in the last DL sudframe of a specific radio frame. The selected ABSs will transmit dedicated sounding signals at the sounding region with a pre-defined transmission power. The sounding signal can be a PA/SA-preamble. Other involved ABSs will measure the sounding signal quality (ex. RSSI) at the same sounding region and report the measurement results to SON server. The SON server will then calculate an acceptable transmission power for the selected ABS and give the response. After configuration, the ABS can advertise the new value of ABS EIRP in its SP1 IE to inform AMSs the new transmission power.

Figure xxx and yyy illustrate the sounding region for transmission side and reception side in frame structure. An ABSs will not schedule any data transmission and allocate any A-MAP for serving AMSs in this region. ABSs which are recommended to transmit dedicated sounding signals will transmit a sounding signal in this region. If AMS does not decode A-MAP at DL subframe, it should ignore that DL subframe.

Figure xxx, sounding region in frame structure (transmission side)

ABSs which are recommended to receive and measure the sounding signal will try to receive the sounding signal at the same region on their frame. To receive the sounding signal at the sounding region, an S-TTG is required for ABSs to make DL/UL translation. The length of S-TTG is equal to the TTG. Once the S-TTG is used in the frame structure, the original TTG shall be reserved to make align with original start timing of UL subframe for AMSs.

Figure yyy, sounding region in frame structure (reception side)

---------------------------------------------------- End of Proposed Text ------------------------------------------------