| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS_Random in Initial Ranging Procedure (15.2.3.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB30a / Area: Chapter 15.2.3 (MAC Control Messages) |  |
| Abstract | This contribution proposes changes to the Section 15.2.3.1 and 15.2.11.2 of IEEE P802.16m/D2 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS_Random in Initial Ranging Procedure (15.2.3.1)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

During network entry, the AMS, after acquiring downlink synchronization and uplink transmission parameters, shall send the selected ranging sequence to the ABS in the selected Ranging Slot. Upon receiving a Success status notification in AAI_RNG-ACK message, the AMS shall wait for the ABS to provide UL BW allocation to send the AAI_RNG-REQ message. Then the ABS shall send an AAI_RNG-RSP message to the AMS as the response to the AAI_RNG-REQ. To pair the AAI_RNG-REQ and AAI_RNG-RSP messages, the “MS_Random” value generated in the AAI_RNG-REQ message shall be included in the corresponding AAI_RNG-RSP message. In current D2, however, MS_Random is randomly generated by the AMS, it is very possible that multiple AMSs allocated by different CDAM-Allocation_A-MAP IEs generate the same MS_Random value. As a result, those AMSs cannot distinguish which AAI_RNG-RSP they shall follow.

Figure 1. The transaction mismatch occurs when multiple AMSs generate the same MS_Random.

To solve this issue, this contribution proposes that MS_Random shall be generated by the ABS and assigned to the AMS through the CDMA_Allocation_A-MAP IE. Since this parameter is no longer generated randomly by the AMS, we suggest using another terminology which is “Ranging_ID”, instead of using “MS_Random”.

- Proposed Text for IEEE P802.16m/D2

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

#### 15.2.3.1 AAI_RNG-REQ

Table 674—Parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |

#### 15.2.3.2 AAI_RNG-RSP

Table 675—Parameters for AAI_ RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| STID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID yet |
| MS_Random Ranging_ID | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. This Ranging_ID shall be identical to the Ranging_ID in the CDMA_Allocation_A-MAP IE of the same transaction. |  |

#### 15.2.11.2 Grant

Table 734—CDMA Allocation A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  | — |
| A-MAP IE Type | 4 | CDMA Allocation A-MAP IE |
| If MCRC is masked with RAID and masking indicator for BR { |  |  |
| Resource Assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Reserved | TBD |  |
| } |  |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Ranging_ID | TBD | The identifier of an initial ranging transaction. |
| Resource assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation |
| Timing Adjust | 10 | Amount of advance time required to adjust AMS transmission. Unsigned in units of 1/Fs |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Offset Frequency Adjust | 6 | Relative change in transmission frequency. Signed in units of Hz |
| Reserved | TBD |  |
| } |  |  |
| MCRC | 16 | CRC masked by RA-ID and Masking Indicator |
| } | — |  |

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D2. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Oct 2009.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep. 2009.