| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging cycle and paging offset configurations (15.2.3.21) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI YihShen Chen MediaTek | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB30a / Area: Chapter 15.2.3 (MAC Control Messages) |  |
| Abstract | This contribution proposes changes to the Section 15.2.3.21 and 15.2.17.2.3 of IEEE P802.16m/D2 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging Cycle and Paging Offset Configurations (15.2.3.20)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

YihShen Chen

MediaTek

- Introduction

In current D2, 4 bits are applied in the paging cycle parameter, but the values are not determined. We need to define the values specified by this 4-bits parameter. In addition, the bit numbers and values of paging offset are also undetermined. We suggest also applying 4 bits for paging offset for the efficiency consideration.

The maximum length of paging cycle shall not exceed the maximum Superframe number, which is 4096 (12 bits) in current D2. The minimum length of the paging cycle should be determined by considering the power saving gain and paging message overheads. Considering the worst case where an AMS may keep monitoring the PGID Info and paging message for the entire paging listening interval (20 ms), the power saving efficiency with different paging cycle lengths for an idle mode AMS is shown in the below table.

| Paging cycle length | Power saving efficiency (compared to operating in connected mode) | Averaged paging latency (ms) |
| --- | --- | --- |
| 2 superframes | 50 % | 20 |
| 4 superframes | 75 % | 40 |
| 8 superframes | 87.5 % | 80 |
| 16 superframes | 93.75 % | 160 |
| 32 superframes | 96.87 % | 320 |
| 64 superframes | 98.43 % | 640 |
| 128 superframes | 99.21 % | 1280 |

In most electronic device requirements, the idle mode function aims to reach a power saving efficiency which is always above 85%. In addition, by referring to 3GPP LTE technical specification [4], the minimum paging cycle (Idle DRX) length is 320 ms, which is equal to 16 superframes in 16m. According to the above two reasons, the paging cycle lengths which are less than 8 superframes are marked in gray, meaning we don’t need to consider them.

- Proposed Text for IEEE P802.16m/D2

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

#### 15.2.3 MAC Control Messages

#### 15.2.3.21 AAI_DREG-RSP message

The following parameter shall be included in AAI_DREG-RSP message:

Control message type

Action code

0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS.

0x01: AMS shall listen to the current ABS but shall not transmit until a RES-CMD message or AAI_DREG-RSP message with action code 0x02 or 0x03 is received.

0x02: AMS shall listen to the current ABS but only transmit on the basic and primary management flows.

0x03: AMS shall return to normal operation and may transmit on any of its active connections.

0x04: This option is valid in response to a AAI_DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Normal Operation with the ABS.

0x05: AMS shall begin idle mode initiation: a) to allow AMS-initiated idle mode request or b) to signal AMS to begin idle mode in unsolicited manner or c) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration

0x06: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration

0x07: This option is valid only in response to an AI_DREG-REQ message with De-Registration

Request Code 0x04 to allow retention of the AMS’s connection information

0x08: This option is valid only in response to an AAI_DREG-REQ message with De-Registration

Request Code 0x04 to reject retention of the AMS’s connection information

When the action code is set to 0x05, the following parameters shall be included in AAI_DREG-RSP message.

Paging information

Paging cycle (4 bits)

0x00: 0 superframes (applied for DCR mode)

0x01: 8 superframes

0x02: 16 superframes

0x03: 32 superframes

0x04: 64 superframes

0x05: 128 superframes

0x06: 256 superframes

0x07: 512 superframes

0x08: 1024 superframes

0x09: 2048 superframes

0x0a: 4096 superframes

0x0b~0x0f: reserved

Paging offset

Paging group ID

Paging controller ID

Idle Mode Retain Information

Deregistration Identifier

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D2. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Oct 2009.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.

- 3GPP TS 36.304 V8.1.0. Evolved Universal Terrestrial Radio Access (E-UTRA), User Equipment (UE) procedures in idle mode, March 2008.