| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Bit Width of HFA Field (15.2.11.2, 15.3.6, and 15.3.8.3.3.2) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Yujian Zhang, Yi Hsuan Intel Corporation | +86-10-82611515 {yujian.zhang,yi.hsuan}@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | Proposed text for Bit Width of HFA Field. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Bit Width of HFA Field (15.2.11.2, 15.3.6, and 15.3.8.3.3.2)

Yujian Zhang, Yi Hsuan

Intel Corporation

- Introduction

This contribution proposes following changes for the HFA field:

- The calculation of HFA index for CDMA Allocation A-MAP IE, Feedback Allocation A-MAP IE, and Feedback Polling A-MAP IE are not specified. It is proposed to use same method as DL/UL basic assignment A-MAP IEs.

- Bit width for HFA field in DL/UL basic assignment A-MAP IEs and CDMA Allocation A-MAP IE is 4 bits, while it is [3] bits for Feedback Allocation A-MAP IE. According to C80216m-09/1366r1, 3 bits are sufficient in these cases. It is therefore proposed to change HFA field in these IEs to 3 bits.

- In addition, bit width for HFA field in DL/UL Individual/Composite Persistent A-MAP IEs has 5 bits. Since HFA for persistent allocation uses explicit indexing, it is proposed to change the bit width to 6 bits to support 64 HFA channels.

- Text Proposal

-------------------------------  Start Text Proposal ---------------------------------------------------

- [  ------ Recommended Text Proposal #1 (page 179, Line 26) ------]

15.2.11.2 Grant

…

Table 734—CDMA Allocation A-MAP IE format

| … | … | … |
| --- | --- | --- |
| Resource Assignment Information | TBD |  |
| HFA | 43 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Reserved | TBD |  |
| } |  |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Resource Assignment Information | TBD |  |
| HFA | 43 | HARQ Feedback Allocation |
| Timing Adjust | 10 | Amount of advance time required to adjust AMS transmission. Unsigned in units of 1/Fs |
| … | … | … |

- [  ------ Recommended Text Proposal #2 (page 339, Line 5) ------]

15.3.6.3.2.3 HARQ Feedback A-MAP

…

For group resource allocation, HF-A-MAP resource index for the l-th AMS in GRA allocation is(istart+l∗NHF−A−MAPNGRA)modNHF−A−MAP, where istart is the ACK Channel Offset in UL group resource allocation A-MAP IE, NHF-A-MAP is the total number of HF-A-MAP configured per HF-A-MAP region, and NGRA is the Use Bit Map Size in UL group resource allocation A-MAP IE.

For resource allocation using UL basic assignment A-MAP, and CDMA Allocation A-MAP IE, HF-A-MAP resource index is (M(j)+n)modNHF−A−MAP, where j is HF-A-MAP Index Parameter in Non-user specific A-MAP IE, n is HFA in UL basic assignment A-MAP IE, NHF-A-MAP is the total number of HF-A-MAP configured per HF-A-MAP region, M(j) is STID when j=0 and M(j) is lowest LRU index of corresponding UL transmission when j=1.

- [  ------ Recommended Text Proposal #3 (page 360, Line 50) ------]

15.3.6.5.2.2 DL basic assignment A-MAP IE

…

Table 801—DL basic assignment A-MAP IE*

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | [4]3 | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Reserved | TBD | Reserved bits |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

…

Long TTI Indication: Indicator to signal allocations span multiple AAI subframes in time.

HFA: TBD aAllocation for HARQ feedback.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

- [  ------ Recommended Text Proposal #4 (page 365, Line 49) ------]

15.3.6.5.2.3 UL basic assignment A-MAP IE

…

Table 802—UL basic assignment A-MAP IE*

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | [4]3 | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | TBD | Reserved bits |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

- [  ------ Recommended Text Proposal #5 (page 379, Line 31) ------]

15.3.6.5.2.8 DL PA A-MAP IE

…

Table 812—DL Individual Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | TBD |  |
| … | … | … |

- [  ------ Recommended Text Proposal #6 (page 380, Line 49) ------]

15.3.6.5.2.8 DL PA A-MAP IE

…

Table 812—DL Individual Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 56 | Explicit index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| … | … | … |

- [  ------ Recommended Text Proposal #7 (page 382, Line 50) ------]

15.3.6.5.2.8 DL PA A-MAP IE

…

Table 813—DL Composite Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| }else{ |  |  |
| … | … | … |

- [  ------ Recommended Text Proposal #8 (page 385, Line 17) ------]

15.3.6.5.2.8 DL PA A-MAP IE

…

Table 813—DL Composite Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation |
| } |  |  |
| … | … | … |

- [  ------ Recommended Text Proposal #9 (page 386, Line 39) ------]

15.3.6.5.2.9 UL PA A-MAP IE

…

Table 815—UL Individual Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| } else if (Allocation Period != 0b00){ |  |  |
| … | … | … |

- [  ------ Recommended Text Proposal #10 (page 387, Line 46) ------]

15.3.6.5.2.9 UL PA A-MAP IE

…

Table 815—UL Individual Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| … | … | … |

- [  ------ Recommended Text Proposal #11 (page 389, Line 35) ------]

15.3.6.5.2.9 UL PA A-MAP IE

…

Table 816—UL Composite Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| } |  |  |
| … | … | … |

- [  ------ Recommended Text Proposal #12 (page 391, Line 15) ------]

15.3.6.5.2.9 UL PA A-MAP IE

…

Table 816—UL Composite Persistent A-MAP IE

| … | … | … |
| --- | --- | --- |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD f number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 56 | Explicit Index for HARQ Feedback Allocation |
| } |  |  |
| … | … | … |

- [  ------ Recommended Text Proposal #13 (page 392, Line 51) ------]

15.3.6.5.2.11 Feedback Allocation A-MAP IE

…

Table 818—Feedback Allocation A-MAP IE

| … | … | … |
| --- | --- | --- |
| If ( ACK allocation flag == 0b1 ) { |  |  |
| HFA | [3] | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } |  |  |
| … | … | … |

- [  ------ Recommended Text Proposal #14 (page 461, Line 18) ------]

15.3.8.3.3.2 Feedback Channels

…

For group resource allocation, index k for the l-th AMS in GRA allocation is   (istart+l∗LHFBNGRA)modLHFB, where istart is the ACK Channel Offset in DL group resource allocation A-MAP IE, LHFB is the total number of HFBCH configured per HARQ feedback region, and NGRA is the Use Bit Map Size in DL group resource allocation A-MAP IE.

For resource allocation using DL basic assignment A-MAP, Feedback Allocation A-MAP IE, and Feedback Polling A-MAP IE, index k is (M(j)+n)modLHFB, where j is HFBCH Index Parameter in Non-user specific A-MAP IE, n is HFA in DL basic assignment A-MAP IE, LHFB is the total number of HFBCH configured per HARQ feedback region, M(j) is STID when j=0 and M(j) is lowest LRU index of corresponding DL transmission when j=1.

-------------------------------------- End of Text Proposal -------------------------------------------