| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced EH Format (section 15.2.2.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Sungjin, Lee, Rakesh Taori, Jungje Son, Youngkyo Baek Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: Enhanced EH Format (Section 15.2.2.2) |  |
| Abstract | This contribution proposes text for enhanced EH format |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Enhanced EH Format (section 15.2.2.2)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son, Youngkyo baek

Samsung Electronics

# Introduction

This contribution summarizes the changes needed (with editorial instructions) for the proposal in the slides S802.16m-09/2269.pdf.

Proposed Text Change1: EH format change to incorporate EH length

Proposed Text Change 2-6: FPEH/FEH change to FPSH/FSH

Proposed Text Change 7-11: FID for encryption

# Proposed text

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

[Change 1: Modify the section 15.2.2.2, page 18 (line 60-65), page 19 (line 1-26) as shown below]

---------------------------------------------------Text Start---------------------------------------------

15.2.2.2 Extended header formats

The inclusion of Extended Header group is indicated by EH bit in MAC Header. The extended header group (see figure xxx), when used, shall always appear immediately after the MAC header. Extended header group shall not be encrypted. The fields of the Eextended header group format is are defined in Table 655— and will be used unless specified otherwise.

Figure xxx Extended Header Group Format

Table 655 – Extended Header Group Format Fields

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended Headers { |  |  |
| Last | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| EH Group Length |  | Extended header Length |
| Type | TBD | Type of extended header |
| Body Contents | Variable | Type dependent content |
| } |  |  |
| Extended header Group Length | 8 | The Extended header Group Length field indicates the total length of the extended header group, including all the extended headers and the Extended header Group length byte. |
| Extended header Type | TBD | Type of extended header as defined in Table 656. |
| Extended header Body | variable | The size of the extended header is determined by extended header type as specified in Table 656. The extended header including the extended header type is byte aligned. |

---------------------------------------------------Text End---------------------------------------------

[Change 2: Insert the section shown below after section 15.2.2.2]

---------------------------------------------------Text Start---------------------------------------------

15.2.2.x MAC Subheaders

Two types of subheaders, FSH & FPSH may be present in a MAC PDU.

------------------------------------------Text End (Change -2)---------------------------------------------

[Change 3: Modify section 15.2.2.2.1, page 19-22, as follows]

---------------------------------------------------Text Start---------------------------------------------

15.2.2.2.1 15.2.2.x.1 Fragmentation and packing extended subheader (FPESH)

The FPESH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists before the SDU/SDU fragments of the transport connection in the MAC PDU. The presence of FPSH in a MAC PDU with single transport connection payload is illustrated in figure xxx. The presence of FPSH in a MAC PDU with multiplexing is illustrated in figure yyy. after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in AGMH set to '1' or after the AGMH if 'EH' in AGMH set to '0'. The FPESH format is defined in Table 657. 'RI' in FPESH is used for indicating whether the payload contains ARQ sub-blocks or not. If 'RI' bit set to '1', 'LSI' and 'SSN' shall be included in FPESH.

Figure xxx FPSH location in a MAC PDU with a transport connection payload

Figure yyy FPSH location in a MAC PDU with multiplexing

Table 657 – FPESH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH() { |  |  |
| RI | 1 | Re-arrangement information indicator |
| SN | 10 | SN is maintained per connection. For non ARQ connection, ‘SN’ represents the MAC PDU Payload Sequence Number and the ‘SN’ value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, ‘SN’ represents the ARQ block sequence number. |
| FC | 2 | Fragmentation Control bits (see Table 659) |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPESH |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in GMH |
| if (RI == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another ‘Length’ and ‘End’ fields are followed 1 = Indicating no more ‘Length’ and ‘End’ fields are followed |
| if (End == 0) { |  |  |
| Length | 11 | MAC PDU with single connection payload: This field indicates the length of SDU or SDU fragment in the MAC PDU payload. If a the MAC PDU payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPEH. These represent the length of first 'N-1' SDU/SDU fragments in MAC PDU payload. The length of the last SDU or SDU fragment in the MAC PDU payload is = MAC PDU payload length (after decryption) - sum of 'N-1' length fields in FPESH. where MAC PDU payload length (after decryption) = Length of MAC PDU (given by length field in GMH) - Length of GMH(2 bytes) - sum of length of all extended headers present in MAC PDU - Length of PN& EKS (3bytes, if present) - Length of ICV ( 8 bytes, if present) – Length of FPSH. MAC PDU with multiple connections payload: MAC PDU payload consists of multiple connection payload. This field indicates the length of SDU or SDU fragment in the connection payload corresponding to this FPSH. If the connection payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPSH. These represent the length of first 'N-1' SDU/SDU fragments in connection payload. The length of the last SDU or SDU fragment in the connection payload is = Connection payload length (see table 661, MEH format) - sum of 'N-1' length fields in FPSH. |
| } |  |  |
| } while (!End) |  |  |
| Reserved | Variable | Reserved bits are added at the end of FPESH for byte alignment. |
| } |  |  |

Table 658 FPEH Fields

[Delete table 658]

Table 659 Encoding of FC Field

[No change, keep it as it is]

------------------------------------------Text End (Change -3) --------------------------------------------

[Change 4: Modify section 15.2.2.2.2, page 22-23, as follows]

---------------------------------------------------Text Start---------------------------------------------

15.2.2.2.2 15.2.2.x.2 Fragmentation extended subheader (FESH)

The FESH shall be used when MAC PDU contains payload from a single management control connection. The FSH exists before the unfragmented control message or control message fragment in the MAC PDU. The presence of FSH in a MAC PDU with single control connection payload is illustrated in figure ppp. The presence of FSH in a MAC PDU with multiplexing is illustrated in figure qqq.  that are The FESH format is defined in Table 660.

Figure ppp FSH location in a MAC PDU with a control connection payload

Figure qqq FSH location in a MAC PDU with multiplexing

Table 660 FESH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FESH () { |  |  |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| If (SN Indicator = 0) { |  |  |
| Reserved | 67 | For byte alignment |
| } else { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see <<<table TBD>>>) |
| SN | 8 | Payload sequence number |
| Reserved | 34 | For byte alignment |
| } |  |  |
| } |  |  |

------------------------------------------Text End (Change-4) --------------------------------------------

[Change 5: Delete ‘Last’ field from table 661, 662, 664, 665, 666 and 667]

[Change 6: Delete fragmentation extended header and fragmentation and packing extended header from table 656 on page 19]

[Change 7: Modify section 15.2.10.1, page 172, line 37-47 as follows]

---------------------------------------------------Text Start---------------------------------------------

15.2.10.1 Management connections

Two One pairs of bi-directional unicast management control connections - basic connection and primary management connection, are is automatically established when an AMS performs initial network entry. The basic connection is used by the ABS MAC and AMS MAC to exchange short, time-urgent MAC control messages. The primary management connection is used by the ABS MAC and AMS MAC to exchange longer, more delay-tolerant MAC control messages. FID with value 0 and 1 are reserved for unicast control connection. FID with value 0 is used when MAC PDU contains unencrypted control connection payload. FID with value 1 is used when MAC PDU contains encrypted control connection payload. these two management connections respectively. The mapping for MAC control messages to connection shall be static based on Table 673.

------------------------------------------Text End (Change -7) --------------------------------------------

[Change 8: Modify table 652, page 16-17 as follows]

---------------------------------------------------Text Start---------------------------------------------

| Value | Description |
| --- | --- |
| 0000 | Basic management FID Control connection FID; used when MAC PDU contains unencrypted control message or control message fragment |
| 0001 | Primary management FID Control connection FID; used when MAC PDU contains encrypted control message or control message fragment |
| 0010 | Signaling Header indicator |
| 0011 | Pre provisioned Transport connection for BE Service |
| 0100-1111 | Transport FID |

------------------------------------------Text End (Change-8) --------------------------------------------

[Change 9: Modify text in section 15.2.3, page 32, line 43-59 as follows]

---------------------------------------------------Text Start---------------------------------------------

15.2.3 MAC Control messages

The peer-to-peer protocol of MAC layers in ABS and AMS communicate using the MAC control messages to perform the control plane functions. MAC control messages shall be carried in a MAC PDU to be transported in broadcast, unicast or random access connections. There is a single unicast Control connection. HARQ shall be enabled for MAC control messages sent on the unicast Control connection. Encryption may be enabled for unicast MAC control messages. MAC control messages may be fragmented. MAC control messages shall not be sent using multiplexing extended header. Table 673 lists the MAC control messages that shall be defined in the ASN.1 format, as shown in <<<Appendix X>>>. The indication to the receiver whether the PDU is encrypted is indicated by the EC=1 in FEH extended header FID in the AGMH. Whether the encryption is applied on a management message or not shall be determined by the message type and MAC procedure context, which is define in <<Table TBD>>. A messages included in a PDU whose EC bit FID value does not match the combined message type and corresponding context defined in the <<Table TBD>> shall be discarded. Encrypted and non encrypted MAC control messages shall not be sent in the same PDU.

------------------------------------------Text End (Change-9) --------------------------------------------

[Change 10: Modify text in section 15.2.5.2.5, page 111, line 24-28 as follows]

---------------------------------------------------Text Start---------------------------------------------

The fragment extended header is used only for management flows. The EC bit FID in the Fragment extended header AGMH is used to indicate whether the PDU contains control message encrypted based on security level. Whether each control message is encrypted or not is decided based on the security level which the message is associated with.

------------------------------------------Text End (Change-10) --------------------------------------------

[Change 11: Modify text in section 15.2.5.4.2, page 115, line 52-58 as follows]

---------------------------------------------------Text Start---------------------------------------------

The selective confidentiality protection over control messages is indicated by the FID EC bit in the AGMH FEH. Contrary to the transport flows where the established SA is applied to all data, the SA is selectively applied to the management flows. Security extended header is used only for management flows to indicate whether PDU contains the control message that is encrypted based on control message type and its usage. In particular, whether control message is encrypted or not is decided on the security level with which the message is associated.

------------------------------------------Text End (Change-11) --------------------------------------------

# Reference

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”